3GPP TSG-SA5 Meeting #147	S5-232529

Athens, Greece, 27th Februry– 3rd Marth, 2023

# Source:	China Unicom

# Title:	Add Conclusion and recommendation for Issue #4

# Document for:	Approval

Agenda Item:	6.8.3

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.832 v0.3.0: “Management Aspects of URLLC”

[2]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM);”

3	Rationale

It was approved in SP-220146 to study the management aspects of URLLC and this contribution proposes to add conclusion and recommendation the following issue in Study on management aspects of URLLC.

- Issue#5.4: Configuration of reliability in slice profiles and service profile

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

# 6	Conclusion and Recommendation

## 6.X	Issue #X: Configuration of reliability in slice profiles and service profile

It’s recommended to enhance the ServiceProfile <<dataType>>, CNSliceSubnetProfile<<dataType>>, RANSliceSubnetProfile<<dataType>> and TopSliceSubnetProfile<<dataType>> in TS 28.541 to support the use cases of different reliability requirement between UL and DL. New attributes, dlReliability and ulReliability, are introduced to the above four IOCs to replace the original attribute, reliability. Detailed description about the solution and the definitions of new attributes is in caluse 5.4.2.

End of changes