3GPP TSG-SA5 Meeting #147 	S5-232554

Athens, Greece, 27 February – 3 March 2023

# Source:	Huawei

# Title:	pCR 28.865 Add solultion of network preparation

# Document for:	Approval

Agenda Item:	6.9.5.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-211442: "New SID on deterministic communication service assurance"

[2]	3GPP TR 28.865: "Study on deterministic communication service assurance"; v0.3.1

[3]	3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3"; v18.2.2

[4]	TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; stage 1"; v18.3.0

[5]	TS 22.261: "Service requirements for cyber-physical control applications in vertical domains; stage 1"; v18.8.0

[6]	SP-220981: "New Rel-18 WID Management Data Analytics phase 2"

[7]	TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"; v18.2.0

[8]	TS 28.535: "Management services for communication service assurance; Requirements"; v17.5.0

[9]	TS 28.536: "Management services for communication service assurance; Stage 2 and stage 3"; v17.4.1

[10]	NG.116: "Generic Network Slice Template"; v7.0; 17 June 2022.

3	Rationale

This tdoc addresses the network capability preparation related to DCSA [1, 2].

To support deterministic communication service, some network functions are defined in RAN and CN specifications and listed in clause 5.X.1 in TR 28.865 [2]. 3GPP management system are responsible to the preparation and configuration of the relevant network capabilities according to the service requirements of the consumer. Some of the network capabilities related to DCSA are described in Table 1. The value of these potential new attributes may be prepared with the assistance of eMDAS analytical services.

Table 1

| Category | Attribute name | Description |
| --- | --- | --- |
| Capacity related network capability preparation | numOfPDUsessionsInLocationList | The number of PDU session are strictly prepared with the mapping of corresponding service location in order to meet the deterministic characteristics. |
|  | numOfUsersInLocationList | The number of users are strictly prepared with the mapping of corresponding service location in order to meet the deterministic characteristics. |
| Reliability related network capability preparation | numOfAdditionalLinks | The number of additional transmission links. Redundant transmissions should be configured for high reliability. |
|  | typeOfAdditionalLink | The following types of additional transmission links are supported: e2e, N3/N9, Transport: - e2e: Dual Connectivity based end to end Redundant User Plane Paths; - N3/N9: Support of redundant transmission on N3/N9 interfaces; - Transport: Support for redundant transmission at transport layer; |

Proposal 1: 3GPP management system should provide the network capability preparation and configuration for deterministic communication services, including latency, capacity and reliability related network capability preparation.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

Editor's note: this clause will contain the issues and potential solutions for deterministic communication service assurance. Relation and potential enhancements to eCOSLA will also be studied for the related issues.

5.X.2	Potential solutions

5.X.2.a	Potential solution #2: Network capability preparation

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for issue#1 at a high-level.

Based on deterministic communication service requirements, network capabilities are prepared, e.g., network performance requirement evaluation and network functions provisioning.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for issue#1.

The network capability preparation should be supported.This may include network functions and resources to support URLLC and industrial IoT related features defined in the network aspects.

Network capability preparation needs to consider network deployment requirements of different locations and areas to minimize network requirements to avoid resource waste. The network capability preparation of deterministic communication services has more stringent SLA requirements and delay constraints.

Some network capability configurations are listed in the following table:

Table 1

| Category | Attribute name | Description |
| --- | --- | --- |
| Capacity related network capability preparation | numOfPDUsessionsInLocationList | The number of PDU session are strictly prepared with the mapping of corresponding service location in order to meet the deterministic characteristics. |
|  | numOfUsersInLocationList | The number of users are strictly prepared with the mapping of corresponding service location in order to meet the deterministic characteristics. |
| Reliability related network capability preparation | numOfAdditionalLinks | The number of additional transmission links. Redundant transmissions should be configured for high reliability. |
|  | typeOfAdditionalLink | The following types of additional transmission links are supported: e2e, N3/N9, Transport: - e2e: Dual Connectivity based end to end Redundant User Plane Paths; - N3/N9: Support of redundant transmission on N3/N9 interfaces; - Transport: Support for redundant transmission at transport layer; |

Note: These attributes will be revisited in the future, and new attributes may be introduced.

End of change