3GPP TSG-SA5 Meeting #147 	S5-232585

Athens, Greece, 27th February - 3rd March 2023

# Source:	Ericsson

# Title:	Adding service identifier based solution to clause 5.3

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

New solution in clause 5.3 based on service identifier as a string together with use of string for rating group.

# 4	Detailed proposal

First change

#### 5.3.5.y	Solution #3.y Enhancement of rating group and service identifier using string

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

The rating group and service identifiers are today defined as unsigned 32-bit integers, even if the number of groups and identifiers can be 4294967296 it still this means that there is a limited amount of information that can be transferred. The use of an integer requires range handling, where different ranges are used for different types and services to precent errors when configuring. The use of string for service id would allow for more information about the service to be past, since it can be grouped and allowing a matching of patterns. To be able to separate rating groups and services that are dedicated to a specific MVNO or tenant, the handling of integers will be a limiting factor.

Using sting would allow unlimited number of rating groups and service identifiers, it would also allow grouping of rating groups and service identifiers. This would mean that a service could be named after the rating groups it belongs to, or it could be named after the MVNO that uses it.

One example could be that all rating groups for MBB (Mobile Broadband) could start with “MBB”. Within the MBB there could be two rating groups one for high speed and one basic speeds, which could then be named “MBB-High” and “MBB-Basic” respectively. Both rating groups could then have a music streaming service, this service could then be named “MBB-MusicStream”, if there was a service only applicable to the to “MBB-High” like movie streaming it could be named “MBB-High-MovieStream”.

The internal format and usage of the strings would be operator specific and outside the 3GPP SA5 specifications.

Having the rating group as string will also allow it to be used as the key in OpenAPI “map”.

Table 5.3.5.y-1: Changes to ChargingDataRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| multipleUnitUsage | array(MultipleUnitUsage) | OC | 0..N | This field contains the parameters for the quota management request and/or usage reporting. |  |
| requestedAndUsedUnitsInfo | map(MultipleUnitUsage) | OC | 0..N | This field is used instead of multipleUnitUsage when the data type of rating group is string. The rating group is used as the key of the map. It contains the parameters for the quota management request and/or usage reporting. | RatingGroupString |

Table 5.3.5.y-2: Changes to ChargingDataResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| multipleUnitInformation | array(MultipleUnitInformation) | OC | 0..N | This field holds the parameters for the quota management and/or usage reporting information. It may have multiple occurrences. |  |
| quotaAndUnitReportMgmtInfo | map(MultipleUnitInformation) | OC | 0..N | This field is used instead of multipleUnitInformation when the data type of rating group is string. The rating group is used as the key of the map. It holds the parameters for the quota management and/or usage reporting information. It may have multiple occurrences. | RatingGroupString |

Table 5.3.5.y-3: Changes to MultipleUnitUsage

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ratingGroup | RatingGroup | OM | 0..1 | The identifier of a rating group, only required if data type for rating group is Uint32. |  |
| serviceIdList | array(string) | OC | 0..N | This field identify of the services that may be used within the rating group. | ServiceIdString |
| requestedUnit | RequestedUnit | OC | 0..1 | This field indicates, if included, that quota management is required. It may additionally contain the amount of requested service units for a particular category. |  |
| usedUnitContainer | array(UsedUnitContainer) | OC | 0..N | This field contains the amount of used non-monetary service units measured. |  |
| usedUnitsReportInfo | map(UsedUnitContainer) | OC | 0..N | This field is used instead of usedUnitContainer when the data type of service id is string. The ServiceIdString is used as the key of the map. This field contains the amount of used non-monetary service units measured. | ServiceIdString |

End of changes