3GPP TSG-SA5 Meeting #147 	S5-232588

Athens, Greece, 27th February - 3rd March 2023

# Source:	Ericsson

# Title:	Additional evaluation to clause 5.1

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Adding evaluations of solutions in in clause 5.1.

# 4	Detailed proposal

First change

### 5.1.6	Evaluation

Solution #1.1 solve key issues #1a, #1b, and #1c.

-	Solution #1.1: introduces a new trigger, that will hold the charging data request until there is traffic.

Solution #1.2 solve key issue #1e.

-	Solution #1.2: requires all used units reported for a rating group to be delivered in the same request.

Solution #1.3 solve key issue #1j.

-	Solution #1.3: requires that the NF (CTF) provides the level of accuracy required by the CHF.

Solutions #1.4, #1.5, #1.6 and #1.7 all solve key issue #1l.

-	Solution #1.4: NF (CTF) stops service delivery if no response is received before all the quotas (between the threshold and the granted quota) have been used and discards used units in the case of termination is received.

-	Solution #1.5: NF (CTF) terminates the service if no response is received before all the quotas (between the threshold and the granted quota) have been used and reports the used units in a termination request.

-	Solution #1.6: NF (CTF) stops or continues service delivery if no response is received all the quotas (between the threshold and the granted quota) have been used depending on CHF indication and reports used units in the next request.

-	Solution #1.7: NF (CTF) stops service delivery and waits for response when all the quotas (between the threshold and the granted quota) have been used and reports used units in the next request.

Solution #1.4 will in some cases discard used units and not repot it, solution #1.5 will in some cases terminate the service even though there are funds on the account, and solution #1.6 requires new functionality in both CHF and NF (CTF) while solution #1.7 is the current solution.

Editor’s note: Further evaluation is FFS.

### 5.1.7	Conclusions

Editor’s note: Conclusions are FFS.

End of changes