3GPP TSG-SA5 Meeting #147 	S5-232598

Athens, Greece, 27th February - 3rd March 2023

# Source:	Ericsson

# Title:	Adding solution in clause 7.1 for PDU session triggers

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding a solution on QBC and FBC common PDU session level triggers in clause 7.1 for key issue #1f covering requirements REQ-CH_VMNO-08.

# 4	Detailed proposal

First change

#### 7.1.4.x	Solution #1.x: QBC and FBC common PDU session level triggers

##### 7.1.4.x.1	General

A possible solution for key issue #1f covering requirement REQ-CH_VMNO-08 about the PDU session level setting for QBC triggers.

The QBC triggers and FBC triggers share PDU session level triggers, this means that there is one set of PDU session level triggers from the CHF, even when both QBC and FBC is applicable at the same time. The roaming charging profile is controlled by the H-CHF, since it is the H-CHF that is the last network function to update the roaming charging profile, and the V-CHF will only be informed of the update. For FBC it is only the H-CHF that sets the triggers. This means that H-CHF that controls both the QBC and FBC triggers and can synchronise the PDU session level triggers. To ensure that the QBC triggers are the same for both home and visited MNO the QBC triggers settings overwrite any setting based on FBC if they would differ.

##### 7.1.4.x.2	Reference architecture

The reference architecture would be the same as in solution #1.3 clause 7.1.4.3.2.

##### 7.1.4.x.3	Message flows

For home routed scenario, the message flow is same as the PDU session establishment message flow in clause 5.2.2.12.2 of TS 32.255 [4]. In step 11ch-d of Figure 5.2.2.12.2.1 in TS 32.255 [4], the H-CHF acknowledges by sending Charging Data Response to the H-SMF and optionally includes a "Roaming Charging Profile". The "Roaming Charging Profile" may include both QoS flow and PDU session level triggers (e.g., both QoS change and user location change) for QBC, for FBC both rating group and PDU session level triggers may be included in the trigger element of Charging Data Response. For FBC the PDU session level triggers may be replaced by the QBC PDU session level triggers.

For local breakout scenario, the message flow is same with the PDU session establishment message flow in clause 5.2.2.18.2 of TS 32.255 [4]. In step 9ch-d2 of Figure 5.2.2.18.2-1 in TS 32.255 [4], the H-CHF acknowledges by sending Charging Data Response to the V-SMF and optionally includes a "Roaming Charging Profile". The "Roaming Charging Profile" include both QoS flow and PDU session level triggers (e.g., both QoS change and user location change) for QBC, for FBC both rating group and PDU session level triggers may be included in the trigger element of Charging Data Response. For FBC the PDU session level triggers are replaced by the QBC PDU session level triggers, if included.

This means that the values and status of PDU session level triggers will be the same for both QBC and FBC. For example, a PDU Session level "Expiry of data time limit per PDU session" for FBC is set to 600 seconds while it for QBC is set to 900 seconds, then the 900 seconds value will be used for all the rating groups and QoS flows.

End of changes