3GPP TSG-SA5 Meeting #147 	S5-232608

# Athens, Greece,  Feb 27- March 3, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | draftCR | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 28.533 DraftCR for roles to policies and criteria usecase |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Role-based access control offers an abstraction layer by defining roles as set of fine grained access rights to resources (managed objects) and corresponding operations. In its simplest form such rules might be defined statically called as policies. Statically defined roles need to be adapted in case the managed system accommodates new objects created dynamically, e.g. due to a network expansion. The new objects are filtered as part of a criteria. By augmenting roles with policies and criteria, the roles are able to associate to static and dynamically created objects. This document proposes to introduce policies and criteria and associate it to roles. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarity on roles to policies and criteria usecase |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clarity on the roles to policies and criteria |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the input to the Rel-18 28.533 DraftCR for S2-227070 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

4.9.x.1.4	 Roles to policies and criteria association

Role-based access control offers an abstraction layer by defining roles as set of fine grained access rights to resources (managed objects) and corresponding operations.

In its simplest form such rules might be defined statically called as policies. Statically defined roles need to be adapted in case the managed system accommodates new objects created dynamically, e.g. due to a network expansion. The new objects are filtered as part of a criteria.

By augmenting roles with policies and criteria, the roles are able to associate to static and dynamically created objects.

On the other hand if combinations of resources and operations referred to as policies, were directly associated to roles, then it would be cumbersome for any change for an administrator in case of duplication or small changes to a role.

Hence the role contains the overall distribution of resources and operations it can work on by resolving the criteria and policies.

### 4.9.x.1.4.1 	Criteria creation

Filters that should be created in order to group managed entities according to a particular condition and are resolved dynamically at runtime

- The condition works on instances of the managed entities.	- e.g: Set of radio NEs in the south region

- Criteria will be resolved by the management function based on the conditions

- Criteria should be resolved before storing it in the authorisation server

The below sequence shows the criteria being created by an operator to identify dynamic instances of a managed objects according to a condition.

Figure 4.9.x.1.4.1 - 1 - Sequence diagram of criteria

#### 4.9.x.1.4.2 	Roles to policies and criteria

Policies and criteria should be associated to roles. The below sequence shows the operator who could be an administrator carrying out the same.

Figure 4.9.x.1.4.1 - 2 - Sequence diagram of associating policies and criteria to roles

End First change

Second change

### 4.9.x.2    Requirements

- UC-MSAC-0x - 3GPPP management system shall support role based access control for the resources represented by the MnS component A, B and C.

- UC-MSAC-0x.03 - 3GPP management system shall support roles which are combination of policies and criteria

End of second change