3GPP TSG-SA5 Meeting #147	S5-232679

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0069 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR for TS28.535 delete redundant monitoring content of the management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Redundant monitoring content of the management system |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the key performance indicators |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cause confusing problem for the monitoring content of the management system |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 6.1.1, 6.1.9 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

6.1.1	Communication service quality assurance and optimization

The goal of the use case is to enable communication service quality assurance and optimization for the set of services provided by the network to certain group (category) of UEs. For example, the set can include the communication services provided via certain NSI(s) or to IoT devices in certain area.

The group of NG-RAN and 5GC nodes (deployed and active), which are essential for the set of E2E services, provide provisioning and PM management services. It is also assumed that the providers of the related NSI / NSSI provisioning and PM management services are deployed and active.

The management system is consuming the afore mentioned management services either directly or through proxy nodes that re-expose the management services; the management system is aware of the performance requirements imposed on the set of communication services.

The management system is collecting the service experience information and monitoring the KPIs, related to the targeted services. Analytics hosted by the MDAF may be utilized for processing of the network data to derive and analyse the KPIs. If the service quality assurance and optimization function detects performance degradation the 3GPP management system may continuously modify the configuration parameters in the corresponding NG-RAN and 5GC nodes and NSI(s)/NSSI(s), to satisfy the SLA requirement. In case that changes of communication service SLS are needed, those changes may result as input to the 3GPP management system.

If the network performance does not recover or improve, the management system may further adjust the network configuration, or roll back to the previous configuration. At all times the management system continues to collect the network data and to monitor the performance indicators.

The Second Change

6.1.9	Targeted Assurance Closed Control Loop

The goal of this use case is to provide the consumer of an assurance closed loop the ability to set targets (example location, UEs) per assurance goal(s) of an ACCL. This implies that an ACCL will be assuring goal(s) pertaining to the configured targets only. The SLS requirement may come for a particular location. It would be beneficial if the ACL can target a particular location while assuring the SLS. Strict SLS assurance for a service can be provided using ACL for a high-valued vertical customer present in a particular location. This will require configuring the ACL accordingly and would need ACL to collect the performance measurements, while monitoring the network, for the same target location. The example of this may include SP assuring assurance goal of guaranteed throughput for an eMBB service inside a customer premises.

End of changes