3GPP TSG-SA5 Meeting #147	S5-232707

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

# Source:	Samsung

# Title:	pCR 28.903 Trigger Key Issues.docx

# Document for:	Approval

Agenda Item:	6.8.2.9

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides Trigger relayed Key Issues and requirements.

# 4	Detailed proposal

First modification

## 4.5	Issue #x: Functions Trigger

### 4.5.1	Issue description

The generic NRM defines several NRM control fragments for various purpose. Some of the fragments are related and should be instantiated in a sequence to achieve the overall objective.  For example, a consumer requesting to instantiate PerfMetricJob or TraceJob should also request to instantiate NtfSubscriptionControl in order to receive related notification with the required performance metrics. In another example, a ThresholdMonitor will require a related PerfMetricjob which in turn will require a NtfSubscriptionControl. A consumer need to explicitly request for all three. Further, one particular NRM fragment can be triggered by an event related with another instantiated MOI. For example, when a threshold is breached the PerfMetricJob is created to collect related measurement data. This can be done in order to ascertain the cause of the threshold breach. For example, when the slice threshold of “UTSNSI” (see clause 6.2.3 of TS 28.554) is breached a PerfMetricJob is created to collect measurement of “Virtualized resource usage” (see clause 6.2 of TS 28.552) related to that slice. In the above example, the consumer need to explicitly create PerfMetricJob as a result of threshold crossing.

It is not optimal for consumer to explicitly request for the instantiation of all required NRM fragments. The producer should instantiate all required NRM fragments, in a particular sequence, with a single request i.e the request to instantiate PerfMetricJob and ThresholdMonitor in the above examples. The producer should also be capable to trigger a particular function on a network event. The current standards lacks a mechanism for a producer to know what are the related NRM fragments that need to be instantiated, in a sequence, after the parent NRM fragments is instantiated and what NRM fragments can be created at what network events.

### 4.5.2	Analysis

The mechanism to support in-sequence and on-trigger creation of the functions are missing and need to be defined.

### 4.5.3 Potential Requirements

- The 3GPP Management System shall enable a mechanism using which an authorized consumer, while asking to create an MOI, can specify details on the IOC(s) that are to be instantiated next.

- The 3GPP Management System shall enable a mechanism using which an MOI can be created based on some network events (e.g threshold crossing) without explicit request from the consumer.

### 4.5.2 Potential Solutions

TBD

### 4.5.3	CR proposal

TBD