| 3GPP TR 28.826 V1.3.0 (2023-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Nchf charging services phase 2 improvements and optimizations (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

5	Charging scenarios and key issues	9

5.1	Charging information optimization	9

5.1.1	General	9

5.1.2	Use cases	9

5.1.2.1	Use Case #1a: Trigger without usage	9

5.1.2.2	Use Case #1b: Size of charging information	10

5.1.2.3	Use Case #1c: Charging Data Request Optimization	10

5.1.2.4	Use Case #1d: Undefined RAT type	10

5.1.2.5	Use Case #1e: Charging information accuracy of location	10

5.1.2.6	Use Case #1f: Bitrate charging	10

5.1.2.7	Use Case #1g: Charging information for threshold based re-authorization triggers	10

5.1.3	Potential charging requirements	11

5.1.4	Key issues	11

5.1.5	Solutions	11

5.1.5.1	Solution #1.1: New immediate trigger variant	11

5.1.5.2	Solution #1.2: Keep rating group information together	15

5.1.5.3	Solution #1.3: NF consumer ensuring accuracy of location	15

5.1.5.4	Solution #1.4: Stop and discard the used units during re-authorization	15

5.1.5.5	Solution #1.5: Stop and report the used units during re-authorization	15

5.1.5.6	Solution #1.6: CHF control of granted quota during re-authorization	16

5.1.5.7	Solution #1.7: Continue and report the used units during re-authorization	16

5.1.5.8	Solution #1.8: CHF control of ChargingInformationData size	16

5.1.5.9	Solution #1.9: CHF control of chargingdata size	17

5.1.5.10	Solution #1.10: Bitrate Charging reporting from SMF	18

5.1.5.11	Solution #1.11 New attribute on all levels of the CDR structure	19

5.1.5.11.1	Undefined attribute	20

5.1.5.12	Solution #1.12 New attribute in the ChargingRecord, using sequence number for multiple attributes	20

5.1.5.12.1	Undefined Information	20

5.1.5.13	Solution #1.13 New attribute in the ChargingRecord, using hash value for multiple attributes	21

5.1.5.13.1	Undefined Information	22

5.1.5.14	Solution #1.14: NF consumer reporting accuracy of location	22

5.1.5.15	Solution #1.15: IoT Charging information Optimization	23

5.1.6	Evaluation	23

5.1.7	Conclusions	24

5.2	Charging request optimization	24

5.2.1	General	24

5.2.2	Use cases	24

5.2.2.1	Use Case #2a: Operator defined trigger	24

5.2.3	Potential charging requirements	24

5.2.4	Key issues	25

5.2.5	Solutions	25

5.2.5.1	Solution #2.1: Reuse of management intervention trigger	25

5.2.5.2	Solution #2.2: New operator defined trigger	25

5.2.6	Evaluation	25

5.2.7	Conclusions	25

5.3	Rating input enhancement	25

5.3.1	General	25

5.3.2	Use cases	26

5.3.2.1	Use Case #3a: Enhancement of input to CHF rating	26

5.3.2.2	Use Case #3b: Service Charging Enhancement	26

5.3.2.3	Use Case #3c: Switching between online and offline	26

5.3.3	Potential charging requirements	28

5.3.4	Key issues	28

5.3.5	Solutions	29

5.3.5.1	Solution #3.1: Enhancement of multiple unit usage with service identifier	29

5.3.5.2	Solution #3.2: Enhancement of requested unit with service identifier	29

5.3.5.3	Solution #3.3: The QoS information in RSU as the input for CHF rating	30

5.3.5.4	Solution #3.4: The Additional Information in RSU as the input for CHF rating	30

5.3.6	Evaluation	31

5.3.7	Conclusions	32

5.4	Chargeable events and sessions cancelling	32

5.4.1	General	32

5.4.2	Use cases	32

5.4.2.1	Use Case # 4a: Cancel failed event charging	32

5.4.3	Potential charging requirements	32

5.4.4	Key issues	32

5.4.5	Solutions	32

5.4.5.1	Solutions #4.1: Cancelling using charging session identifier	32

5.4.5.2	Solutions #4.2: Cancelling using a reference identifier for Charging Event	33

5.4.6	Evaluation	34

5.4.7	Conclusions	34

5.5	Non-blocking mechanism enhancement	34

5.5.1	General	34

5.5.2	Use cases	35

5.5.2.1	Use Case # 5a: CHF failed to grant quota	35

5.5.3	Potential charging requirements	35

5.5.4	The key issues	35

5.5.5	Solutions	35

5.5.5.1	Solution #5.1: Dedicated non-blocking indicator at service start	35

5.5.5.2	Solution #5.2: The non-blocking mode change from CHF to SMF	36

5.5.5.3	Solution #5.3: The non-blocking mode change from CHF to PCF via SMF	37

5.5.5.4	Solution #5.4: The non-blocking mode change per UE from CHF to PCF	37

5.5.5.5	Solution #5.5: Dedicated non-blocking indicator before service start	38

5.5.5.6	Solution #5.6: The Non-blocking mode change using policy counters from CHF to PCF	39

5.5.6	Evaluation	39

5.5.7	Conclusions	40

6	Documentation improvements	40

6.1	General	40

6.2 Use cases	41

6.2.1	Use Case # 6a: The binding description	41

6.2.2	Use Case # 6b: The simplification of CDR records	42

6.2.3	Use Case # 6c: Common IEs applied in the service charging specifications	42

6.2	Key issues	42

6.3	Solutions	42

6.3.1	Solution #6.1: Binding in stage2 and stage3 documents	42

6.3.2	Solution #6.2: Binding in TS 32.298	43

6.3.3	Solution #6.3: Binding in TS 32.291 and TS 32.298	43

6.3.4	Solution #6.4: Separate the CDR records	43

6.3.5	Solution #6.5: Complete the Common IE applied	43

6.3.6	Solution #6.6: Partial the Common IE applied	44

7	Conclusions and recommendations	46

Annex A (informative): Change history	47

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This report is to study enhancement of the Nchf converged charging service.

# 1	Scope

The present document studies the potential use cases, requirements, and solutions for the enhancements of the Nchf converge charging service.

The study will cover and identify gaps for:

-	optimizations regarding amount of information in the request and number of requests;

-	enhancement of input to rating;

- 	cancelling of chargeable events for unsuccessful scenarios;

- 	enhancement of non-blocking mechanism;

-	enhancements of triggers having more consistent triggers and use of NF specific triggers.

Improvements on how to maintain the source code presentations (yaml and ASN.1) the document structure for these will be studied,

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 32.255: "5G data connectivity domain charging; stage 2".

[3]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[6]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[7]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[8]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[9]	3GPP TS 23.501: "3GPP TS 23.501:"System Architecture for the 5G System".

[10]	3GPP TS 23.203: "Policy and charging control architecture".

[11]	3GPP TS 29.594: "5G System; Spending Limit Control Service; Stage 3".

[12]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[13]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[14]	IETF RFC 8506 (2019): "Diameter Credit-Control Application".

[15]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[16]	3GPP TS 29.594: "5G System; Spending Limit Control Service; Stage 3".

[17]	3GPP TS 32.254: “Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Overview

The Nchf converged charging services was first released in Rel-15. This first iteration was to a large extent, with some minor exceptions, copying and combining its predecessors, Ro and Rf, based on Diameter to a service based interface (SBI) supporting converged charging service. The depiction of the internal architecture of the Converged Charging System (CCS) was to a large extent based on the Online Charging System (OCS).

The combination of the Diameter Ro and Rf interfaces into one converged charging service together with the move of the integration point for CDRs have led to that all the information previously reported and transported over two or more interfaces now needs to be transported over a single interface. This makes it essential that the converged charging interface is optimized in regard to the amount of information and number of reports sent, or at least allows for such an optimization if needed.

The information that may be used to assist the determine of reservation in the CCS is not defined except for the rating group. This means that for an immediate event or in an initial request the information about the event or session is limited and lacks accuracy.

Low latency requirements increased by new business cases may need enhancements to the non-blocking mechanism.

Charging of new service together with converged charging have increased the need for more information in the input to rating.

The use of event-based charging has led to a need for cancelling the charging for the chargeable event, e.g. refund, for unsuccessful scenarios or cases not possible to fulfil.

The document structure of the actual service, i.e. yaml, followed the handling of Ro and Rf, based on Diameter, which may need improvements.

# 5	Charging scenarios and key issues

## 5.1	Charging information optimization

### 5.1.1	General

When there is no 5G data connectivity usage, which may be quite frequent in the case of always connected, when a trigger is active and the event occurs, sending a lot of information might be unnecessary. For online the need of getting the trigger even if there is no usage depends on if it requires rating and may impact the granted quota. For the offline it will depend on the use case if the trigger event is needs to be reported in the case of no usage.

The size of a request is only limited by the maximum size of the request in TS 29.500 [4] and controlled by the NF consumer. The CHF may reject the request if the request is over that limit. There is no other way for the CHF to control the size of the message other than controlling or limit the number of charging conditions changes before reporting, this will not state the size or take compression into consideration.

The CDR specification TS 32.298 [7] and the Nchf specification in TS 32.291 [6] may not always be updated to support the same information, this especially applies to attributes that is inherited from other specifications in TS 32.291 [6]. If the attribute or attribute value isn’t supported by the CDR and it is optional the attribute will be discarded. This would mean that even if it is supported in a later stage of the TS 32.298 [7], the value cannot be restored.

The reported charging information accuracy directly impact on the charging accuracy in the CHF. The charging information accuracy of location should be taken into consideration.

When there is the tiered charging per bitrate, for 5G converged charging, the volume aggregation based on the range of bitrate as the charging information should be taken into consideration.

### 5.1.2	Use cases

#### 5.1.2.1	Use Case #1a: Trigger without usage

The operator has an offer for 5G data connectivity, which is volume based and available for both online and offline (in converged charging this means quota and not quota managed) charged subscribers. The offer has different rates depending on the PLMN accessed i.e., if it is roaming and where it is roaming. The operator would also like to get statics on how much the users are using in different PLMNs, as well as for some IoT devices how many times they change PLMN.

A user has a UE that has a PDU session ongoing and is on the border between HPLMN and VPLMN and the UE keeps switching between the networks. The user chooses to only use the UE when it is connected to the HPLMN.

An enterprise has several IoT devices that each have their PDU sessions ongoing.

The operator would like to minimize the number of interrogations and the information sent between the SMF and CHF, this means that this means that for:

-	UEs that are online (quota managed) charged and have the offer, a PLMN change should only immediately trigger a CHF when there is usage.

-	UEs that are offline (not quota managed) charged and have the offer, a PLMN change should create a container only when there is usage.

-	some IoT devices that are offline (not quota managed) charged, a PLMN change should create a container independent of if there is usage or not.

This could be relevant for the following triggers: QoS, GFBR guaranteed status, user location, serving node, Presence Reporting Area(s), 3GPP PS data off status, tariff time, UE time zone, PLMN, RAT type, Session-AMBR, UPF, I-SMF, handover status, access addition/removal and redundant transmission change.

#### 5.1.2.2	Use Case #1b: Size of charging information

The operator has a number of CHFs wants to optimize their storage and would therefore like to control the size of the converged charging requests based on the available space in each CHF. It should be noted that there is a trade of between size of the requests and the number of requests.

There are two parts of to the storage in the CHF one is the CDR storage before sending it to the CGF and the allocation of the resource for the SBI. The resource may be stored uncompressed for fast access, whereas the charging information in the converged charging request may be compressed to limit network utilization. This means that the operator would like to control the size of the message both before and after the compression.

When a converged charging request is sent the amount of information included in the intimal is often limited since not much usage can have been done, so it is more important to be able to control the sizes of the update and termination than the initial.

The converged charging request can contain request for units, offline unit reports and online units report for the same rating group.

#### 5.1.2.3	Use Case #1c: Charging Data Request Optimization

The operator has an offer for 5G Data Connectivity, which is volume based, though, this needs to be clearly optimized due to the expected number of IoT devices which an enterprise can have. Each one of the IoT devices may have a PDU session which could lead a major 5GS overload due to burst of Charging Data Requests.

The operator would like to minimize the number of interrogations and the information sent between the SMF and CHF, by:

-	Simplify charging for loT devices

- 	Protect 5GS from charging system overload due to many CDRs

#### 5.1.2.4	Use Case #1d: Undefined RAT type

An End User (subscriber) uses a service with a RAT type has that isn’t supported by the CDR. The SMF reports the usage with the RAT type. The CHF will not be able to store the RAT type in the CDR and since it is optional it may discard the value.

#### 5.1.2.5	Use Case #1e: Charging information accuracy of location

The operator has an offering service for subscribers, e.g., 5G data connectivity service specified in the TS 32.255 [2], which is volume based charging. During the 5G data connectivity service consumption, the charging information is collected by the SMF and reported to the CHF when the triggers is encountered. That means whether the CHF can accurately charge for subscribers depends on the received charging information from SMF is correct or not.

#### 5.1.2.6	Use Case #1f: Bitrate charging

The operator has an offer for 5G data connectivity services charged subscribers to provide the tiered charging per bitrate.

The end user subscribes the volume/time based tiered charging based on the averaged bitrate per time interval with special range (High speed/middle or low downlink or uplink). In order to make sure the user experience, the 5G system collects the statistics on traffic volume by averaged bitrate per time interval to implement tiered charging per bitrate.

Account the volume or time based on the range for the averaged bitrate per time interval.

#### 5.1.2.7	Use Case #1g: Charging information for threshold based re-authorization triggers

When received quota threshold based re-authorization triggers (i.e. timeQuotaThreshold, volumeQuotaThreshold, unitQuotaThreshold), the NF consumer shall seek re-authorization for the quota when fall below the supplied threshold. The NF consumer allows the service to continue whilst the re-authorization is progress, until the remaining part had been used up. The description in the TS 32.290 [12] clause 5.4.2 about the Threshold based re-authorization triggers is unclear.

### 5.1.3	Potential charging requirements

REQ-CH_INFO-01: Optimization of charging information may be needed when zero units used.

REQ-CH_INFO-02: Support for control of charging request size.

REQ-CH_INFO-03: Optimization of charging information for small data delivery cases.

REQ-CH_INFO-04: Possibility to support undefined attributes and values, for later usage, in the CDR.

REQ-CH_INFO-05: Support for the optimization of charging information accuracy of location.

REQ-CH_INFO-06: Support for the tiered charging based on averaged bitrate per time interval.

REQ-CH_INFO-07: Support for control of remaining grant quotas during the re-authorization.

### 5.1.4	Key issues

Key Issue #1a: Identify the charging scenarios and triggers which can be controlled when there is no usage.

Key Issue #1b: How to control if information is sent when there is no usage.

Key Issue #1c: How to control if containers are created when these is no usage.

Key Issue #1d: How to control the size of the information in the converged charging request, dependent or independent of compression.

Key Issue #1e: How to handle when unit request and report for one rating group are split over several requests

Key Issue #1f: Identify the charging scenarios for small data delivery

Key Issue #1g: How to optimize small data delivery charging requests

Key Issue #1h: Where in CDR structure to store undefined values

Key Issue #1i: How to determine where the attribute belongs in the CDR structure

Key Issue #1j: How to ensure the charging information accuracy of location.

Key Issue #1k: How to handle tiered charging based on averaged bitrate per time interval.

Key Issue #1l: How to handle the remaining granted quotas during the re-authorization.

### 5.1.5	Solutions

#### 5.1.5.1	Solution #1.1: New immediate trigger variant

A possible solution for key issues #1a, #1b, and #1c covering requirements REQ-CH_INFO-01, charging information optimization.

This solution adds a variant of the immediate trigger category.

For the current immediate report chargeable events for which, when occurring, charging data generated by the NF consumer and sent to the CHF in a Charging Data Request message. Zero usage may be reported.

Figure 5.1.5.1-1: Message flow for current immediate triggers

1)	Service delivery ongoing: there is a connection setup for service delivery and a charging session is established between the NF (CTF) and the CHF.

2)	Trigger event: the NF (CTF) generates charging information related to the service delivered, due to that an immediate trigger for usage reporting or quota request is met.

3)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the CHF reporting the used units and if the service is under quota management it includes a quota request for the service to continue.

4)	Account, rating, and reservation control: The CHF performs the process related to the reported usage and if the service is under quota management it performs processes related to quota requested, it may involve the rating entity and user's account balance.

5)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

6)	Charging Data Response [Update, Quota granted]: The CHF responds and if the service is under quota management it includes a quota granted for the service to continue.

7)	Uplink/downlink transmission: first data or events transmission after the trigger event.

8)	Service delivery continuing service delivery is continuing and the charging session is continuing between the NF (CTF) and the CHF.

For the deferred report chargeable events for which, when occurring, charging data generated by the NF consumer and stored. Zero usage may be included. The charging data will be sent to the CHF in next a Charging Data Request message.

Figure 5.1.5.1-2: Message flow for deferred triggers

1)	Service delivery ongoing: there is a connection setup for service delivery and a charging session is established between the NF (CTF) and the CHF.

2)	Trigger event: the NF (CTF) generates charging information related to the service delivered, deferred trigger for usage reporting is met.

3)	Close counts and start new counts: the NF (CTF) close all counts related to the service and starts new, keeping the closed counts for reporting when an immediate trigger occurs.

4)	Uplink/downlink transmission: first data or events transmission after the trigger event.

5)	Service delivery continuing service delivery is continuing and the charging session is continuing between the NF (CTF) and the CHF.

For the new immediate report chargeable events for which, when occurring and there is usage, charging data generated by the NF consumer and sent to the CHF in a Charging Data Request message. Zero usage may be reported.

Figure 5.1.5.1-3: Message flow for new immediate triggers

1)	Service delivery ongoing: there is a connection setup for service delivery and a charging session is established between the NF (CTF) and the CHF.

2)	Trigger event: the NF (CTF) notices that a new immediate with "zero data transmission without reporting" indicator is met. If there is zero usage, NF(CTF) will ignore the reporting for this trigger.

NOTE: which triggers are applicable for the immediate trigger "zero data transmission without reporting" should be specified in the normative work. If the immediate trigger without the "zero data transmission without reporting" indicator, the charging data message should be reported even zero usage for the immediate triggers.

3)	Uplink/downlink transmission: There are data or events transmission start after the immediate trigger "zero data transmission without reporting ", was met, and the NF (CTF) generates charging information related to the service delivered.

4) Trigger event: the NF (CTF) notices that new immediate triggers (e.g. Start of service data flow, in case no valid quota for this rating group) are met, the NF(CTF) generates charging data request message with the new immediate trigger and timestamp, the previous immediate trigger "zero data transmission without reporting" in step 2 is ignored. If there are no new immediate trigger are met, NF(CTF) generates the charging data request with the previous immediate trigger "zero data transmission without reporting" in step 2 and the timestamp in the step 4 to request the new quota and reports the used unit.

5)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the CHF reporting the used units with the trigger and if the service is under quota management it includes a quota request for the service to continue.

6)	Account, rating, and reservation control: The CHF performs the process related to the reported usage and if the service is under quota management it performs processes related to quota requested, it may involve the rating entity and user's account balance.

7)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

8)	Charging Data Response [Update, Quota granted]: The CHF responds and if the service is under quota management it includes a quota granted for the service to continue.

9)	Service delivery continuing service delivery is continuing and the charging session is continuing between the NF (CTF) and the CHF.

#### 5.1.5.2	Solution #1.2: Keep rating group information together

A possible solution for key issue #1e covering requirements REQ-CH_INFO-01, charging request optimization.

If the rating group have service with quota management (online), then the charging data request containing a quota request it must include all used units for that rating group, including the quota management(online) and without quota management(offline). Only the quota management services reported will be used for quota allocation. This is to ensure that the CHF won’t interpret any reporting of used units for the rating group as a release of all granted service units for the rating group.

#### 5.1.5.3	Solution #1.3: NF consumer ensuring accuracy of location

A possible solution for key issue #1j covering requirements REQ-CH_INFO-05, charging information accuracy of location.

Add a general description in the TS 32.240 [5] in line with the following:

-	the NF (CTF) (e.g., SMF, AMF, NEF) ensures that location reporting in the charging information is accurate for charging purposes.

#### 5.1.5.4	Solution #1.4: Stop and discard the used units during re-authorization

A possible solution for key issue #1l covering requirements REQ-CH_INFO-07, control of granted quota during re-authorization.

The NF Service Consumer allows the service to continue whilst the re-authorization is progress, using the quota between the threshold and the granted quota. Either until the quota has been consumed or receiving a charging data response from CHF.

If the remaining part is used up before receiving the Charging Data Response:

-	NF Service Consumer stops the service delivery and discards the recorded used units.

If the Charging Data Response with new quota is received before the quota is consumed:

-	NF Service consumer continues the service delivery, deducting the used units from the new granted quota.

If the Charging Data Response with result code other than success or a final unit action, described in TS 32.290 [12], is received before the quota is consumed:

-	the NF Service consumer stops service delivery, discards the recorded used unit, and performs the actions associated with the result code or final unit action.

#### 5.1.5.5	Solution #1.5: Stop and report the used units during re-authorization

A possible solution for key issue #1l covering requirements REQ-CH_INFO-07, control of granted quota during re-authorization.

Clarify the description in clause 5.4.2 of TS 32.290 [12]. The NF Service Consumer allows the service to continue whilst the re-authorization is progress, until the remaining part had been used up.

The NF Service Consumer allows the service to continue whilst the re-authorization is progress, using the quota between the threshold and the granted quota. Either until the quota has been consumed or receiving the charging data response from CHF.

If the remaining part is used up before receiving a Charging Data Response:

-	NF Service Consumer terminates the service date flow and reports the used unit in the next Charging Data Request.

If the Charging Data Response with new quota is received before the quota is consumed:

-	NF Service consumer continues the service delivery, deducting the used units from the new granted quota.

If the Charging Data Response with result code other than success or a final unit action, described in TS 32.290 [12], is received before the quota is consumed:

-	the NF Service consumer stops service delivery, reports the used unit in a Charging Data Request [Update/Termination], and performs the actions associated with the result code or final unit action.

#### 5.1.5.6	Solution #1.6: CHF control of granted quota during re-authorization

A possible solution for key issue #1l covering requirements REQ-CH_INFO-07, control of granted quota during re-authorization.

The quota is granted per RG. There may be several RGs in one charging session. The failure of granted quota for one RG will not impact on the quota granted for other RGs. For one special RG, the following mechanism is present.

When received quota threshold based re-authorization triggers (i.e. timeQuotaThreshold, volumeQuotaThreshold, unitQuotaThreshold), the NF consumer shall seek re-authorization for the quota when fall below the supplied threshold.

When the granting new quota is successful from the CHF, the used units during the re-authorization process are treated as the used part of the new granted quota.

When the granting new quota is unsuccessful from the CHF, the CHF may indicate that the remaining quota can be used up, the NF consumer shall allow the service to continue until the remaining part is used up, and then report the usage of remaining quota with the quota management indication. Otherwise, the NF consumer stops the service after receiving failure response if no indication from CHF.

#### 5.1.5.7	Solution #1.7: Continue and report the used units during re-authorization

A possible solution for key issue #1l covering requirements REQ-CH_INFO-07, control of granted quota during re-authorization.

The NF Service Consumer allows the service to continue whilst the re-authorization is progress, using the quota between the threshold and the granted quota. Either until the quota has been consumed or receiving a Charging Data Response from CHF.

If the remaining part is used up before receiving a Charging Data Response:

-	NF Service Consumer stops the service delivery and waits for the Charging Data Response. When the Charging Data Response is received it will follow the same actions as if it was received before the quota was consumed, see next two if-statements.

If the Charging Data Response with new quota is received before the quota is consumed:

-	NF Service Consumer continues the service delivery, deducting the used units from the new granted quota.

If the Charging Data Response with result code other than success or a final unit action, described in TS 32.290 [12], is received before the quota is consumed:

-	the NF Service Consumer stops service delivery and reports the recorded used unit in a Charging Data Request [Update/Termination], and performs the actions associated with the result code or final unit action.

#### 5.1.5.8	Solution #1.8: CHF control of ChargingInformationData size

A possible solution for key issues 1d covering requirements REQ-CH_INFO-02, control of charging request size.

CHF can provide the max data of charging information collection in the NF(CTF) to limits the charging information data size that NF(CTF) can use in the charging information reporting, if the corresponding supported feature is supported.

Example of maxzise in the Charging Data Response in the TS 32.291 [6] table 6.1.6.2.1.2:

Table 6.1.6.2.1.2-1: Definition of type ChargingDataResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| invocationTimestamp | DateTime | M | 1 | This field holds the timestamp of the charging service response from the CHF. |  |
| invocationResult | InvocationResult | OC | 0..1 | This field holds the result of charging service invocation by the NF consumer |  |
| invocationSequenceNumber | Uint32 | M | 1 | This field contains the sequence number of the charging service invocation by the NF consumer. The same value of the sequence number received in the request should be used in the response |  |
| sessionFailover | SessionFailover | OC | 0..1 | This field indicates whether alternative CHF is supported for ongoing charging service failover handling by NF consumer. |  |
| supportedFeatures | SupportedFeatures | OC | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| multipleUnitInformation | array(MultipleUnitInformation) | OC | 0..N | This field holds the parameters for the quota management and/or usage reporting information. It may have multiple occurrences. |  |
| Triggers | array(Trigger) | OC | 0..N | This field identifies the chargeable event(s) supplied by CHF to override/activate the existing chargeable event(s) in NF consumer. The presence of the triggers attribute without any triggerType is used by CHF to disable all the triggers except rating group level triggers. |  |
| maxChargingData | Uint64 | OC | 0..1 | Maximum payload size if needed by CHF and the supported feature “ MaxChargingData” is applicable. The fileld will be effective until next resetting of maxChargingData. | MaxChargingData |

#### 5.1.5.9	Solution #1.9: CHF control of chargingdata size

A possible solution for key issues #1d covering requirements REQ-CH_INFO-02, control of charging request size.

The CHF can today set the following limits for a request to trigger reporting of charging information:

-	time: limits the duration the CTF can wait before reporting.

-	volume: limits the volume measured by the CTF before reporting.

-	event: limits the number of events measured by the CTF before reporting.

-	Number Of CCC: limits the number of charging condition changes performed CTF before reporting.

This solution would add two new limits the size of payload (before and after any compression) of the Charging data request:

-	charging data: limits the charging data size that CTF can use in the charging data report before compression is applied, if any.

The new attribute e.g., maxChargingData, would be handled in the same way as the current maxNumberOfccc.

Example of how the definition of trigger could look in the TS 32.291 [6] table 6.1.6.2.1.7-1:

Table 5.1.5.9-1: Definition of type Trigger

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| triggerType |  | TriggerType |  | Oc |  | 0..1 |  | the events whose occurrence lead to charging event is issued towards the CHF |  |  |  |
| triggerCategory |  | TriggerCategory |  | M |  | 1 |  | This field indicates whether the charging data generated by the NF consumer for the trigger lead to a Charging Event towards the CHF immediately or not. |  |  |  |
| timeLimit |  | DurationSec |  | OC |  | 0..1 |  | Time limit if trigger type is “Expiry of data time limit” |  |  |  |
| volumeLimit |  | Uint32 |  | OC |  | 0..1 |  | Volume limit if trigger type is “Expiry of data volume limit”. This attribute is not valid from Nchf_ ConvergedCharging API version v2.0.0 |  |  |  |
| volumeLimit64 |  | Uint64 |  | OC |  | 0..1 |  | Volume limit if trigger type is “Expiry of data volume limit”. This attribute replaces the volumeLimit attribute from Nchf_ ConvergedCharging API v2.0.0 |  |  |  |
| eventLimit |  | Uint32 |  | OC |  | 0..1 |  | Event limit if trigger type is “Expiry of data event limit”. |  |  |  |
| maxNumberOfccc |  | Uint32 |  | OC |  | 0..1 |  | Maximum number if trigger type is “Max nb of number of charging condition changes” |  |  |  |
| maxChargingData |  | Uint64 |  | OC |  | 0..1 |  | Maximum payload size without any compression, if trigger type is “Max charging data”. |  |  |  |
| tariffTimeChange |  | DateTime |  | OC |  | 0..1 |  | This field contains UTC time indicating the switch time when the tariff will be changed. |  |  |  |

#### 5.1.5.10	Solution #1.10: Bitrate Charging reporting from SMF

A possible solution for key issue #1k and requirement REQ-CH_INFO-06, Bitrate Charging.

The SMF collects the statistics on traffic volume by averaged bitrate per time interval to implement tiered charging per bitrate.

Figure 5.1.5.10-1: Message flow for SMF solution

- The SMF sends the Charging Data Request [Initial/Update] to CHF.

- CHF generates/updates the CDR based on the charging data request.

3)	CHF decides to start the tiered bitrate charging and determines the threshold for the tiered charging. Optional, the CHF can decide the time interval for the averaged bitrate accounting.

4)	CHF sends Charging Data Response [Initial/Update] to SMF with the threshold, e.g. threshold 1 is the bitrate more than100 M/bps, threshold 2 is bitrate between 100 M/bps and 50 M/bps, threshold 3 is bitrate lesser than 50 M/bps. Optionally, the time interval is 10 seconds if needed.

5)	If the time interval is provided, SMF sends the N4 request to the UPF with the time interval or SMF sends the threshold for the tiered charging.

6)	The UPF reports the traffic volume per time interval to SMF or traffic volume for tiered charging per threshold.

7)	SMF starts the new counts for the trigger “tiered charging threshold” and counts the volume based on the corresponding threshold when the deferred trigger “tiered charging threshold” is enable.

8)	When the immediate trigger is met for reporting, the SMF close the counts for the traffic data volume, ncluding the volume per threshold

9)	The SMF sends the Charging Data Request [Update, Used Units], e.g., the Used Units for threshold 1, the volume is 5 M, for threshold 2, the volume is10M, for threshold 3, the volume is 20 M, to the CHF. CHF processes the related charging data for CDR generation purpose.

10)	The CHF stores received charging information for the CDR.

11)	The CHF sends the Charging Data Response [Update] the SMF.

#### 5.1.5.11	Solution #1.11 New attribute on all levels of the CDR structure

A possible solution for key issues #1h and #1i covering requirement REQ-CH_INFO-04, support undefined attributes and values in CDR.

Adding a new attribute for storing undefined attributes and values, to all places in the CDR where an undefined attribute or attribute value could occur. This means that a new attribute would be added to all CDR attributes that is a group of attributes. An example on how this new attribute, undefinedAttribute, could be defined is described in clause 5.1.5.11.1.

##### 5.1.5.11.1	Undefined attribute

This field consists of two parts, the attribute name and value.

The attributeName would contain the name of the attribute e.g., “rATType”

The value would contain the actual the value of the attribute as a string, since all attributes (for openAPI) would be possible to represent as a string e.g., "NR_GEO".

Figure 5.1.5.11-1: Example of new attribute and usage for solution 1.11

#### 5.1.5.12	Solution #1.12 New attribute in the ChargingRecord, using sequence number for multiple attributes

A possible solution for key issues #1h and #1i covering requirement REQ-CH_INFO-04, support undefined attributes and values in CDR.

Adding a new attribute or sequence of attributes directly in the ChargingRecord for storing undefined attributes and values. To be able to find in which part of the CDR the attribute belongs both the full resource path as well as a sequence number is needed, the sequence number would refer to which array item the undefined attribute belonged in the case of it being with in an attribute having multiple occurrences e.g., a “SEQUENCE OF”, in the CDR.

An example on how this new attribute, undefinedInformation, could be defined is described in clause 5.1.5.12.1.

##### 5.1.5.12.1	Undefined Information

This field would be array of the undefined attribute as defined in clause 5.1.5.11.1 with the addition of a sequence number reference.

The reference sequence number would identify the specific occurrence in a sequence that the undefined attribute would belong to e.g., LocalSequenceNumber in the MultipleQFIContainer and UsedUnitContainer. If a sequence number doesn’t exist for a specific attribute that is a sequence of, this would have to be added.

Figure 5.1.5.12-1: Example of new attribute and usage for solution 1.12

#### 5.1.5.13	Solution #1.13 New attribute in the ChargingRecord, using hash value for multiple attributes

A possible solution for key issues #1h and #1i covering requirement REQ-CH_INFO-04, support undefined attributes and values in CDR.

Adding a new attribute or sequence of attributes directly in in the ChargingRecord for storing undefined attributes and values. To be able to find in which part of the CDR the attribute belongs both the full resource path as well as a hash value for the specific array occurrence, the hash value would be calculated from the attributes and values of a specific array item and thereby uniquely identifying this array item.

The ChargingRecord would contain a new field undefinedInformation defined in clause 5.1.5.13.1.

##### 5.1.5.13.1	Undefined Information

This field would be an array of undefined attribute as defined in clause 5.1.5.11.1 with the addition of a reference hash value reference.

The reference hash value reference would uniquely identify the specific occurrence in a sequence where the undefined attribute would belong. For example, if an attribute could not be added to one item in the sequence of NGRANSecondaryRATUsageReports then the hash value would be created based on the attributes and values that could be stored in that specific occurrence in the sequence.

Figure 5.1.5.13-1: Example of new attribute and usage for solution 1.13

#### 5.1.5.14	Solution #1.14: NF consumer reporting accuracy of location

A possible solution for key issue #1j covering requirements REQ-CH_INFO-05, charging information accuracy of location.

Add a general description in the TS 32.240[5] in line with the following:

-	the NF (CTF) (e.g., SMF, AMF, NEF) includes in the location reporting the accuracy of the location information and time the location was retrieved by the NF (CTF) in the charging information, to be used by the CHF to determine if the accuracy meets current requirements.

#### 5.1.5.15	Solution #1.15: IoT Charging information Optimization

A possible solution for key issues #1f and #1g covering requirements REQ-CH_INFO-03, optimization of charging information.

Currently, charging for data transfer is based on volume based measurement of the connections in down and uplink directions which has been enhanced to cover big data transfers.

This solution would bring the following advantages for IoT Charging:

- Charging for loT subscriptions (which can be devices, group of devices, loT applications, external Identifiers) is simplified.

- Overload of charging system due to many CDRs, one CDR per payload, may be avoided, by reducing the number of requests to Charging, limit down the Charging Data to IoT related one.

- A new option to control loT devices and/or their environment.

The solution proposes the availability of a new count for the measurement and calculation of Non-IP Data Delivery of the Charging Data Request.

The new attributes Payload-Input-Count and Payload-Output-Count, would be used to indicate the number of times/events each one of an input payload for the subscription and an output payload from the subscription are delivered., it would subsequently generate the respective charging information, once it reaches a threshold leading to a reduction of Charging Requests towards CHF.

Editor’s note:	If the Payload-Input-Count and Payload-Output-Count should be seen as triggers is ffs.

Example of Payload-Input-Count and Payload-Output-Count in the Charging Data Request Message in the TS 32.254 [17] table 6.2a.1.2.1.1:

Table 5.1.5.15-1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57], and holds the identifier of the AF |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| Payload-Input-Count | OC | This field indicates a number of times the input payload for the subscription is delivered |
| Payload-Output-Count | OC | This field indicates the number of times the output payload from the subscription is delivered |
| One-time Event | OC | This field indicates, if included, that this is a one-time event and that there will be no update or termination. |
| One-time Event Type | OC | Described in TS 32.290 [12]. |
| Notify URI | - | This field is only applicable for the notification of abort charging. |
| Triggers | OC | This field is described in TS 32.290 [12] and holds the NEF specific triggers described in clause 5.4.1.x |
| Multiple Unit Usage | OC | This field contains the parameters for the quota management request and/or usage reporting. |
| Rating Group | M | Described in TS 32.290 [12] |
| Requested Unit | OC | Described in TS 32.290 [12] |
| Used Unit Container | OC | Described in TS 32.290 [12] |
| NEF API Charging Information | OM | This field holds the NEF API specific information described in clause 6.3.1.4 |

### 5.1.6	Evaluation

Solution #1.1 solve key issues #1a, #1b, and #1c.

-	Solution #1.1: introduces a new trigger, that will hold the charging data request until there is traffic or if there is no traffic the trigger will be ignored.

Solution #1.2 solve key issue #1e.

-	Solution #1.2: requires all used units reported for a rating group, both online and offline, to be delivered in the same request.

Solution #1.3 solve key issue #1j.

-	Solution #1.3: requires that the NF (CTF) provides the level of accuracy required by the CHF.

Solutions #1.4, #1.5, #1.6, and #1.7 all solve key issue #1l.

-	Solution #1.4: NF (CTF) stops service delivery if no response is received before all the quotas (between the threshold and the granted quota) have been used and discards used units in the case of termination is received.

-	Solution #1.5: NF (CTF) terminates the service if no response is received before all the quotas (between the threshold and the granted quota) have been used and reports the used units in a termination request. This means that it will in some cases terminate the service even though there are funds on the account.

-	Solution #1.6: NF (CTF) stops or continues service delivery if no response is received all the quotas (between the threshold and the granted quota) have been used depending on CHF indication and reports used units in the next request. This requires new functionality in both CHF and NF (CTF).

-	Solution #1.7: NF (CTF) stops service delivery and waits for response when all the quotas (between the threshold and the granted quota) have been used and reports used units in the next request. This is possible even if it is not specified.

Editor’s note: Further evaluation is FFS.

### 5.1.7	Conclusions

The solution #1.7 is selected for the normative work.

Editor’s note: Conclusions are FFS.

## 5.2	Charging request optimization

### 5.2.1	General

The number of requests from the network function to the CHF is controlled by the number events that require immediate triggering. This means that being able to have less immediate triggers will also minimize the number of requests. There is today a possibility to generate management intervention trigger immediately for events from management requiring reporting of unit usage immediately.

### 5.2.2	Use cases

#### 5.2.2.1	Use Case #2a: Operator defined trigger

An operator has set up some own defined triggers related to events generated by the network or management systems, these events does not require immediate reporting, just that the event is recorded, and the units counted before and after.

### 5.2.3	Potential charging requirements

REQ-CH_REQ-01: Possibility to support other than immediate trigger for own defined management or network triggers.

### 5.2.4	Key issues

The following key issues are identified:

-	Key Issue #2a: What trigger to use.

-	Key Issue #2b: What information to be included with the trigger, if any.

### 5.2.5	Solutions

#### 5.2.5.1	Solution #2.1: Reuse of management intervention trigger

A possible solution for key issues 2a and 2b covering requirements REQ-CH_REQ-01, charging request optimization.

There is no definition of the management intervention trigger, the only information is that it relates to a management event, it is always immediate, cannot be changed by the CHF and used to trigger an update.

Changing the management intervention trigger to allow the CHF to change the category as well as enable/disable the trigger.

#### 5.2.5.2	Solution #2.2: New operator defined trigger

A possible solution for key issues 2a and 2b covering requirements REQ-CH_REQ-01, charging request optimization.

Introduce a new trigger e.g., "trigger extension", which then the CHF could change the category as well as enable/disable the trigger. This could be seen as a new management intervention trigger, but would allow the operator to associate it with any type of chargeable event, including management interventions.

### 5.2.6	Evaluation

Solutions #2.1, and #2.2 both solve key issues #2a and #2b.

-	Solution #2.1: reuses the current triggers. It has already defined usage for operation management events, the usage of operation management triggers are related to management events and have no more information about why it was triggered therefore it may cause confusion as to why the trigger was considered met.

-	Solution #2.2: requires introduction of a new trigger. Separate the trigger from any other types of triggers.

Editor’s note: Further evaluation is FFS.

### 5.2.7	Conclusions

Editor’s note: Conclusions are FFS.

## 5.3	Rating input enhancement

### 5.3.1	General

A rating group is not defined in the context of SBI, it is however defined in TS 32.299 [8] as the same as the rating group of RFC 4006 [13] obsoleted by RFC 8506 [14] and linked to the charging key defined in TS 23.203 [10], the corresponding spec for SBI is TS 23.503 [3]. In TS 23.503 [3] the charging key is defined as "information used by the CHF for rating purposes".

The rating group gathers a set of services that is subject to the same cost and rates. One rating group can contain several rates if all rates are applicable to all services belonging to the rating group and if quota is granted it can be consumed by all services, belonging to the rating group, equally. How a service is identified is dependent on the network function.

This means that the cost and rates can be determined by the rating group but not the consumption rate of the quota i.e., how fast quota is used by the services belonging to the same rating group, and in the extension how much quota that should be reserved for a specific request.

### 5.3.2	Use cases

#### 5.3.2.1	Use Case #3a: Enhancement of input to CHF rating

A End User (subscriber) has subscribed to a service with multiple QoSs. The PCF dynamically decides to authorize a QoS based on the network information. When granting a quota, the CHF may need additional authorized QoS information corresponding to the Rating Group to assist CHF to grant quota per Rating Group.

As per the present document, the PDU charging information includes the default QoS information (Authorized QoS Information for PDU session and Subscribed QoS Information), which there is not the QoS information corresponding to the Rating Group.

The QoS information is an example as the input to the CHF rating.

Editor's note: The other information for the enhancement of input to CHF rating will be introduced case by case and ffs.

#### 5.3.2.2	Use Case #3b: Service Charging Enhancement

An End User (subscriber) subscribed to a service, once the service consumption is authorized for the End User, CHF can provide the right Rating Group to be used for service charging whilst the End User consumes it. Furthermore, this can be used for a clear improvement on the charging statistics and reporting.

As per the present document, the Charging Data Request [initial] message has the RequestedUnit Type which will be used by the CHF on the response, though the ServiceID type is not available. Service ID is only available in the UsedUnitContainer type (after initial Chargind Data Request message).

#### 5.3.2.3	Use Case #3c: Switching between online and offline

A UE has four services on going (A, B, C, and D). The NF providing the services (e.g., SMF) determines that these four services belong to three different rating groups (RGs), where service A belongs to RG 1, services B and C belongs to RG 2, and service D belongs to RG 3. It further determines that service A and B are always offline, while C and D are online, but may be set to quota management suspended.

Figure 5.3.2.3-1: Session based charging with suspend and resume of quota management.

1	Request for resource usage: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by the CHF to start.

2	Units Determination: the NF (CTF)) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3	Charging Data Request [Initial, Quota request RG2 and RG3]: The NF (CTF) sends the request to the CHF for the services C and D (i.e., RG 2 and 3), to be granted quota.

4	Account, rating, and reservation control, and open CDR: the CHF may open a CDR, rate the request, and check need for quota management. RG 2 will be granted quota while quota management will be suspended for RG 3.

5	Charging Data Response [Initial, Quota granted RG2, Result code RG3]: The CHF grants quota for RG 2 and gives a result code for RG 3 to indicating that quota management is suspended.

6	Service delivery starts and is ongoing: the NF (CTF) delivers the services.

7	Usage Reporting Trigger: the NF (CTF) generates charging data related to the services delivered, based on a trigger for usage reporting is met.

8	Charging Data Request [Update, Unit used, Quota request RG2 and RG3]: the NF (CTF) sends the request to the CHF, for quota to be granted for RG 2 and 3. It also reports the used units for services A, B, C, and D. In this case the QMI is reported for: A and B as offline, C as online, and D as suspended.

9	Account, rating, and reservation control, and update CDR: the CHF may update the CDR, rate the request, and check need for quota management. RG 3 will be granted quota while quota management will not be applicable for RG 2.

10	Charging Data Response [Update, Quota granted RG3, Result code RG2]: The CHF grants quota for RG 3 and gives a result code for RG 2 to indicating that quota management is not applicable.

11	Service delivery is ongoing: the NF (CTF) delivers the services.

12	Usage Reporting Trigger: the NF (CTF) generates charging data related to the services delivered, based on a trigger for usage reporting is met.

13	Charging Data Request [Update, Unit used, Quota request RG3]: the NF (CTF) sends the request to the CHF, for quota to be granted for RG 3. It also reports the used units for services A, B, C, and D. In this case the QMI is reported for: A and B as offline, C as offline, and D as online.

14	Account, rating, and reservation control, and update CDR: the CHF may update the CDR, rate the request, and check need for quota management. RG 3.

15	Charging Data Response [Update, Quota granted RG3]: The CHF grants quota for RG 3.

14	Session released: after some time, the session is released.

15	Charging Data Request [Termination, Units used]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units for services A, B, and D. In this acase the QMI is reported for: A and B as offline, C as offline, and D as online.

9	Account, and rating control, and close CDR: the CHF may close the CDR, and rate the request.

18	Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

In this case it is unclear how the quota management indicator (QMI) is influenced by the result code sent by the CHF.

### 5.3.3	Potential charging requirements

REQ-3GPPCH-ER-01	The 5G system should support the enhancement of input to CHF rating based on the QoS information.

REQ-3GPPCH-ER-02	The 5G system should support the enhancement of input to CHF rating based on the Service ID information.

REQ-3GPPCH_ER-03: The 5G system should support reporting QMI used for used unit.

### 5.3.4	Key issues

Key Issue #3a: Identification and classification of information which can be used as the input to CHF rating.

Key Issue #3b: Identify the Network Functions which can provide the input to support the CHF rating.

Key Issue #3c: Determine the interaction to support the enhancement of input to CHF rating.

Key Issue #3d: How QMI is influenced by the result code.

### 5.3.5	Solutions

#### 5.3.5.1	Solution #3.1: Enhancement of multiple unit usage with service identifier

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

Any information in the PCC rule could potentially be connected to a specific service identifier, this means that a specific QoS can be identified by the service identifier.

A solution could be to allow the service identifier in the MultipleUnitUsage as well as the rating group in the request for quota, to be able to allocate the right amount of quota needed at that moment for that rating group. The service identifier would in this case only be included as indicative i.e., which services that may be started. This means that both the service identifier(s) that triggered the request (if any) as well as the already started would be included in the request in the case of an update.

Table 5.3.5.1-1: Definition of type MultipleUnitUsage

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ratingGroup | RatingGroup | M | 1 | The identifier of a rating group. |  |
| serviceIdList | ServiceId | OC | 0..N | This field identity of the used service |  |
| requestedUnit | RequestedUnit | OC | 0..1 | This field indicates, if included, that quota management is required. It may additionally contain the amount of requested service units for a particular category. |  |
| usedUnitContainer | array(UsedUnitContainer) | OC | 0..N | This field contains the amount of used non-monetary service units measured. |  |

#### 5.3.5.2	Solution #3.2: Enhancement of requested unit with service identifier

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

Any information in the PCC rule could potentially be connected to a specific service identifier, this means that a specific QoS can be identified by the service identifier.

A solution could be to allow the service identifier in the requested unit, to be able to allocate the right amount of quota needed at that moment for that rating group. The service identifier would in this case only be included as indicative i.e., which services that may be started. This means that both the service identifier(s) that triggered the request (if any) as well as the already started would be included in the request in the case of an update.

Table 5.3.5.2-1: Definition of type RequestedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceIdList | ServiceId | OC | 0..N | This field identity of the used service |  |
| time | Uint32 | OC | 0..1 | This field holds the amount of requested time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of requested service specific units. |  |
| NOTE: f none of them is included, "RequestedUnit": {}, the category and amount is determined by CHF for online charging with centralized unit determination and rating scenario. |  |  |  |  |  |

#### 5.3.5.3	Solution #3.3: The QoS information in RSU as the input for CHF rating

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

The possible solution supports the potential requirements REQ-3GPPCH-ER-01 and partial supports Key Issue #3a, #3b and #3c to describe the enhancement of input to CHF rating.

The QoS information is the measurement of the overall performance of a service. A solution could be to allow the QoS information in the RequestedUnit as well as the rating group in the request for quota specified in the TS 32.291 [6], as the input for CHF rating consideration, to be able to assist to allocate the right amount of quota needed at that moment for that rating group.

Table 5.3.5.3-1: Definition of type RequestedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| time | Uint32 | OC | 0..1 | This field holds the amount of requested time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of requested service specific units. |  |
| QosInformation | QosData | OC | 0..1 | This field describes the QoS applied for the RG. | CHFRatingInput |
| NOTE: if none of them is included, "RequestedUnit": {}, the category and amount is determined by CHF for online charging with centralized unit determination and rating scenario. |  |  |  |  |  |

#### 5.3.5.4	Solution #3.4: The Additional Information in RSU as the input for CHF rating

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

The possible solutions support the potential requirements REQ-3GPPCH-ER-01 and REQ-3GPPCH-ER-02 and Key Issue #3a, #3b and #3c to describe the enhancement of input to CHF rating.

The QoS information is measurement of the overall performance of a service. The service identifier is the detailed identification of a service data flow. A solution could be to allow the QoS information and service id in the Additional information as well as the rating group in the request for quota specified in the TS 32.291 [6], as the input for CHF rating consideration, to be able to assist to allocate the right amount of quota needed at that moment for that rating group.

Table 5.3.5.4-1: Definition of type RequestedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| time | Uint32 | OC | 0..1 | This field holds the amount of requested time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of requested service specific units. |  |
| additionalInfo | AdditionalInfo | OC | 0..N | This field holds the additional information for the input of CHF rating consideration. | CHFRatingInput |
| NOTE: if none of them is included, "RequestedUnit": {}, the category and amount is determined by CHF for online charging with centralized unit determination and rating scenario. |  |  |  |  |  |

Table 5.3.5.4-2: Definition of type AdditionalInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qoSInformation | QosData | OC | 0..1 | This field describes the QoS applied for the requested used unit. |  |
| serviceId | ServiceId | OC | 0..N | This field identity of the used service linked to the rating group. |  |

### 5.3.6	Evaluation

Solution #3.1, #3.2, #3.3, #3.4, and #3.5 all solve key issues #3a, #3b, and #3c.

-	Solution #3.1: the new service id list holds the used service ids which will make it easier for the CHF to allocate quota for the rating group. It may cause confusion as to if this is the service id for the requested units or the reported used units.

-	Solution #3.2: the new service id list holds the used service ids and is part of the requested units which will make it easier for the CHF to allocate quota for the rating group.

- Solution #3.3: the new QoS information holds the currently applied QoS for the rating group and is part of the requested units which will make it easier for the CHF to allocate quota for the rating group.

- Solution #3.4: the new additional information holds both the QoS information and service ids the currently applied for the rating group and is part of the requested units which will make it easier for the CHF to allocate quota for the rating group.

Editor’s note: Further evaluation is FFS.

### 5.3.7	Conclusions

Editor’s note: Conclusions are FFS.

## 5.4	Chargeable events and sessions cancelling

### 5.4.1	General

In the present document for the Nchf service there is a possibility charge before the service has been delivered i.e., IEC. This is similar to the DCCA IEC, however in Diameter there is a possibility to do a refund, something similar is missing in Nchf.

### 5.4.2	Use cases

#### 5.4.2.1	Use Case # 4a: Cancel failed event charging

The operator would like to use IEC instead of PEC to catch all service delivery events, even the ones that might be lost or not delivered. When using IEC, the charging request is sent before the service has been deliver, when the service delivery fails, the operator would like to have the possibility to cancel (e.g., by doing a refund) the previous charging request.

### 5.4.3	Potential charging requirements

REQ-3GPPCH-ESC-01	The 5G system should support to cancelling (e.g., ref und) chargeable service delivery events.

### 5.4.4	Key issues

The following key issues are identified:

-	Key Issue #4a: How to locate the event that should be cancelled (e.g., refund).

### 5.4.5	Solutions

#### 5.4.5.1	Solutions #4.1: Cancelling using charging session identifier

A possible solution for key issues 4a, covering requirements REQ-3GPPCH-ESC-01, cancelling of event would be to use the charging session identifier, see TS 32.290 [12], which corresponded to ChargingDataRef, TS 32.291 [6]. When a resource is created in the CHF it will always get a ChargingDataRef. This ChargingDataRef can be put in a new attribute and used in a new service operation (e.g., cancel) to cancel any previously created resources. The NF (CTF) provides the information for cancelling of event(s) in new service operation request, which would be handled as a new EventType. After received the new service operation request, the CHF performs the cancel operation. Otherwise, the CHF responses results to indicate the cancel unsuccessfully.

Figure 5.4.5.1-1 Message flow of new service operation (e.g., cancel)

Editor’s Note: The use of ChargingDataRef is ffs.

#### 5.4.5.2	Solutions #4.2: Cancelling using a reference identifier for Charging Event

A possible solution for key issues 4a, covering requirements REQ-3GPPCH-ESC-01, cancelling of event would be to add a reference identifier (e.g. refundInformation) in Charging Data Response of Nchf_ConvergedCharging_Create operation -> multipleUnitInformation for IEC, which can later be sent back in Charging Data Request of Nchf_ConvergedCharging_Release operation -> multipleUnitUsage, to identify the event to be cancelled or refunded.

Figure 5.4.5.2-1 Message flow of the Refund for IEC

### 5.4.6	Evaluation

Solutions #4.1 and #4.2 both solve key issue #4a.

-	Solution #4.1: requires an extension to current event with a specific type and uses the current chargingDataRef in a new attribute.

-	Solution #4.2: reuses the current event with specific information to determine refund, making the refund implicitly defined.

Editor’s note: Further evaluation is FFS.

### 5.4.7	Conclusions

Editor’s note: Conclusions are FFS.

## 5.5	Non-blocking mechanism enhancement

### 5.5.1	General

The non-blocking mode scenario is introduced for the 5G charging in clause 5.3.2 Charging scenarios of TS 32.290 [12]. The Converged Charging service offers charging with quota management, including the support for both blocking mode and non-blocking mode. The non-blocking charging mode allow the service delivery before sending the request to the CHF for the service to be granted authorization to start. In TS 23.503 [3], the "Service Data flow handling while requesting credit" in clause 6.3 Policy and charging control rule is introduced for indicating whether the service data flow is allowed to start while the SMF is waiting for the response to the credit request (Values: blocking or non-blocking).

In the TS 23.501 [9], when the PCC Rule attribute Service Data flow handling while requesting credit indicates "non-blocking", the SMF requests the report of the relevant usage information for the Charging key and Sponsor Identity (if applicable) and provide a default threshold value to the UPF while waiting for the quota from the CHF.

### 5.5.2	Use cases

#### 5.5.2.1	Use Case # 5a: CHF failed to grant quota

When end user accesses the services and the non-blocking mode is active based on the PCC rule, the quota is not granted from the CHF before the service delivery. The CHF may not grant quota for the service delivery after service delivery have started in e.g., the following cases:

- the end-user service restrictions or limitations;

- the end user's account could not cover the requested service delivery;

- CHF denies the service request for other reasons; etc.

The service delivery will be started and then stopped because of the service start is beyond the control of the CHF.

To avoid the UE frequently initiates service requests and start of service delivery affecting the user's experience, in this case, the non-blocking mode could be disabled temporarily based on the information of CHF and resumed later.

### 5.5.3	Potential charging requirements

REQ-3GPPCH-NB-01	The 5G system should support to disable temporarily the non-blocking mode.

REQ-3GPPCH-NB-02	The 5G system should support to resume the non-blocking mode.

### 5.5.4	The key issues

The following key issues are identified:

-	Key Issue #5a: Non-blocking mode disable/enable affect only specific rating group or all rating groups for a UE.

-	Key Issue #5b: Identify the Network Functions to disable/enable non-blocking mode.

-	Key Issue #5c: Determine of the interactions required to disable/enable non-blocking mode for the special user/service.

-	Key Issue #5d: Determine if non-blocking mode is possible for a rating group.

### 5.5.5	Solutions

#### 5.5.5.1	Solution #5.1: Dedicated non-blocking indicator at service start

The possible solution partially supports the potential requirements REQ-3GPPCH-NB-01, REQ-3GPPCH-NB-02 and Key Issue #5d for the non-blocking mode reporting to CHF.

Figure 5.5.5.1-1: Message flow for dedicated non-blocking indicator

1. PCF sends the PCC rules to SMF with the "Service Data flow handling while requesting credit" to indicate SMF the Non-Blocking mode is adapted.

2. The service data flow are allowed to start while the SMF is waiting for the response to the quota request.

3. SMF sends the Charging Data Request to CHF for the quota request with the non-blocking mode indication when detect the non-blocking mode is usage for the service data flow.

The charging reporting from the SMF with the Non-blocking mode is per Rating Group. After the PDU session establishment, when any service date flow delivery with the non-blocking mode in the RG is detected (may be the first service data flow of the PDU session or the subsequent service data flows of the PDU session), the SMF should immediately report to the CHF with the non-blocking mode indication in the Charging Data Request [Initial] for first service data flow or in the Charging Data Request [Update] for the subsequent service data flow when detected with the non-blocking mode.

#### 5.5.5.2	Solution #5.2: The non-blocking mode change from CHF to SMF

The possible solution supports the potential requirements REQ-3GPPCH-NB-01, REQ-3GPPCH-NB-02 and Key Issue #5a, Key Issue #5b, Key Issue #5c to describe the non-blocking mode change from CHF enhances or extends the Nchf_Convergedcharging defined in TS 32.255 [3] and SMF operation.

Figure 5.5.5.2-1: Message flow for Non-blocking Change on SMF

1-3.	The same steps description with the Figure 5.5.5.1-1.

4.	When received the Charging Data Request and Non-Blocking mode, CHF determines to change the Non-Blocking mode based on the user information (e.g. Account Balance and account status) in the charging system. CHF sends the Charging Data Response with the Non-Blocking mode change.

The CHF can change the Non-blocking mode when received the Charging Data Request with the NB mode indication or change it in the any Charging Data Response spontaneously.

The CHF can change the NB mode per UE, per RG or per Service Id.

5.	The CHF sends the non-blocking mode in the Charging Data Response to SMF. SMF has the PCC rule about the "service data flow handling while requesting credit" previously and the non-blocking mode change from CHF subsequently, For the subsequent service data flows with the same RG in the PDU session, the SMF will perform the setting from CHF with blocking mode.

The CHF can change the NB mode per UE, per RG, per Service Identifier.

The non-blocking mode set by CHF is only active during the PDU session, When the PDU session is terminated, the non-blocking mode setting by the CHF is inactive. For next PDU session establishment, the SMF will perform the PCC rules from PCF.

#### 5.5.5.3	Solution #5.3: The non-blocking mode change from CHF to PCF via SMF

The possible solutions to support the potential requirements REQ-3GPPCH-NB-01, REQ-3GPPCH-NB-02 and Key Issue #5a, Key Issue #5b, Key Issue #5c could be the following enhancements or extensions to the Nchf_Convergedcharging defined in TS 32.255 [3] and the interaction between PCF and SMF.

Figure 5.5.5.3-1: Message flow for Non-blocking Change on PCF via SMF

1-4.	 The same steps description with the Figure 5.5.5.3-1.

5.	The CHF sends the non-blocking mode in the charging data response to SMF

6.	The SMF transfers the non-blocking mode change to the PCF in the PDU session. The PCF will set the new PCC rules for the "service data flow handling while requesting credit" to SMF.

The CHF can change the NB mode per UE, per RG, per Service Identifier.

When the PDU session is terminated, the PCF still adapts the updated non-blocking mode considering the feedback from CHF.

#### 5.5.5.4	Solution #5.4: The non-blocking mode change per UE from CHF to PCF

The possible solutions to support the potential requirements REQ-3GPPCH-NB-01, REQ-3GPPCH-NB-02 and Key Issue #5a, Key Issue #5b, Key Issue #5c about change the non-blocking mode per UE could be the following enhancements or extensions to the Nchf_SpendingLimitControldefined in TS 23.503 [3].

Figure 5.5.5.4-1: Message flow for Non-blocking Mode Change on PCF from CHF

0.	The CHF subscribes the policy counter status information relating to subscriber spending limits.

1-4.	The same steps description with the Figure 5.5.5.1-1.

5.	The CHF sent the non-blocking mode change in the spending limit control service to PCF via the Policycounterstatus. According to the TS 23.503 [3] and TS 29.594 [11], The policy counter staus is per UE. the policy counter status is a label whose values are not standardized and that is associated with a policy counter's value relative to the spending limit(s) (the number of possible policy counter status values for a policy counter is one greater than the number of thresholds associated with that policy counter, i.e. policy counter status values describe the status around the thresholds). This is used to convey information relating to subscriber spending from CHF to PCF. Specific labels are configured jointly in CHF and PCF.

The specific labels is the non-blocking mode. Configure the special policy counter per UE for the Non-blocking mode, if CHF determines to change the Non-blocking mode (i.e. Enable/disable) for any service data flow, the CHF notifies the PCF to change the policy counter status for UE.

The CHF can change the NB mode per UE.

When the PDU session is terminated, the PCF still adapts the updated non-blocking mode considering the feedback from CHF.

#### 5.5.5.5	Solution #5.5: Dedicated non-blocking indicator before service start

The possible solution partially supports the potential requirements REQ-3GPPCH-NB-01, REQ-3GPPCH-NB-02 and Key Issue #5d for the non-blocking mode reporting to CHF.

Figure 5.5.5.5-1: Message flow for dedicated non-blocking indicator

1.	PCF sends the PCC rules to SMF with the sdfHandl set to true, indicating that the service data flow is allowed to start while the SMF is waiting for the response to the credit request i.e., non-blocking mode.

2.	SMF sends the Charging Data Request to CHF for the quota request with a non-blocking indication for the rating group with the sdfHandl set to true.

3.	The service data flow is allowed to start while the SMF is waiting for the response to the quota request.

The non-blocking charging reporting from the SMF is per rating group. After the PDU session establishment, when any service date flow delivery for a rating group with non-blocking (can be the first or the subsequent service data flows of the PDU session), the SMF should immediately report to the CHF with the non-blocking indication in the Charging Data Request [Initial] if it is the first service data flow or in the Charging Data Request [Update] for the subsequent service data flow.

#### 5.5.5.6	Solution #5.6: The Non-blocking mode change using policy counters from CHF to PCF

A possible solution to support the potential requirements REQ-3GPPCH-NB-01, REQ-3GPPCH-NB-02 and Key Issue #5a, Key Issue #5b, Key Issue #5c reuses the policy and charging control framework defined in TS 23.503 [3], TS 29.513 [15], TS 29.594 [16].

The PCF is configured with PCC Rules that include information for service data flow detection and charging information. The charging information includes Charging Key, Charging method, and Service Data flow handling while requesting credit (see TS 23.503 [3] table 6.3.1).

The CHF is configured with Rating Groups (corresponding to Charging Key) and policy counter information which includes Policy counter identifiers and statuses.

The PCF will use the Nchf_SpendingLimitControl_Subscribe for a specific SUPI to retrieve the initial status of the policy counters. Based on these statues, and their configured relationship with the charging keys if any, it may select PCC Rules or even update PCC Rules. It would then provide these to the SMF if requested.

The CHF will, based on Nchf_ ConvergedCharging requests and account balance changes, check if the status of the policy counters should be updated, if the statuses is changed it will use the Nchf_SpendingLimitControl_Notify to send the new status to the PCF, if the PCF have a subscription for the counters.

One policy counter identifier may be connected to one or more Rating Groups in the CHF, this connection is preferably mirrored in the PCF i.e., the policy counter identifier is relevant for one or more Charging Keys. The Charging Keys/Rating Groups, Policy Counter Identifiers, and Policy Counter Statuses need to be synchronized between the PCF and CHF. Since if any of these have different meanings in the PCF and CHF it will lead to issues e.g., if a Charging Key is defined in PCF but there is no rating for the Rating Group this will lead to faulty charging the same is applicable to the Policy counter identifiers and statuses which may lead to faulty PCC Rules being applied.

This means that the PCF can based on the policy counter status set the Service Data flow handling while requesting credit to either blocking or non-blocking in the PCC Rule applicable to a specific Charging Key. The CHF can have policy counter applicable to one or more Rating Groups, and the status can be based on the account balance status e.g., the account balance is near to the limit for a Rating Group.

### 5.5.6	Evaluation

Solution #5.1, #5.5 and #5.6 all solve key issue #5d.

Solution #5.1:

-	keep alignment with the Figure 5.3.2.3.2: SCUR - Session based charging with Decentralized and Centralized Unit Determination, Centralized Rating, immediate start of service delivery (non-blocking mode) specified in TS 32.290 [12], before sent the charging data request, the service delivery starts.

- new indication should be added in the charging data request.

Solution #5.5:

-	The start of the service delivery does not need to wait the Charging Data Response from CHF. keep alignment with the common service flow about non-blocking specified in the TS 32.290 [12]. It is the same solution with solution #5.1.

-	new indication should be added in the charging data request.

Solution #5.6:

-	reuses the current mechanism and by that the start of the service delivery does not need to wait the Charging Data Response from CHF. keep alignment with the common service flow about non-blocking specified in the TS 32.290 [12].

-	no impact on the charging data request.

Editor’s note: How the non-blocking mode is taken into account needs to be elaborated in the evaluations.

Editor’s note: Further evaluation is FFS.

### 5.5.7	Conclusions

.

# 6	Documentation improvements

## 6.1	General

The charging management documentation is today organized according to TS 32.240 [5] with a set of domain/subsystem/service specific TSs covers the domains (CS, PS, 5GS), subsystem (IMS) and service (SMS, MMTel etc.) levels, respectively, in the TS 32.25x, TS 32.26x and TS 32.27x TS number ranges. Network Slicing is covered under TS 28-series. These TSs describe the mapping of the common architecture specified onto the specific domain/subsystem/service as well as the scenarios and information for converged charging that are specific to the domain/subsystem/service. They are commonly referred to as the middle tier charging TSs.

A set of TSs in the TS 32.28x range covers common services, such as the Announcement service.

A set of TSs in the TS 32.29x range covers common aspects, such as CDR parameter and syntax descriptions, converged charging applications, and the charging interactions between the network and the Billing Domain (CDR file transfer).

The complete document structure for these TSs is outlined in the following figure 6.1-1:

Figure 6.1-1: Charging specifications structure

This means that all information elements are first specified in the middle tier charging TSs and then converted into the OpenAPI in TS 32.291 [6], ASN.1 in TS 32.298 [7] and Diameter in TS 32.299 [8]. In some case there are mapping between the middle tier charging TSs information element and one or more of the protocols.

## 6.2 Use cases

### 6.2.1	Use Case # 6a: The binding description

As per the description of clause 7 Bindings of CDR field, Information Element and Resource Attribute of TS 32.291[1], the mapping between the Service Charging Information element, Resource Attribute and CDR fields for 5G charging are present.

The table 7.1-1 in the clause 7.1 describes common CDR field, Information Element and Resource Attribute

The table 7.x-1 in the clause 7.x (From 7.2 to 7.10 based on the version 17.4.0 of TS 32.291[1]) describe the special CDR field, Information Element and Resource Attribute for each service.

The Information Elements are also specified in the 5G service charging specifications, such as TS 32.255[3]

The CDF field are also specified in the 5G serivce charging specifications and TS 32.298 [2].

The attribute are specified in the TS 32.291[1] yaml.

But the link between 5G service charging specifications, CDRs and Yaml based on the binding is unclear.

The following key issues are identified:

-	Key Issue #6a: optimize the binding for the CDR fields, Information Elements and Resource Attributes.

### 6.2.2	Use Case # 6b: The simplification of CDR records

The TS 32.298 provide the charging data record (CDR) parameters in stage3 and ASN.1 for GPRS charging, EPC charging and 5G charging, including the generic CDR parameters, bearer level, subsystem level and common charging data in CHF- CDR.

The TS 32.298 should be considered in the structure Optimization.

The following key issues are identified:

-	Key Issue #6a: optimize the CDR and ASN.1 for 5G Charging.

### 6.2.3	Use Case # 6c: Common IEs applied in the service charging specifications

As per the description of clause 7 Message contents of TS 32.290 [57], the common information structure used for services operation are specified. The services charging specifications (e.g. data connectivity in TS 32.255[x]), refer to the common information fields and provide the special services related Information Element (IE). How to refer to the common IEs uniformly in the service charging specifications should be considered.

The following key issues are identified:

-	Key Issue #6c: How to construct the common IEs applied in the service charging specifications

## 6.2	Key issues

The following key issues are identified:

-	Key Issue #6a: Separation of charging architecture and principle for 5G i.e., replacing TS 32.240 [5] for 5GC.

-	Key Issue #6b: Organization of the Nchf (OpenAPI) parts specific for the different domains, subsystems, and services.

-	Key Issue #6c: Organization of the CDR (ASN.1) parts specific for the different domains, subsystems, and services.

## 6.3	Solutions

### 6.3.1	Solution #6.1: Binding in stage2 and stage3 documents

According to the 5G service charging specifications (e.g. 5G data connectivity, SMS charging), CHF CDRs described in the 5G service charging specifications (e.g. TS 32.255, TS 32.254, TS 32.256) and in the CDR records (TS 32.298 ASN.1), the attributes in the TS 32.291 Yaml.

5G service charging specifications provide the binding between Information Elements and CHF CDRs, TS 32.291 provides the binding between Information Elements and Resource Attributes, TS 32.298 provides the binding fields of CHF CDR and ASN.1.

### 6.3.2	Solution #6.2: Binding in TS 32.298

TS 32.298 provides the binding fields of CHF CDR and ASN.1.

### 6.3.3	Solution #6.3: Binding in TS 32.291 and TS 32.298

Delete the binding table of "CDR Field" in the TS 32.291, the binding of Information Elements, CHF CDRs and ASN.1 are present in the TS 32.298.

### 6.3.4	Solution #6.4: Separate the CDR records

From R18, Separate the CHF-CDR related content for the 5G charging from TS 32.298, new TS such as TS 32.298-01 or new TS numbering TS 32.XXX about 5G Charging of CHF CDR parameters.

### 6.3.5	Solution #6.5: Complete the Common IE applied

A possible solution for key issue #6c.

In the service charging specifications, it copies all the common IEs specified in TS 32.290[57] and describes whether to use (Described in TS 32.290 [57]) or not use (This field is not applicable.) in the service charging.

Table 6.3.5-1: Message content structure

| Information Element | Converged Charging Category | Offline Only Charging Category | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | Described in TS 32.290 [57]. |
| Subscriber Identifier | OM | OM | Described in TS 32.290 [57]. |
| NF Consumer Identification | M | M | Described in TS 32.290 [57]. |
| NF Functionality | M | M | Described in TS 32.290 [57]. |
| NF Name | OC | OC | This field is not applicable. |
| NF Address | OC | OC | This field is not applicable. |
| NF PLMN ID | OC | OC | Described in TS 32.290 [57]. |
| Charging Identifier | OM | - | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | M | Described in TS 32.290 [57]. |
| Invocation Sequence Number | M | M | Described in TS 32.290 [57]. |
| Retransmission Indicator | OC | OC | Described in TS 32.290 [57]. |
| One-time Event | OC | - | Described in TS 32.290 [57]. |
| One-time Event Type | OC | - | Described in TS 32.290 [57]. |
| Notify URI | OC | - | Described in TS 32.290 [57]. |
| Supported Features | OC | - | Described in TS 32.290 [57]. |
| Service Specification Information | OC | - | Described in TS 32.290 [57]. |
| Triggers | OC | OC | Described in TS 32.290 [57]. |
| Multiple Unit Usage | - | - | Described in TS 32.290 [57] |
| Rating Group | - | - | Described in TS 32.290 [57] |
| Requested Unit | - | - | This field is not applicable. |
| Time | - | - | This field is not applicable. |
| Total Volume | - | - | This field is not applicable. |
| Uplink Volume | - | - | This field is not applicable. |
| Downlink Volume | - | - | This field is not applicable. |
| Service Specific Units | - | - | This field is not applicable. |
| Used Unit Container | - | - | Described in TS 32.290 [57] |
| Service Identifier | - | - | Described in TS 32.290 [57] |
| Quota management Indicator | - | - | Described in TS 32.290 [57] |
| Triggers | - | - | Described in TS 32.290 [57] |
| Trigger Timestamp | - | - | Described in TS 32.290 [57] |
| Time | - | - | Described in TS 32.290 [57] |
| Total Volume | - | - | Described in TS 32.290 [57] |
| Uplink Volume | - | - | Described in TS 32.290 [57] |
| Downlink Volume | - | - | Described in TS 32.290 [57] |
| Service Specific Unit | - | - | Described in TS 32.290 [57] |
| Event Time Stamps | - | - | Described in TS 32.290 [57] |
| Local Sequence Number | - | - | Described in TS 32.290 [57] |

For example, expands all the sub-IEs in the second Layer and third layer which are used for the service charging.

Add the general principle in to the TS 32.290 [57] clause 7.

### 6.3.6	Solution #6.6: Partial the Common IE applied

A possible solution for key issue #6c.

In the service charging specifications, only expand the required layers of common IEs and describe whether to use (Described in TS 32.290 [57]) or not use (This field is not applicable.) in the service charging.

Table 6.3.6 -1: Message content structure

| Information Element | Converged Charging Category | Offline Only Charging Category | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | Described in TS 32.290 [57]. |
| Subscriber Identifier | OM | OM | Described in TS 32.290 [57]. |
| NF Consumer Identification | M | M | Described in TS 32.290 [57]. |
| NF Functionality | M | M | Described in TS 32.290 [57]. |
| NF Name | OC | OC | This field is not applicable. |
| NF Address | OC | OC | This field is not applicable. |
| NF PLMN ID | OC | OC | Described in TS 32.290 [57]. |
| Charging Identifier | OM | - | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | M | Described in TS 32.290 [57]. |
| Invocation Sequence Number | M | M | Described in TS 32.290 [57]. |
| Retransmission Indicator | OC | OC | Described in TS 32.290 [57]. |
| One-time Event | OC | - | Described in TS 32.290 [57]. |
| One-time Event Type | OC | - | Described in TS 32.290 [57]. |
| Notify URI | OC | - | Described in TS 32.290 [57]. |
| Supported Features | OC | - | Described in TS 32.290 [57]. |
| Service Specification Information | OC | - | Described in TS 32.290 [57]. |
| Triggers | OC | OC | Described in TS 32.290 [57]. |
| Multiple Unit Usage | - | - | Described in TS 32.290 [57] |
| Rating Group | - | - | Described in TS 32.290 [57] |
| Requested Unit | - | - | This field is not applicable. |
| Used Unit Container | - | - | Described in TS 32.290 [57] |
| XXX | - | - | This field indicated the special service. |

If all the sub-IEs compared to the upper level are the same (i.e. This field is not applicable or Described in TS 32.290 [57]) for the service charging, only show and refer to the upper level

- Take the "Requested Unit" as the example, the sub-IEs (i.e. time, totalVolume, uplinkVolume, downlinkVolume and serviceSpecificUnits) are not applicable. In the table, only "Requested Unit" is described "This field is not applicable", without expand and show the sub-IEs, please see Table 6.1.Y.Y -1: Message content structure.

If partial sub-IEs compared to the upper level is different (even, only one sub-IEs), expand the upper layer, show and refer to the all sub-IEs and upper level.

- Take the "Requested Unit" as the example, the sub-IEs (i.e. totalVolume, uplinkVolume, downlinkVolume and serviceSpecificUnits) are not applicable and the sub-IEs (i.e. time) refer to the TS 32.290. In the table, the"Requested Unit" includes all the sub-IEs should be described.

Table 6.3.6-2: Example of Requested Unit

| Requested Unit | OC | OC | Described in TS 32.290 [57]. |
| --- | --- | --- | --- |
| Time | OC | OC | Described in TS 32.290 [57]. |
| Total Volume | - | - | This field is not applicable. |
| Uplink Volume | - | - | This field is not applicable. |
| Downlink Volume | - | - | This field is not applicable. |
| Service Specific Units | - | - | This field is not applicable. |

If special sub-IEs for the service charging should be added in the upper layer and partial sub-IEs compared to the upper level is different, refer to the upper layer with the need to expand the upper layer.

- Take the "Requested Unit" as the example, the new sub-IEs (i.e. New xxx) are added and the all the sub-IEs are showed in the table to present its own description.

Table 6.3.6-4: Example of Requested Unit for addition

| Requested Unit | OC | OC | Described in TS 32.290 [57]. |
| --- | --- | --- | --- |
| Time | OC | OC | Described in TS 32.290 [57]. |
| Total Volume | - | - | This field is not applicable. |
| Uplink Volume | - | - | This field is not applicable. |
| Downlink Volume | - | - | This field is not applicable. |
| Service Specific Units | - | - | This field is not applicable. |
| New xxx | OC | OC | This field is used for the ….. |

# 7	Conclusions and recommendations

To be defined.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-09 | SA5#138e | S5-214430 |  |  |  | Initial skeleton (v0.0.0) of TR 28.826 | 0.0.0 |
| 2021-10 | SA5#139e | S5-215241 |  |  |  | Study document structure | 0.1.0 |
| 2021-10 | SA5#139e | S5-215469 |  |  |  | Addition of introduction and scope | 0.1.0 |
| 2021-10 | SA5#139e | S5-215470 |  |  |  | Addition of overview | 0.1.0 |
| 2021-10 | SA5#139e | S5-215590 |  |  |  | Add the topic for Non-blocking Mode Enhancement | 0.1.0 |
| 2021-10 | SA5#139e | S5-215591 |  |  |  | Add the topic for Enhancement of input to CHF Rating | 0.1.0 |
| 2021-11 | SA5#140e | S5-216523 |  |  |  | Use case trigger without usage | 0.2.0 |
| 2021-11 | SA5#140e | S5-216525 |  |  |  | Use case size of charging information | 0.2.0 |
| 2022-02 | SA5#141e | S5-221120 |  |  |  | Update the References | 0.3.0 |
| 2022-02 | SA5#141e | S5-221682 |  |  |  | Restructuring clauses 5.3 to 5.5 | 0.3.0 |
| 2022-02 | SA5#141e | S5-221726 |  |  |  | Adding use case for operator defined trigger | 0.3.0 |
| 2022-02 | SA5#141e | S5-221736 |  |  |  | Adding use case for cancel failed event | 0.3.0 |
| 2022-04 | SA5#142e | S5-222450 |  |  |  | Adding general clause for documentation | 0.4.0 |
| 2022-04 | SA5#142e | S5-222773 |  |  |  | Add the solution for NB Enhancement | 0.4.0 |
| 2022-05 | SA5#143e | S5-223700 |  |  |  | Adding trigger solutions to clause 5.2 | 0.5.0 |
| 2022-05 | SA5#143e | S5-223716 |  |  |  | ServiceID in Charging Data Request | 0.5.0 |
| 2022-07 | SA5#144e | S5-224138 |  |  |  | Update the Reference | 0.6.0 |
| 2022-07 | SA5#144e | S5-224194 |  |  |  | Correcting clause 5.5.5 | 0.6.0 |
| 2022-07 | SA5#144e | S5‑224464 |  |  |  | Add the QoS solution for CHF Rating | 0.6.0 |
| 2022-07 | SA5#144e | S5‑224465 |  |  |  | Add the General for NB mechanism enhancement | 0.6.0 |
| 2022-07 | SA5#144e | S5‑224466 |  |  |  | Add the Additional Information for CHF Rating | 0.6.0 |
| 2022-07 | SA5#144e | S5-224467 |  |  |  | Adding issue and solution to identifying non-blocking mode | 0.6.0 |
| 2022-07 | SA5#144e | S5-224468 |  |  |  | Adding solution for cancel failed event | 0.6.0 |
| 2022-07 | SA5#144e | S5-224481 |  |  |  | Correcting clause 5.2.5 | 0.6.0 |
| 2022-08 | SA5#145e | S5-225754 |  |  |  | Adding potential solution for locating cancel failed event | 0.7.0 |
| 2022-08 | SA5#145e | S5-225777 |  |  |  | Adding solution for non-blocking mode change using PCF | 0.7.0 |
| 2022-09 | SA#97e | SP-220845 |  |  |  | Edithelp review and presented for information | 1.0.0 |
| 2022-11 | SA5#146 | S5-226136 |  |  |  | Adding new use case for IoT Devices | 1.1.0 |
| 2022-11 | SA5#146 | S5-226433 |  |  |  | Adding key issues for documentation structure | 1.1.0 |
| 2022-11 | SA5#146 | S5-226855 |  |  |  | Undefined attribute handling in CDR | 1.1.0 |
| 2022-11 | SA5#146 | S5-226856 |  |  |  | Adding solution in clause 5.1 using new trigger category | 1.1.0 |
| 2022-11 | SA5#146 | S5-226857 |  |  |  | Adding solution in clause 5.1 not splitting rating group | 1.1.0 |
| 2022-11 | SA5#146 | S5-226858 |  |  |  | Addition of optimization on charging information accuracy | 1.1.0 |
| 2022-11 | SA5#146 | S5-226859 |  |  |  | Addition of Bitrate charging | 1.1.0 |
| 2022-11 | SA5#146 | S5-226860 |  |  |  | Addition of Threshold based re-authorization triggers optimization | 1.1.0 |
| 2022-11 | SA5#146 | S5-226861 |  |  |  | Clarifiy the Cancel Operation | 1.1.0 |
| 2022-11 | SA5#146 | S5-226862 |  |  |  | Clarify the Non-Blocking solution | 1.1.0 |
| 2022-11 | SA5#146 | S5-226863 |  |  |  | Optimization of the Binding | 1.1.0 |
| 2022-11 | SA5#146 | S5-226864 |  |  |  | Simplify the structure of CDR record | 1.1.0 |
| 2023-01 | SA5#146Bis-e | S5-231057 |  |  |  | Add the max size solution | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231250 |  |  |  | Adding solution for size of charging information | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231251 |  |  |  | Correcting solutions in clause 5.1 | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231254 |  |  |  | Common IEs applied in the service charging specifications | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231276 |  |  |  | Adding solution for key issue #1l in clause 5.1 | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231286 |  |  |  | Correction on the key issues in the topic 5.5 | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231288 |  |  |  | Evaluation and Conclusion for Topic 5.5 | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231289 |  |  |  | Evaluation and Conclusion for Topic 5.4 | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231290 |  |  |  | Evaluation and Conclusion for Topic 5.3 | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231291 |  |  |  | Evaluation and Conclusion for threshold | 1.2.0 |
| 2023-01 | SA5#146Bis-e | S5-231292 |  |  |  | Add the solution for keeping reporting based on the triggers | 1.2.0 |
| 2023-03 | SA5#147 | S5-232584 |  |  |  | Add the potential solution for Common IEs applied | 1.3.0 |
| 2023-03 | SA5#147 | S5-232747 |  |  |  | Clarify the solution 1.1 | 1.3.0 |
| 2023-03 | SA5#147 | S5-232749 |  |  |  | Correction on the potential solutions for key issue #1l | 1.3.0 |
| 2023-03 | SA5#147 | S5-232750 |  |  |  | Solving termination action issue | 1.3.0 |
| 2023-03 | SA5#147 | S5-232797 |  |  |  | Add the solution for Bitrate charging | 1.3.0 |
| 2023-03 | SA5#147 | S5-232798 |  |  |  | Adding solutions to undefined attribute handling in CDR | 1.3.0 |
| 2023-03 | SA5#147 | S5-232801 |  |  |  | Adding use case requested units and quota management indication | 1.3.0 |
| 2023-03 | SA5#147 | S5-232802 |  |  |  | Adding solution to clause 5.1 on location accuracy | 1.3.0 |
| 2023-03 | SA5#147 | S5-232803 |  |  |  | Updating solution in clause 5.4 | 1.3.0 |
| 2023-03 | SA5#147 | S5-232804 |  |  |  | Additional evaluation to clause 5.1 | 1.3.0 |
| 2023-03 | SA5#147 | S5-232805 |  |  |  | Adding evaluation to clause 5.2 | 1.3.0 |
| 2023-03 | SA5#147 | S5-232806 |  |  |  | Additional evaluation to clause 5.3 | 1.3.0 |
| 2023-03 | SA5#147 | S5-232816 |  |  |  | Additional evaluation in clause 5.4 | 1.3.0 |
| 2023-03 | SA5#147 | S5-232817 |  |  |  | Add evaluation to clause 5.5 of solution #5.6 | 1.3.0 |
| 2023-03 | SA5#147 | S5-232818 |  |  |  | Evaluation and Conclusion | 1.3.0 |
| 2023-03 | SA5#147 | S5-232819 |  |  |  | Solution for Key issue#1f and #1g | 1.3.0 |
| 2023-03 | SA5#147 | S5-232820 |  |  |  | Add the potential solution for Common IEs applied | 1.3.0 |