3GPP TSG-SA5 Meeting #147 	S5-232742

Athens, Greece, 27th February - 3rd March 2023										(S5-232217)

# Source:	MATRIXX Software

# Title:	pCR TR 32.847 Solve Editor's Note in Key Issue #6

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

This pCR is to Solve Editor's Note in Key Issue #6

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

This pCR is to Solve Editor's Note in Key issue #6

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

### 6.6.5	Solution#6.4  NSACF based Network slice charging – CTF based

#### 6.6.5.1	General description

This solution addresses the Key Issue#6 for REQ-NSCH-01, and REQ-NSCH-11 potential requirements and is based Network slice charging performed based on NSACF (CTF).

#### 6.6.5.2	Architecture description

Figure 6.6.5.2-1: NSACF (CTF) based Network slice charging

#### 6.6.5.3	Flow description

The following figure 6.6.5.3-1 describes the high level charging procedure for UE PDU session Converged Charging influenced by Network slice converged charging based on NSACF (CTF).

Figure 6.6.5.3-1: UE PDU session converged Charging influenced by Network slice converged charging based on NSACF

1. Same steps as Figure 6.1.6.3-1 referred-to for solution #2.5: Tenant CCS (instead of UE CCS) control of NS_quota under use of Nchf_ConvergedCharging service.

2. Same steps as Figure 6.6.2.3-1 between UE, SMF, UE CCS and Tenant CCS with:

9ch-b1.	UE PDU session charging needs NS charging information related to "Nb of PDU sessions" for this S-NSSAI.

9ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE PDU session charging information, UE requested NS charging information related to "Nb of PDU sessions").

9ch-b2b.	Tenant CCS correlates received UE charging information with existing content of  NS charging information for the S-NSSAI related to "Nb of PDU sessions".

The procedure under step1 between the Tenant CCS and NSACF, takes place independent from other steps under step 2 (i.e. between UE, SMF, UE CCS and Tenant CCS); For the correlation in 9ch-b2b, the Tenant CCS uses the NS charging information applicable at the time the Charging service request in 9ch-b2a is received. This correlation process does not directly trigger Tenant CCS and NSACF interaction.

This is similar as for :

- for solutions #6.1, where the procedure between the Tenant CCS and Mns Producer/CEF (steps 1ch and 2ch of figure 6.6.2.3-1)

- for solutions #6.2, where the procedure between the Tenant CCS and NWDAF/CEF (steps 1ch and 2ch of figure 6.6.2.3-1)

- for solutions #6.3, where the procedure between the Tenant CCS and NSACF/CEF (step 1 of Figure 6.6.4.3-1)

takes place independent from other steps (i.e. between UE, SMF, UE CCS and Tenant CCS);

The following figure 6.6.5.3-2 describes the high-level charging procedure for UE Registration Converged Charging influenced by Network slice converged charging based on NSACF(CTF).

Figure 6.6.5.3-2: UE Registration converged Charging influenced by Network slice converged charging based on NSACF

1. Same steps as Figure 6.1.6.3-1: Tenant CCS (instead of UE CCS) control of NS_quota under use of Nchf_ConvergedCharging service.

2. Same steps as Figure 6.6.2.3-2 between UE, AMF, UE CCS and Tenant CCS with:

16ch-b1.	Allowed S-NSSAI(s) in the Allowed NSSAI received for UE registration converged charging, needs NS charging information related to "Nb of Reg UEs".

16ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE registration charging information, UE requested NS charging information related to "Nb of Reg  UEs").

16ch-b2b.	Tenant CCS correlates received UE charging information with existing content of NS charging information for the S-NSSAI related to "Nb of Reg UEs".

The procedure under step1 between the Tenant CCS and NSACF, takes place independent from other steps under step 2 (i.e. between UE, AMF, UE CCS and Tenant CCS); For the correlation in 16ch-b2b, the Tenant CCS uses the NS charging information applicable at the time the Charging service request in 16ch-b2a is received. This correlation process does not directly trigger Tenant CCS and NSACF interaction.

End of change