3GPP TSG-SA5 Meeting #147	S5-232784

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

# Source:	Samsung

# Title:	pCR 28.921 Transparent Intent Reporting Window.docx

# Document for:	Approval

Agenda Item:	6.7.3.2

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

While the report (with the current performance values for corresponding expectation targets) is provided at the end of each observation period, the consumer may also wish to know the target values and fulfilment info for the entire observation period. This will enable consumer to know whether the reported target values and the fulfilment info were consistent throughout the observation period or there was some deviation at some point of time. For example, the intent expectation may be reported FULFILLED at the end of observation period. However, there were time windows (within observation period) at which the intent expectation was NOTFULFILLED. The consumer may wish to know those time windows and the values for the target during that time window. This information can be important for the MnS consumer to understand whether the observation period they specified need an update (e.g shortened) or not.  Moreover, it helps the MnS consumer to understand whether their expectation is fulfilled during the entire observation period which also gives transparency to the MnS consumer to update their observation period and/or expectations targets specified in the intent.

The following figure shows multiple variance of intent target fulfilment status within an observation/reporting period.

The variance A suggests that there are some times the expectation was NOTFULFILLED while it was reported FULFILLED at the end of the observation period. This may indicate significant QoS degradation to MnS consumer during observation period. This may also indicate the execution time required by the MnS producer to fulfil the expectation target. Leveraging these information, the MnS consumer may want to update their observationTime, i.e. shorten it to have a better granularity for their expectation fulfilment. The variance B suggests that there were times that the expectation was in FULFILLED state while it was reported NOTFULFILLED at the end of the observation period. This may indicate that there was a problem (e.g. another intent conflict) and it would be important to know when the problem started in order to find an optimum solution. The variance C suggests that there were fluctuations in expectation state which was NOTFULFILLED at short periods of time windows while it was reported as in FULFILLED state. These fluctuations may indicate that the MnS producer may need a health-check as there may be some resource limitations. These short fluctuations may also be ignored by the MnS consumer; however, this should be at MnS consumer’s discretion.

# 4	Detailed proposal

First modification

## 4.5	Issue#4.5: Intent Report

### 4.5.1	Description

In TS 28.312 [2], the Fulfilment information (including the intentFulfilmentinfo, expectationfulfilmentInfo and targetfulfilmentInfo) are defined for the MnS consumer to monitor the intent fulfilment information. The intent report also can contain the current and optional predicted value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated if needed (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled or not fulfilled with the reason of target confliction). Besides, intent conflict information which send by MnS producer to MnS consumer is another type of intent report information. So, following are the three types of information needs to be monitored by MnS consumer:

- Intent Fulfilment information, which represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent). The detailed information see clause 6.2.1.3.6 in TS 28.312 [2].

- Achieved values for targets, which represent current performance values for corresponding expectation targets.

- Intent conflict information, which represents conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solution recommendation to address the conflicts.

Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others don't want, Different MnS consumer may want to calculate or monitor the performance value in different period).

While the report (with the current performance values for corresponding expectation targets) is provided at the end of each observation period, the consumer may also wish to know whether the fulfilment info was consistent for the entire observation period. For example, the intent expectation may be reported FULFILLED at the end of observation period. However, may be possible that within observation period the intent expectation was NOTFULFILLED. The consumer may wish to know this information. This information can be important for the MnS consumer to understand whether the observation period they specified need an update (e.g shortened) or not.  Moreover, it helps the MnS consumer to understand whether their expectation is fulfilled during the entire observation period which also gives transparency to the MnS consumer to update their observation period and/or expectations targets specified in the intent.

REQ-Intent_Driven_MnS_Report-1: The intent driven MnS should have the capability to enable the MnS consumer to request intent report information.

REQ-Intent_Driven_MnS_Report-2: The intent driven MnS should have the capability to enable the MnS consumer to obtain intent report information with current performance value for corresponding expectation targets.

REQ-Intent_Driven_MnS_Report-3: The intent driven MnS should have the capability to enable the MnS consumer to obtain intent report information with intent fulfilment information.

REQ-Intent_Driven_MnS_Report-4: The intent driven MnS should have the capability to enable the MnS consumer to obtain intent report information with intent conflict information.

REQ-Intent_Driven_MnS_Report_05: The intent driven MnS should have capability enabling MnS consumer to specify the content of the report.

REQ-Intent_Driven_MnS_Report_06: The intent driven MnS should have capability enabling MnS consumer to configure the frequency of the intent reporting.

REQ-Intent_Driven_MnS_Report_07: The intent driven MnS should allow consumers to receive reports, with differing content and intervals.

REQ-Intent_Driven_MnS_Report_08: The intent driven MnS should allow reports to contain current values for specified expectation targets.

REQ-Intent_Driven_MnS_Report_09: The intent driven MnS should have capability enabling MnS consumer to obtain intent report information with current context information for corresponding expectation targets.

REQ-Intent_Driven_MnS_Report_10: The intent driven MnS should allow reports to contain information on whether the fulfilment info was consistent throughout the observation period.

End of Modification