3GPP TSG-SA5 Meeting #147 	S5-232852

# Athens, Greece, 27 February - 3 March 2023 												was S5-231659

# Source:	Samsung, EUTC, EDF, BMWK

# Title:	Rel-18 pCR 28.829 Update 6.4, 6.5, 6.6

# Document for:	Approval

Agenda Item:	6.9.3.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This pCR provides a use case to address editor's notes in clause 6.4 of the FS_NSOEU study.

| clause | EN | Rationale to remove. |
| --- | --- | --- |
| 6.4.3 | Editor's Note: It is FFS how to define latency in the context of triggering an alarm. | This requirement has been removed. |
| 6.4.3 | Editor's Note: It is FFS how to define packet loss in the context of triggering an alarm. | Packet loss will not be defined in context of alarm. It will be defined in context of performance measurement. |
| 6.5.3 | Editor's Note: the granularity and semantics of latency performance metric acquisition are for further study. | Addressed as part of solution in 7.1.2. The granularity of latency will be determined using monitorGranularityPeriod |
| 6.5.3 | Editor's Note: It is FFS how to define packet loss in the context of triggering an alarm. | Packet loss will not be defined in context of alarm. It will be defined in context of performance measurement. |
| 6.5.3 | Editor's Note: It is FFS how the performance metrics are associated with the configuration. | The required performance metrics will be configured as performanceMetrics as part of ThresholdMonitor. |
| 6.5.3 | Editor's Note: It is FFS whether service loss is in scope of 3GPP standardization or if it is in scope of the BSS. | The service loss is not in-scope of 3GPP. The solution in 7.1.2 considers network/cell availability instead. |
| 6.6.3 | Editor's Note: the granularity and semantics of latency performance metric acquisition are for further study. | Addressed as part of solution in 7.1.2. The granularity of latency will be determined using monitorGranularityPeriod |
| 6.6.3 | Editor's Note: It is FFS whether service loss is in scope of 3GPP standardization or if it is in scope of the BSS. | The service loss is not in-scope of 3GPP. The solution in 7.1.2 considers network/cell availability instead. |

# 4	Detailed proposal

The following change is proposed to be made to TR 28.829-050.

First modification

### 6.4.3	Potential Requirements

PR 6.4.3-1. The 5G 3GPP management system shall according to mobile network operator policy expose interfaces to third parties that provide the ability to:

a) enumerate alarms;

b) create alarms;

c) remove alarms;

d) configure alarms.

NOTE: 	The intention of this requirement is to enable DSOs to manage the set of alarms that the MNO will expose. That is, the DSO can create, configure, remove and enumerate the alarms that they request that the MNO expose to the DSO.

PR 6.4.3-2. The 5G 3GPP management system shall according to mobile network operator policy expose interfaces to third parties that provide a mechanism for the mobile network operator to send alarms to the third party.

PR 6.4.3-3. The 5G 3GPP management system shall support the following alarms and associated configuration:

a) 	Maximum throughput threshold crossed  (where the trigger characteristic corresponds to measurement of [an average for the third party's network traffic]);

b) 	Maximum packet loss threshold crossed (where the trigger characteristic corresponds to measurement of [an average for the third party's network traffic]);

c) 	Cell outage (where the trigger characteristic corresponds to measurement of [where communication service is not possible at all on the the third party]).

Next modification

### 6.5.3	Potential Requirements

PR 6.5.3-1. The 3GPP management system shall, according to mobile network operator policy, regulatory requirements and contractual obligations, expose standarized interfaces to authorized third parties that provide the ability to initiate and terminate requests for monitoring including the configuration of the monitoring (e.g. monitoring interval, measurement period granularity, location of interest, etc.)

PR 6.5.3-2. The 3GPP management system shall, according to mobile network operator policy, regulatory requirements and contractual obligations, expose standardized interfaces to authorized third parties that provide a mechanism for the mobile network operator to send reports containing  required performance metrics measurements to the DSO. The measurements in these reports are provided in a form such that it will be possible to ascertain the number of measurements made as well as to calculate the standard deviation of those measurements, in order to aid in the interpretation of the reported measurement.

PR 6.5.3-3. The 3GPP management system shall, support the following performance metrics to monitor information according to the associated configuration:

a) Latency between the UE and the PSA UPF, that is latency introduced by the RAN and the 5G CN, for a specific cell, base station or network slice;

b) Throughput [an average for the DSO's network traffic];

c) Packet loss [an average for the DSO's network traffic];

Next modification

### 6.6.3	Potential Requirements

PR 6.6.3-1. Subject to operator policies, regulatory requirements and contractual obligations, the 3GPP management system may support a means for authorized third parties to provide network performance information to an MNO. The data model of the performance information provided by the DSO to the MNO shall include at least the following elements: {measurement granularity, location of measurement, latency, packet loss, throughput.}

a) Latency measurement by the DSO, end to end [an average for the DSO's network traffic];

NOTE 1:	Latency measurement end to end will include delay arising from a network outside the mobile telecommunication system.

b) Throughput [an average for the DSO's network traffic];

c) Packet loss [an average for the DSO's network traffic];

NOTE 2:		Packet loss can be determined by means of observation of ICMP and TCP control information for traffic, sending probes, etc. This methodology is out of scope of 3GPP.

NOTE 3:	The information elements to include in the incident report will be further clarified during the 'solution definition' stage of this study.

NOTE 4:	The measurements provided by the DSO can be acquired by means that are outside the scope of 3GPP. Measurements may be acquired from management MIBs of routers that are operated by the DSO.

End of modification