3GPP TSG-SA5 Meeting #147	S5-232862

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

# Source:	Samsung

# Title:	pCR 28.903 NBI related Key Issues.docx

# Document for:	Approval

Agenda Item:	6.9.7.2

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides NBI relayed Key Issues and requirements. The contribution is based on the endorsed document S5-226814

# 4	Detailed proposal

First modification

## 6.a	Key Issue# 1: Virtual Infrastructure Resource Requirements

As described in clause 5.1.1.2.2 GSMA OPG [2], the OP shall support the Infrastructure resource requirements from the Application providers. This includes CPU, memory, storage, hypervisor and networking definition used by the a[application. In relation to this, an ASP can provide required infrastructure resources for an EAS while requesting to deploy the same.

6.a.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability allowing EAS to declare its requirements on the virtual infrastructure.

## 6.b	Key Issue# x: Querying Available Edge Services

- As described in Clause 5.1.1.3 of GSMA OPG[2], OP shall provide the following capabilities to the Application Provider:An OP shall offer a range of quality policies so that an Application Provider can choose the performance that their application requires. These policies are defined based on objectively measured end-to-end parameters that include performance aspects of both the network and the Cloudlet, such as latency and packet loss (measured as average statistics).

- The NBI shall enable a request-response mechanism through which the Application Provider can state a geographical point where a typical user could be and get informed of the mean latency performance expected.

- The OP shall describe the capabilities of the Edge Cloud including the geographical zones where it is provided and the type and “granularity” of edge cloud and network service (typically generic Compute, memory, storage, and specialised compute, such as GPU and future resource types).

The above requirements is calling for a mechanism using which the available edge service and their characteristics (e.g available SLS, infrastructure resources) are described. This information could be anchored at a particular location i.e edge service and their characteristics available at a particular location.

  - SLA

6.b.1 	Requirements

REQ-FUN-PLOC-1: ECSP management system shall have a capability allowing ASP to query the available edge services locations and their capabilities including available SLA and infrastructure resources.

## 6.c	Key Issue# 1: SLA Requirements Key Issue

As described in clause 5.1.1.2.2 GSMA OPG [2], the OP shall support the SLA requirements from the Application providers. A SLA description allows an Application Provider to describe the physical constraints in an edge network that should be met for the application to run successfully and provide a correct Quality of Experience (QoE) for the end-user at the UE. The SLA requirements include the following properties:

- Bandwidth, bidirectional data rate between UE and backend application.

- Latency, the round trip delay between UE and backend application.

In relation to this, the ASP can declare such requirements for an EAS. The EAS need to be deployed at an EDN which can satisfy these requirements.

6.c.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability allowing EAS to declare its SLA requirements, in terms of bandwidth, latency and jitter, for the data connection with the UE.

REQ-FUN-NBI-1: ECSP management system shall be able to select an EDN based on the SLA requirements of an EAS.

## 6.d	Key Issue# 1: EAS relocation policies Key Issue

As described in clause 5.2.2.3 of GSMA OPG[2] , because of the OP’s measurements or knowledge, or hints from the application about performance degradations, the OP may decide that a different edge compute resource can better host the Edge Application. The EAS relocation trigger from OP are related with lifecycle management of its edge compute resources (for example, the overload of an edge compute resource, a failure or planned maintenance, a new or expanded edge compute resource, an issue with the network for its edge compute resource). The OP should be cognisant of the policy indication from the Application Provider about its sensitivity to a change of the edge compute resource hosting the Edge Application.

As described in clause 5.1.1.2.2 GSMA OPG [2], the NBI allows an Application Provider to specify their support for a stateful or stateless Edge Application, i.e. whether the Edge Application can be moved from one edge compute resource to another and this with or without prior notification.

The Application Provider shall be able to indicate the following policies:

- Its Edge Application cannot be moved from one edge compute resource to another;

- Its Edge Application can be moved from one edge compute resource to another, without any notification;

- Its Edge Application can be moved from one edge compute resource to another with prior notification.

TS 28.538 does not support this yet. This requires an EAS to be moved from one (source) EDN to another (target) EDN. This will involve terminating EAS from the source EDN and then instantiating EAS on the target EDN. This involves the following issues to be addressed

- How the ASP will provide the policies

- How the existing application context is retained from source to target EAS.

6.d.1 	Requirements

REQ-FUN-MOB-1: ECSP management system shall have a capability allowing EAS to declare its mobility policies indicating whether EAS can be moved from one EDN to another

REQ-FUN-MOB-2: ECSP management system shall have a capability allowing EAS to declare its mobility policies indicating whether EAS can be moved from one EDN to another, without any notification.

## 6.e	Key Issue# x: Application Resource Catalogue

- As described in Clause 3.5.1.3 of GSMA OPG[2], OP shall provide ASP with application resource catalogue with all the available application and there characteristics. The following requirements relates to application resource catalogue in GSMA.The NBI shall allow applications providers to access the resource catalogue. The Resource catalogue shall consider local resources (CPU, memory, storage).

- The OP shall offer Application Providers a registry to store their application images and update or delete them. The registry may be centralised or distributed, depending upon the Application Provider’s needs to reduce boot time and recovery

TS 28.538 does not address the requirements related with application resource catalogue. An ASP can first query the catalogue to know what all applications are already available and if there is one which can satisfy ASPs requirements.

6.e.1 	Requirements

REQ-FUN-PLOC-1: ECSP management system should enable ASP to access the application (EAS) resource catalogue containing the available applications and there characteristics.