# 3GPP TSG-SA5 Meeting #147	S5-232649

# Athens, Greece, 27th Feb 2023 to 3rd Mar 2023

# Source:	Samsung, EUTC, EDF, BMWK

# Title:	Rel-18 pCR 28.829 – Potential Solution for Energy utility and telecommunication coordinated rapid recovery of energy service

# Document for:	Approval

Agenda Item:	6.9.3.10

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1]	3GPP TR 22.867: "Study on 5G smart energy and infrastructure

[2]            3GPP TR 28.829: "Study on Network and Service Operations for Energy Utilities"

[3]            S5-216428 : New SID on Network and Service Operations for Energy Utilities

# 3	Rationale

This document proposes the potential solution for the Key Issue of Energy utility and telecommunication coordinated recovery of energy service outage as mentioned in clause 6.8 and 6.X of TR 28.829 [2].

# 4	Detailed proposal

Start of Changes

# 7	Key Issues and potential solutions

### 7.3.2      Potential Solutions

Editor's Note:	This Solution will be added below the new Key Issue #i "Energy utility and communication coordinated recovery of energy service" in S5-23zzzz. This is a solution to that key issue. This corresponds to objective #3 in the FS_NSOEU study.

#### 7.3.2.1	Potential Solution # j: Energy utility and telecommunication coordinated rapid recovery of energy service

###### 7.3.2.1.1	Introduction

The solution addresses key issues of Energy utility and telecommunication coordinated rapid recovery of energy service. It involves mutual exchange of information between a third party energy utility (DSO) and MNO using mechanisms exposed by 3GPP network management system. The mechanisms are for rapid recovery of utility’s energy service outage and subsequent fallback by MNO from its UPS to utility’s energy supply for MNO communications service/network operations. The solution also involves mechanisms to protect MNO sites from complete breakdown in case of UPS running out of its capacity due to prolonged DSO energy service outage.

The DSO informs about its energy service outages like time of outage, locations, expected recovery time etc. to MNO. MNOs Rapid Intervention (MRI) is the approach by which DSO seeks dedicated communication service support from MNO using its UPS backup for network operations. DSO needs this to use its automated smart energy services

for the rapid recovery of its energy distribution services.

MNO has information on the state of its power backup e.g. UPS backup duration at different base station locations, base station IDs that are enduring DSO’s power outage and are depending on UPS backup, base stations that provide communications services to DSOs substations for smart energy services implementation etc.

The DSO can obtain these UPS related information from the MNO and can avoid shutting down of energy service for base station IDs that their UEs camp on or for the critical MNO infrastructure that have insufficient UPS backup capacity to survive the planned outage. The DSO can potentially create the strategic energy service interruptions as long as there is sufficient UPS backup capacity remaining for the duration of the outage at the MNO site.

##### 7.3.2.1.2	Description

The procedure below includes the interactions required between the DSO MnS and MNO MnS in order to satisfy the operational requirements in key issue 7.3.1.2 and 7.3.1.3. The procedure is general in that it can be extended to other attributes easily, and it is based on existing mechanisms specified by SA5.

1. createMOI () Request

DSO

(MnS Consumer)

MNO

(MnS Producer)

3. createMOI Response

2. IoC will be created

5. createMOI(IntfSubscriptionCtrl) Request

4. IOC containing

UPS info instantiated

6. IoC will be created

7. CreateMOI Response

8. notifyMOIAttributeValueChanges

9. modifyMOIAttributes Request

10. modifyMOIAttributesResponse

11. getMOIAttribute Request

12. getMOIAttribute Response

Initialization

MNO informs DSO of the changes

DSO updates the information

DSO retrieves the changes

Figure 7.3.2.1.2-1: Coordinated Rapid Recovery and strategic outage plan Procedure

NOTE 1: 	Steps 1-7 in the procedure flow can be considered 'initial set up' to enable operations to address the functional requirements in 6.8 and 6.9 and described in Key Issues 7.3.1.2 and 7.3.1.3.

1.	In order to be able to provide energy service outage and recovery related information for MNO, DSO sends createMOI request for creating an IOC which contains information attributes for outage and rapid recovery by DSO such as:

a)	Time stamp of DSO’s energy distribution service outage

b)	Locations (latitude-longitude pair and Energy Supply Id)  where DSO’s energy service outage occurs

c)	Time by when DSO expects restoration of its distribution services for MNO.

d)	Time when DSO has restored its energy transmission service and starts expecting rapid intervention by MNO.

e)	Time duration for which DSO expects to require MNO’s rapid intervention for being able to use smart energy services to restore its energy distribution services.

f)	Information of locations where for example DSO substations need to restore distribution services on priority.

g)	The time at which MNO will actually be able to provide rapid intervention to DSO.

h)	The time duration for which MNO will actually be able to provide rapid intervention to DS

i)	The information of locations where MNO will actually be able to provide rapid intervention  to DSO.

j)	The time stamp at which DSO’s energy distribution services are finally restored.

k)	The information of locations where (e.g. DSO substations) distribution energy service has been finally restored.

NOTE 2: Not all of the above will be configured at the time of IOC creation.

2.	MnS producer in MNO creates the MOI that contains these information attribute records for multiple sites.

3.	createMOI response is sent by MNO MnS producer to DSO.

4.	MNO creates the MOI representing the UPS related information of MNO site. It is name-contained by IOC created in step 1. This is required by DSO from MNO to achieve coordinated intelligent outage planning by DSO during its energy outage recovery. UPS related information of MNO site shall be:

4.1	Information on whether the MNO site has UPS installed/available or not.

4.2	Information on the total UPS backup capacity installed in MNO site (suggested granularity: number of in minutes) i.e. the MNO site UPS backup has installed capacity of how many minutes.

4.3	Information on the status of the remaining UPS backup capacity (suggested granularity: number of minutes) available for MNO site at any given time.

4.4	The identity of DSO energy supply meter present in MNO site.

4.5	The identity of base station present in MNO site.

5.	In order to be able to get automatically notified of any changes in the attributes information in the MOI, DSO sends createMOI request for NtfSubscriptionControl IOC to MNO.

NOTE 3: 	In step 5, if done, a subscription is created such that, subject to parameters in the NtfSubscriptionControl IOC, notifications are sent from the MNO to the DSO . This allows, for example, the MNO  to notifiy the DSO of changes in the UPS backup capacity over time. This step is shown as step 8 below.

6.	MNO MnS producer creates the MOI for NtfSubscriptionControl IOC .

7.	createMOI response is sent by MNO MnS producer to DSO.

8.	If there is a change in the attributes, the MNO sends a notifyMOIAttributeValueChanges notification to inform DSO about the changes.

NOTE 4:	In step 9 and 10 below, there is a change in an attribute of the MOI. This is done by DSO to update MNO on the changes, e.g. information about an expected outage for a specific site.

9.	DSO can modify/update any information like outage start time stamp by sending a modifyMOIAttributes request to MNO. DSO can create/read/update/delete (CRUD operations) the information in the MOI by using provisioning MnS (defined in 3GPP TS 28.532).

10.	MNO MnS producer provides the modify response to the DSO.

NOTE 5:	In step 11 and 12 below, the DSO MnS requests the current value of one or more attribute(s).

DSO can also query the UPS related information as and when required by using the getMOIAttributes operation. For example the information could include identity of DSO energy supply meter, MNO base station,  remaining UPS backup duration of a particular site, etc.

11.	MNO provides the required information in the response to the DSO.

End of Changes