3GPP TSG-SA5 Meeting #147 	S5-232902

Athens, Greece, 27th February - 3rd March 2023

# Source:	Huawei

# Title:	DP on way forward for multiple data collection mechanim

# Document for:		Discussion and Endorsement

Agenda Item:	6.1

1	Decision/action requested

The group is asked to discuss and endorse.

2	References

[1]	3GPP TS 28.532: "Management and orchestration; Architecture framework"

3	Rationale

This discussion paper related to PM_KPI_5G_Ph3 WI, FS_5GMDT_Ph2 SIand 	FS_MADCOL_ph2 SI.

For 5G Measurements, KPIs, MDT and trace data, there are several mechanisms defined to support data collection control functionality and data report functionality in several TS, see below:

- Measurements/KPIs:

Table 1 Alternative solutions for Measurements and KPIs data collection control and data report

|  | Mechanism | Solutions |
| --- | --- | --- |
| Data Collection Control | Mechanism#1: CRUD operation+PM control NRM fragment (TS28.532+TS 28.622) | SS#1.1: RESTFUL |
|  |  | SS#1.2: NETCONF/YANG |
|  | Mechanism#2: CRUD operation+managementDataCollection NRM fragment (TS28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
|  | Mechanism#3: Measurement job control (TS 28.550) | SS#3: RESTFUL |
| Data Report | Mechanism#1: File data reporting service (TS 28.532) | SS#1: RESTFUL |
|  | Mechanism#2: CRUD operation+ File retrieval NRM fragment (TS 28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
|  | Mechanism#3: Streaming data reporting service (TS 28.532) | SS#3: RESTFUL |

- MDT/Trace data:

Table 2 Alternative solutions for MDT/Trace data collection control and data report

|  | Mechanism | Solutions |
| --- | --- | --- |
| Data Collection Control | Mechanism#1: CRUD operation+Trace control NRM fragment (TS28.532+TS 28.622) | SS#1.1: RESTFUL |
|  |  | SS#1.2: NETCONF/YANG |
|  | Mechanism#2: CRUD operation+managementDataCollection NRM fragment (TS28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
| Data Report | Mechanism#1: File data reporting service (TS 28.532) | SS#1: RESTFUL |
|  | Mechanism#2: CRUD operation+ File retrieval NRM fragment (TS 28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
|  | Mechanism#3: Streaming data reporting service (TS 28.532) | SS#3: RESTFUL |

Based on above table, it is obvious that:

- 5 alternative solutions for Measurements and KPI collection control, 4 alternative solutions for Measurements and KPI data reporting.

- 4 alternative solutions for MDT/Trace data collection control, 4 alternative solutions for trace and MDT data reporting.

4	Detailed proposal

Proposal#2: It proposes to add following content in TS 28.550 and TS 32.423.

/****************Content proposed to be added in TS 28.550*************************/

Annex J: Solutions for Measurements and KPIs data collection control and data report

Table J-1 illustrate the scenarios for Measurements and KPIs data collection control and data report

Table J-1 Solutions for Measurements and KPIs data collection control and data report

|  | Mechanism | Solutions |
| --- | --- | --- |
| Data Collection Control | Mechanism#1: CRUD operation+PM control NRM fragment (TS28.532+TS 28.622) | SS#1.1: RESTFUL |
|  |  | SS#1.2: NETCONF/YANG |
|  | Mechanism#2: CRUD operation+managementDataCollection NRM fragment (TS28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
|  | Mechanism#3: Measurement job control (TS 28.550) | SS#3: RESTFUL |
| Data Report | Mechanism#1: File data reporting service (TS 28.532) | SS#1: RESTFUL |
|  | Mechanism#2: CRUD operation+ File retrieval NRM fragment (TS 28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
|  | Mechanism#3: Streaming data reporting service (TS 28.532) | SS#3: RESTFUL |

/****************Content proposed to be added in TS 32.423*************************/

Annex I: Solutions for MDT/Trace data collection control and data report

Table I-1 illustrate the Solutions for MDT/Trace data collection control and data report data collection control and data report

Table I-1 Solutions for MDT/Trace data collection control and data report

|  | Mechanism | Solutions |
| --- | --- | --- |
| Data Collection Control | Mechanism#1: CRUD operation+Trace control NRM fragment (TS28.532+TS 28.622) | SS#1.1: RESTFUL |
|  |  | SS#1.2: NETCONF/YANG |
|  | Mechanism#2: CRUD operation+managementDataCollection NRM fragment (TS28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
| Data Report | Mechanism#1: File data reporting service (TS 28.532) | SS#1: RESTFUL |
|  | Mechanism#2: CRUD operation+ File retrieval NRM fragment (TS 28.532+TS 28.622) | SS#2.1: RESTFUL |
|  |  | SS#2.2: NETCONF/YANG |
|  | Mechanism#3: Streaming data reporting service (TS 28.532) | SS#3: RESTFUL |