3GPP TSG-SA5 Meeting #147 	S5-232938

Athens, Greece, 27th February - 3rd March 2023

# Source:	Ericsson

# Title:	Adding evaluation of solutions for clause 7.1

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding evaluation of solutions for clause 7.1 covering charging in visited MNO for wholesale charging towards home MNO and correcting the issues solved by solutions #1.4 and #1.7.

# 4	Detailed proposal

First change

### 7.1.5	Evaluation

Solutions #1.1, #1.3, and #1.9 all solves key issue #1a.

-	Solution #1.1 has CDRs for the interconnect charging generated in the visited MNO using V-SMF to V-CHF. The CDR will only be generated in the visited network. This is currently supported, but not described.

-	Solution #1.3 has CDRs for the interconnect charging generated in the visited MNO using V-SMF to V-CHF and requires retail charging in the home using V-SMF to H-CHF. This is currently supported.

-	Solution #1.9 has CDRs for the interconnect charging generated in the visited MNO using V-SMF to V-CHF and requires retail charging in the home using V-SMF via the V-CHF to H-CHF.

The main difference from an interconnect perspective is that solutions #1.3 and #1.9 requires a V-SMF to H-CHF interaction while solution #1.1 don’t.

Solutions #1.2 and #1.6 both solves key issue #1b.

-	Solution #1.2 has CDRs for the interconnect charging generated in the visited MNO using AMF to V-CHF. The CDR will only be generated in the visited network. This is currently supported, but not described.

-	Solution #1.6 has CDRs for the interconnect charging generated in the visited MNO using AMF to V-CHF and requires retail charging in the home using AMF to H-CHF. This is currently supported.

The main difference from an interconnect perspective is that solution #1.6 requires a V-SMF to H-CHF interaction while solution #1.2 don’t.

Solution #1.5 solves key issue #1c.

-	Solution #1.5 has CDRs for the interconnect charging generated in the visited MNO using V-SMSF to V-CHF.

Solutions #1.8 solves key issue #1d.

-	Solution #1.8 allows for negotiation of roaming charging profile with triggers for QBC.

Solutions #1.10 solve key issue #1f.

-	Solution #1.10 have separate PDU session level triggers for FBC and QBC.

Solutions #1.1, and #1.7 both solve key issue #1g.

-	Solution #1.1 would require solution #1.7 since there is no connection to the home NMO.

Editor’s note:	Further evaluations are FFS.

End of changes