3GPP TSG-SA5 Meeting #147 	S5-232961

Athens, Greece, 27th February - 3rd March 2023                          revision of S5-232236

# Source:	AsiaInfo

# Title:	pCR 28.834 Add key issues for usecase 6 and usecase 7

# Document for:	Approval

Agenda Item:	6.8.5.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.834 v0.4.0 Study on Management of Cloud Native Virtualized Network Functions

3	Rationale

This contribution adds key issues for usecase 6 and usecase 7

4	Detailed proposal

It proposes to make the following changes to TR 28.834.

1st Change

5.6	Use case# 6: VNF package update of the cloud-native VNF

5.6.1	Description

As a pre-condition to this use case, the network is running normally. The 3GPP management system has subscribed to VNF instance lifecycle notifications from ETSI NFV MANO.

The use case begins when the 3GPP management system decides to update VNF package for a VNF instance.

The 3GPP management system requests ETSI NFV MANO to change current VNF package (as defined in clause 7.2.24 ETSI GS NFV-IFA 008 V4.3.1 [8]) for the case where EM communicates with VNFM, and clause 7.3.5 in ETSI NFV-IFA013 V4.3.1 [9] for the case where OSS communicates with NFVO).

ETSI NFV MANO inspects the current and target VNF Descriptor (as defined in clause 7.1.15 ETSI GS NFV-IFA 011 V4.3.1[12]) to differentiate possible changing values that concern to some VNF component (e.g. VDU, internal VLD, etc.) or property (e.g. a Scaling Aspect, etc.).

The ETSI NFV MANO will change of both the VNF software version and the VNF virtualised resources , including terminating the virtualised resource instances running the current software version and instantiating new virtualised resource instances with the target VNF software version.

When the VNF software version and the VNF virtualised resources have been changed, ETSI NFV MANO sends a notification to the 3GPP management system to notify that the VNF package has been updated. This notification includes information on the changes to VNF package.

### 5.6.2	Issues

TS 28.526 [10] describes the procedures to be used by the 3GPP management system to update. However, TS 28.526 [10] is not compatible with 5G/SBMA architecture and only contains reference of the release 2 specifications from ETSI NFV which do not support containerized VNF LCM.

5.6.3		Requirements

REQ-CVNF_Updating-1  The 3GPP management system shall be able to send a request for VNF packager update of the cloud-native VNF to ETSI NFV MANO

REQ-CVNF_Updating-2  The 3GPP management system shall be able to receive notification from ETSI NFV MANO about VNF package update of cloud-native VNF.

5.6.3		Potential solution

The 3GPP management system invokes the UpdateNsRequest operation (see clause 7.3.5 in ETSI GS NFV-IFA 013 [9]) to request NFVO via the Os-Ma-nfvo interface to update VNF package of the cloud-native VNF. When the NFVO receives the UpdateNsRequest for VNF package update of cloud-native VNF from the 3GPP management system, the NFVO sends the NS Lifecycle Change notification to the 3GPP management system indicating the result of VNF package update of cloud-native VNF(see clause 7.3.12 of ETSI GS NFV-IFA 013 [6]).

2st Change

## 5.7	Use case# 7: Performance monitoring of the cloud-native VNF using generic OAM functions

### 5.7.1	Description

This use case is about the performance monitoring of cloud-native VNFs using the " VNF metrics aggregator function and VNF metrics analyser function", which belongs to generic OAM functions proposed in [4]. The VNF metrics aggregator function can collects the metrics from the VNF/VNFC/NFV-MANO, the VNF metrics analyser function can analyses the metrics provided by the VNF metrics aggregator and can be configured to send notifications based on e.g. statistical processing, abnormal behaviour detection, or threshold crossing.

The 3GPP management system sends a performance monitoring request to the VNF metrics analyser, then the VNF metrics analyser retrieves the metrics of the VNF/VNFC/NFV-MANO from the VNF metrics aggregator.

The VNF metrics analyser processes and evaluates the metrics (e.g. analysis and identifies possible issues), the 3GPP management system will receive the metrics analytics result from VNF metrics analyser function.

### 5.7.2	Issues

The APIs related to the VNF metrics aggregator function and VNF metrics analyser function have not been published yet as an ETSI NFV solution. ETSI NFV may publish new APIs as a result of normative work, as a result 3GPP specifications may need to be updated to refer to the new APIs.

### 5.7.3	Requirements

REQ-CVNF_TM_CON-1 The 3GPP management system shall be able to send a performance monitoring request for a cloud-native VNF to the VNF metrics analyser function.

REQ-CVNF_TM_CON-2 The 3GPP management system shall be able to receive the notify from VNF metrics analyser function about the performance monitoring of cloud-native VNFs.

3st Change