3GPP TSG-SA5 Meeting #147 	S5-232962

Athens, Greece, 27th February - 3rd March 2023                          revision of S5-232441

# Source:	AsiaInfo

# Title:	pCR 28.834 Add use case and solution for healing of cloud-native VNF

# Document for:	Approval

Agenda Item:	6.8.5.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.834 v0.4.0 Study on Management of Cloud Native Virtualized Network Functions

3	Rationale

This contribution describes the use case and potential solution for healing of cloud-native VNF

4	Detailed proposal

It proposes to make the following changes to TR 28.834.

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- NGMN Cloud Native Enabling Future Telco Platforms v5.2. 17. May 2021

- ETSI GR NFV-IFA029: “Report on the Enhancements of the NFV architecture towards cloud-native and Paas”

- ETSI GR NFV-EVE019: “Report on VNF generic OAM functions”

- ETSI GR NFV-IFA037: “Report on further NFV support for 5G”

- ETSI GS NFV-IFA040: “Requirements for service interfaces and object models for OS container management and orchestration specification”

- ETSI GS NFV-EVE 011 “Specification of the Classification of Cloud Native VNF implementations”

- ETSI GS NFV-IFA 008 V4.3.1 (2022-05) : "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

- ETSI GS NFV-IFA013 V4.3.1 (2022-06) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

- 3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

- 3GPP TS 28.533: "Management and orchestration; Architecture framework".

- ETSI GS NFV-IFA011 V4.3.1 (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; VNF Descriptor and Packaging Specification".

- ETSI GS NFV-IFA 049 V0.0.3 “Network Functions Virtualisation (NFV) Release 4; Architectural Framework；VNF generic OAM functions specification”.

- 3GPP TS 28.516: "Fault Management (FM) for mobile networks that include virtualized network functions; Procedure".

2st Change

5. X	Use case# X: Healing of cloud-native VNF

5.X.1	Description

As a pre-condition to this use case, the 3GPP management system has subscribed to VNF instance lifecycle notifications from ETSI NFV MANO.

The use case begins when a faulty virtualization-specific aspect of the VNF has been notified to the 3GPP management system.

the 3GPP management decide to initiate a healing procedure to recover the faulty virtualization-specific aspects of the VNF.

The 3GPP management system requests ETSI NFV MANO to execute healing VNF with indication of VNF Components to recover and indication of the healing procedure to execute (as defined in clause 7.2.10 ETSI GS NFV-IFA 008 [8] for the case where EM communicates with VNFM, and ETSI NFV-IFA013 V4.3.1 [9] for the case where OSS communicates with NFVO).

ETSI NFV MANO sends a notification to 3GPP management system about the start of the VNF healing execution.

ETSI NFV MANO identifies the VNF instance and executes the procedure to heal the VNF instance.

ETSI NFV MANO sends a notification to 3GPP management system about the end and the result of the VNF Healing execution, this notification includes success or failure. In case of success, the VNF/VNFC has been healed, and the associated instance of a VnfInfo information element has been updated. In case of failure, appropriate error information is provided in the notification.

5.X.2		Issues

TS 28.516 [14] describes the procedures to be used by the 3GPP management system to request healing. However, TS 28.516 [X] is written in terms (for example Element Manager) which are not compatible with the 5G architecture as described in TS 28.533 [11].

5.X.3		Requirements

REQ-CVNF_Healing -1: The 3GPP management system shall have a capability to interact with ETSI NFV MANO for healing a cloud-native VNF

REQ-CVNF_Healing -2: The 3GPP management system shall have a capability to interact with ETSI NFV VNFM for healing a cloud-native VNF

3st Change

6.X	Potential solution# X: Solution for healing of cloud-native NFV

6.X.1	Solution for healing of cloud-native NFV with ETSI NFV MANO

6.X.1.1	Introduction

The following solution corresponds to the use case # X Healing of cloud-native VNF.

In this solution, 3GPP management system interacts with ETSI NFV MANO when initiating healing a cloud-native VNF.

6.X.1.2		Description

3GPP management system determines to initiate a healing procedure to recover the faulty virtualization-specific aspects of the cloud-native VNF. The 3GPP management system invokes the heal NS operation (see clause 7.3.9 in ETSI GS NFV-IFA 013[9]) to request NFVO via the Os-Ma-nfvo interface to heal a cloud-native VNF with HealVnfData as included. When the NFVO receives the heal NS operation of a cloud native VNF from the 3GPP management system, the NFVO sends the NS Lifecycle Change notification to the 3GPP management system indicating the result of Heal NS operation (see clause 7.3.12 of ETSI GS NFV-IFA 013[6]).

6.X.2	Solution for healing of cloud-native NFV with ETSI NFV VNFM

6.X.2.1	Introduction

The following solution corresponds to the use case # X Healing of cloud-native VNF.

In this solution, 3GPP management system interacts with ETSI NFV VNFM when initiating healing a cloud-native VNF.

6.X.2.2		Description

3GPP management system determines to initiate a healing procedure to recover the faulty virtualization-specific aspects of the cloud-native VNF. The 3GPP management system invokes the heal VNF operation (see clause 7.2.10 in ETSI GS NFV-IFA 008[8]) to request VNFM via the Ve-Vnfm-em interface to heal a cloud-native VNF. When the VNFM receives the heal VNF operation of a cloud native VNF from the 3GPP management system, the VNFM sends the VNF Lifecycle Change notification to the 3GPP management system indicating the result of Heal VNF operation (see clause 7.2.15 of ETSI GS NFV-IFA 008[8]).

4st Change

7	Conclusions and recommendations

7.X Issue #X Healing of cloud-native VNF

It is recommended that 3GPP management system interacts with ETSI NFV MANO for healing a cloud-native VNF. The operation as defined in release 4 of ETSI GS NFV-IFA013 [9] should be used.

It is recommended that 3GPP management system interacts with ETSI NFV VNFM for healing a cloud-native VNF. The operation as defined in release 4 of ETSI GS NFV-IFA008 [8] should be used.

The detailed solution see clause 6.X