3GPP TSG-SA5 Meeting #147 	S5-232988

Athens, Greece, 27th February - 3rd March 2023

# Source:	Huawei

# Title:	Improve requirements for network slice traffic prediction

# Document for:	Approval

Agenda Item:	6.4.2.2

# 1	Decision/action requested

For approval.

# 2	References

[1]	S5-227066 Draft CR 28.104 Use case and Requirements for Prediction and Statistics for Management data

# 3	Rationale

Requirements for network slice traffic prediction use two different terminology to describe the relation between network functions and a network slice instance. This contribution clarifies that the network functions are constituents of a network slice instance.

Requirement REQ-TRA_MDA-02 may be read as saying that the full list of information shall be provided. This contribution clarifies that only the applicable information shall be provided.

# 4	Detailed proposal

The following changes are proposed to [1]

1st change

#### 7.2.2.3	Network slice traffic prediction

##### 7.2.2.3.1	Description

This MDA capability is for the prediction of network slice traffic patterns.

##### 7.2.2.3.2	Use case

It is desirable to use MDAS to get the network slice traffic predictions including individual traffic load predictions on each of the constituent network function instance present in the network slice. The traffic load predictions per constituent network function instances can be used for better resource provisioning of the network slice. For example, resources can be pre-configured considering the predicted traffic on the network slice.

##### 7.2.2.3.3	Requirements

Table 7.2.2.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-TRA_MDA--01 | MDA capability for network slice traffic prediction shall be able to provide analytics output describing traffic load prediction of the network slice including traffic load prediction for each of its constituent network function instances. | Network slice traffic prediction |
| REQ-TRA_MDA-02 | MDA capability for network slice traffic prediction shall be able to provide analytics output describing traffic load prediction for the network slice which include (where applicable) the following information: - Predicted uplink and downlink throughput on each constituent User Plane Function instance (UPF). - Predicted number of Packet Data Unit (PDU) session for each constituent Session Management Function (SMF) instance. - Predicted number of UE or Registered subscriptions for each constituent AMF instance. - Predicted maximum packet size for each constituent UPF instance. - Predicted UE uplink and downlink throughput on each constituent gNodeB (gNB) instance. - Predicted number of UE for each constituent gNB/NR cell instance. | Network slice traffic prediction |

End of changes