3GPP TSG-SA5 Meeting #147	S5-232995

# 27 February - 03 March 2023, Athens, GR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Input to Draft CR] Add MDA capability for resource utilization analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The 3GPP system is a resource limited system, no matter whether the NF is working on virtualized resources or physical resources. Resource shortage would affect the QoS and potentially impact users’ quality of experience (QoE), e.g., by lowering the users’ data throughput, prolonging the users’ data delay, raising the rejections for the establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increasing the drops of the existing connections, sessions, and resources. Resource excess would cause wastage that leads to additional CapEx and OpEx. Therefore, it is imperative to ensure optimum and efficient resource utilization for the NFs. The resource utilization of an NF is heavily dependent on traffic patterns, which could vary in different areas (e.g., business area, entertainment area, and residential area) in different time periods. It is desirable that the spare resource of the low-usage areas can be allocated to the busy areas. It is expected that MDA can perform an analysis of the resource utilization for NFs to indicate the resource usage patterns in the past and predict the resource usage trend for some time periods in the future. It is also very useful that MDA provides recommendations to orchestrate the resources among NFs between the low usage and high usage areas for some time periods. The recommended actions could be for example to schedule the "scale in" and "scale out" of VNFs to optimize the allocation of the virtualized resources. The resource utilization analysis is also studied in TR 28.809. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added UC, requirements, and solutions for resource utilization analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource utilization analysis cannot be supported by MDA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x (new), 8.4.x (new), 8.5.x (new), 8.5.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 7.2.x	Resource related analytics

#### 7.2.x.1	NF resource utilization analysis

##### 7.2.x.1.1	Description

This MDA capability is for analysis of resource utilization of 3GPP NFs.

##### 7.2.x.1.2	Use case

The 3GPP system is a resource limited system, no matter whether the NF is working on virtualized resources or physical resources.

Resource shortage would affect the QoS and potentially impact users’ quality of experience (QoE), e.g., by lowering the users’ data throughput, prolonging the users’ data delay, raising the rejections for the establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increasing the drops of the existing connections, sessions, and resources.

Resource excess would cause wastage that leads to additional CapEx and OpEx.

Therefore, it is imperative to ensure optimum and efficient resource utilization for the NFs.

The resource utilization of an NF is heavily dependent on load or traffic patterns, which could vary in different coverage areas (e.g., business area, entertainment area, and residential area) and in different time periods (weekdays and time of the day). It is desirable that the spare resource of the low-usage areas can be allocated to the busy areas.

It is expected that MDA can perform an analysis of the resource utilization for physical NFs or virtualized NFs to indicate the resource usage patterns in the past and predict the resource usage trend for some time periods in the future.

For physical NFs(e.g., gNBs), the resource to be analyzed may include hardware resource (e.g., CPU), DL and UL PRBs (for gNB),  etc.

For virtualized NFs, the resources to be analyzed may include virtual CPU, virtual memory, virtual disk, etc.

It is also very useful that MDA provides recommendations that can be utlised to efficiently orchestrate the resources among NFs between the low usage and high usage areas for some time periods. The recommended actions could be for example to schedule the "scale in" and "scale out" of VNFs to optimize the allocation of the virtualized resources.

##### 7.2.x.1.3	Requirements

Table 7.2.1.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-RES_UTI_ANA-01 | MDA capability for resource utilization analysis shall be able to identify the physical NFs with low or under-utilised resource usage in a specific coverage area or a sub-network during some time periods in the past. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-02 | MDA capability for resource utilization analysis shall be able to identify the physical NFs with high resource usage in a specific coverage area or a sub-network during some time periods in the past. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-03 | MDA capability for resource utilization analysis shall be able to provide the prediction of resource usage of physical NFs in a specific area or a sub-network for some time periods in the future. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-04 | MDA capability for resource utilization analysis shall be able to identify the virtualized NFs with low or under-utilised resource usage in a specific coverage area or a sub-network during some time periods in the past. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-05 | MDA capability for resource utilization analysis shall be able to identify the virtualized NFs with high resource usage in a specific coverage area or a sub-network during some time periods in the past. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-06 | MDA capability for resource utilization analysis shall be able to provide the prediction of resource usage of virtualized NFs in a specific area or a sub-network for some time periods in the future. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-07 | MDA capability for resource utilization analysis shall be able to provide recommended actions to orchestrate the resource allocation for NFs. | NF resource utilization analysis |

End of modified sections