3GPP TSG-SA5 Meeting #147	S5-233021

# Athens, Greece, 27 February – 3 March 2023

# Source:	Huawei, CMCC, Nokia, Nokia Shangai Bell

# Title:	pCR 28.830 add description of management capability

# Document for:	Approval

Agenda Item:	6.7.7.1

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	S5-225661: "draft TR 28.830 Fault supervision evolution"; v0.3.0

[3]	TS 32.111-1: "Telecommunication management; Fault Management; Part 1: 3G fault management requirements (Release 17)”

[4]	TS 28.532: “Management and orchestration; Generic management services; (Release 17)”

[5]	ITU-T Recommendation X.733 (1992) | ISO/IEC 10164-4: 1992, Information technology – Open Systems Interconnection – Systems Management – Alarm reporting function.

[6]	TS 28.104: "Management Data Analytics (MDA)"; v17.1.1

[7]	TR 28.809: "Management and orchestration; Study on enhancement of Management Data Analytics (MDA)"; v17.0.0

[8]	S5-232552: "pCR TR 28.830 Add description of fault related analysis".

[9]	S5-226947d1: "pCR TR 28.830 Add alarm related definitions"

3	Rationale

This tdoc discusses fault related analysis and prediction capability in FSEV study [1, 2].

Basic alarm/fault related management capabilities:

Alarm notification related interfaces are defined in existing specs for Alarm/fault management, e.g. TS 28.532, TS 28.541, TS 32.111-1, TS 32.111-2 etc.

Analytics management capabilities related to Alarm/fault:

There are texts regarding analytics management capabilities in existing specs for Alarm/fault management in TS 32.111-1 [3], e.g., FM also includes…such as…the provision and analysis of the alarm and state history of the network; “4.1.9 Root Cause Analysis” etc.

The MDA capabily “Failure prediction” has been specified for MDA assisted fault management in TS 28.104 [6]. Some fault related analysis capabilities have been included in TR 28.809 [7], which are KPI anomaly analysis, alarm incident analysis and alarm malfunction analytics.

In [1], the relation and interaction with MDAS is one of the objectives to study in FS_FSEV. MDA capability may be used or enhanced in the FSEV study for the alarm/fault related analysis.

Discussions:

Existing FM specs have not defined concrete fault related analysis capabilities, instead, the analysis information may be implicated in existing alarm notifications e.g. probableCause, trendIndication, correlatedNotifications, proposedRepairActions, rootCauseIndicator etc [3, 4, 5].

In TS 28.104 [6], the MDA capabily “Failure prediction” has implemented more concrete analytical information such as failurePredictionObject, potentialFailureType, issueID, perceivedSeverity etc.

In [9], the alarm related definitions are discussed. The differences of the terms error, failure and fault are identified. For the MDA capability “Failure prediction”, the termilogies need to be clarified.

In TR 28.809 [7]:

The analytics report describing the alarm incident should include Alarm incident Identifier, List of Correlated AlarmInfo, List of Correlated performance measurements info, Location, Affected objects and object relations etc.

The analytics report describing alarm malfunction information should include the location related to the alarm malfunction or faulty performance measurement(s), the affected object(s) related to malfunction alarm or faulty performance measurement(s) etc.

The analytics report describing the KPIs anomaly should include the information describing the KPI anomaly aspects and potentially future prediction, e.g., the predictive anomaly KPI(s) or observed anomaly KPI(s), Cross domain and domain KPI anomaly analysis, the recommendations for the configurations of network resource and KPI threshold etc.

NOTE: KPI anomaly analysis in TR 28.809 is not classified as the fault management related analysis. However, they are relevant MDA capabilities.

These analytical reports of MDA assisted fault management provide more concrete analysis information based on correlate analysis of multiple data sources e.g. performance measurement information, configuration data, network topology information, etc as described in TS 28.104 [6].

Observation 1: In addition to MDA capability of failure prediction, more alarm/fault related analysis scenarios and capabilities should be considered. The terms of error, failure and fault need to be clarified.

Proposal 1: Potential enhancements for new alarm/fault related analytics capabilities should be studied based on MDA assisted fault management in FS_FSEV. Existing alarm data is needed as one of the data sources for the analysis.

4	Detailed proposal

1st Change

1	Scope

The present document investigates potential use cases, requirements and solutions (from the perspective of service based management architecture) for the fault supervision evolution, its relation with performance management and fault supervision, relation and interaction with MDAS and COSLA, and potential enhancements for MDA assisted fault management. The document provides conclusions and recommendations on the normative work.

2nd change

4	Background

Since several decades the telecommunication management network offers a multitude of possibilities to inform about specific states of the system [2, 3], errors and faults by using alarms [4, 5], and about the performance related indications like counters, KPI gauges, aggregations, statistics, and thresholds, e.g [6 - 9].

Already the first paragraph on the model of alarm reporting [4, clause 7] describes the importance to use thresholds and to detect trends in order to provide warnings to the managers. This means the managed systems are encouraged to use means to detect abnormal conditions as early as possible in order to inform the management system by standardized means about the situation. Any new proposal has to consider already existing solutions in order to avoid diverging, non-interoperable frameworks.

[4, clause 7] also highlights the importance to correlate multiple events. While the correlation is an internal function of management systems, the interfaces are supporting the correlation by specific fields to associate multiple events to each other. This also is true for the corresponding 3GPP specifications, which to a large extent are based on the specifications by ITU-T. Correlation in existing specifications mainly covers alarm notifications, although other type of data e.g. normal performance measurements, KPIs, historical data etc could also be considered for more comprehensive analysis.

The combination of alarm reporting and state management would be able to reduce the number of alarm messages very efficiently if certain best practices are followed: If alarms are used to indicate that a resource requires maintenance, and states are used to inform about the well-being of a resource.

For example, in case a backhaul link towards a gNB has a problem, many logical and physical interfaces of the gNB, many protocol layers, and all cells will experience certain abnormal conditions. If all these resources are raising alarms, then the management system will choke in alarms -although none of these alarms requires any maintenance, since the problem is caused by the link, while the base station as such has no problem at all.

If in such situation the resources would consider the rule to issue alarms only in case they require maintenance, then the base station would not send any alarm, while all affected resources would set their operational state to “disabled” and the availability state to “dependency”. In this case the human operator would be aware that the base station does not work as expected and would be also aware of the fact that the base station as such does not require any maintenance. However, although the mechanism described above have been standardized by ITU-T in 1992, such mechanisms are not applied in currents systems. Reducing the number of alarms in the network elements by simple filtering of alarms imposes the risk to miss important information that might be needed by other management functions. Therefore, it requires the network elements to perform thorough correlation of notifications and state information in order to suppress redundant information only, but not to suppress information that is needed by higher level management tools.

It is an unfortunate fact that -since ever- the management systems as well as the human operators are choking in alarms, although a combination of alarm reporting and state management would offer a technical means to reduce the number of alarms. As a matter of fact, the determination whether an abnormal behavior is caused by an entity itself or by another entity (or subsystem) requires sophisticated correlation functions, that must be able be reliable in order to avoid erroneous correlations resulting in false statements about the root cause.. Implementation of such functions require high implementation effort because it requires the knowledge of all dependencies.

An additional problem is that TS 28.532, clause 11.2, which defines the Fault Supervision MnS, does not provide the necessary definitions and descriptions required to understand the current state of art as to alarm management. This is because much of the material specified and available for the IRP Framework was not moved to SBMA.

For that reason, this study investigates which definitions and descriptions need to be added to TS 28.532, clause 11.2 to make this clause understandable without need to consult other specifications. Besides descriptions for alarm management, the role and importance of state management shall be highlighted as well.

It is also in scope of this study to look at possibilities to clarify in TS 28.532 that internal behavior of functions is not subject to standardization. For example, the algorithm used to accomplish alarm correlation is outside the scope of standards. This implies that deliberations on if AI/ML is used for correlation or not is also outside the scope of standards. It is a vendor decision to use AI/ML or not.

The scope of this study include potential enhancements to MDA assisted fault management. MDA capability may be used or enhanced for the fault related analysis. The MDA capabily “failure prediction” supports the prediction of the running trend of network and potential failures to intervene in advance. More alarm/fault related analysis scenarios and capabilities will be studied. Existing alarm data is needed as one of the data sources for the analysis.

3rd change

5.1	Key Issue 1: Missing definitions

5.1.1	Description

TS 28.532, clause 11.2, which defines the Fault Supervision MnS, does not provide the necessary definitions and descriptions required to understand the current state of art as to alarm management. This is because much of the material specified and available for the IRP Framework was not moved to SBMA.

5.1.2	Potential solutions

It is proposed to add the following definitions to an appropriate TS:

Event: Anything that occurs at a certain point in time, for example a configuration change, a threshold crossing, a transition to an error state or a transition to a failure state. Events do not have states.

Error: A state of the system different from the correct system state as defined by the service specification. An error may or may not lead to a service failure. An error has a begin and end time.

Failure: A state of inability to deliver the correct service as defined by the service specification. A service failure may be the result of an error or a poor service function design.

Fault: The (hypothesized or adjudged) cause for an error or a failure.

Alarm: An error or failure that requires attention or reaction by an operator or some machine. Alarms have state.

Root cause: The primary fault (cause), if any, leading to one or multiple errors or failures.

End of change