| 3GPP TR 28.903 V0.5.0 (2023-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on alignment with ETSI MEC for Edge computing management (FS_MEC_ECM) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Related Work in Other SDOs	7

4.1 ETSI MEC	7

4.2 GSMA OPG	8

5	Issue investigations and potential solutions for alignment with ETSI MEC	10

5.1	Key Issue# 1: Management of collocated platforms for EES and MEC platform	10

5.2	Key Issue# 2: Application lifecycle management	11

5.2.2.1	Potential solution#1	12

5.2.2.1.1	Description	12

5.2.2.1.2	Solution evaluation	12

5.2.2.2	Potential solution# mapping application lifecycle management operation produced by MEO/MEAO with EAS LCM requirements	12

5.2.2.2.1	Description	12

5.2.2.2.2	Solution evaluation	13

6	Issue investigations and potential solutions for alignment with GSMA OPG	14

6.1	Key Issue# 1: Issue for resource reservation	14

6.1.2.1	Potential solution#1	14

6.1.2.2	Solution evaluation	15

6.2.	Key Issue#2: Issue for federation management	16

6.3.	Key Issue#3: EDN Sharing	17

6.4.	Key Issue#4: Federated EAS resource reservation management	18

6.5.	Key Issue#5: Federated EAS deployment and termination	18

6.6	Key Issue# 6: Virtual Infrastructure Resource Requirements	18

6.7	Key Issue# 7: Querying Available Edge Services	19

6.8	Key Issue# 8: SLA Requirements Key Issue	19

6.9	Key Issue# 9: EAS relocation policies Key Issue	20

6.10	Key Issue# 10: Application Resource Catalogue	21

7	Conclusion and recommendation	21

Annex A (informative): Change history	22

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This report is to study on alignment with ETSI MEC for Edge computing management.

1	Scope

The present document studies the solutions to support alignment with ETSI MEC from the management perspective for edge computing.

In this TR, it also identifies potential issues and solutions to support GSMA OPG [2] requirement in coordination with ETSI MEC [3].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	GSMA Operator Platform Telco Edge Requirements - https://www.gsma.com/futurenetworks/wp-content/uploads/2022/04/GSMA-Operator-Platform-Telco-Edge-Requirements-2022-v2.0.pdf

[3]	ETSI GS MEC 010-2 (v 2.2.1) (2022-02): " Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[4]	ETSI ISG MEC 003, "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[6]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management (ECM)".

[7]	ETSI GS NFV-IFA 013 V4.3.1 "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Os-Ma-nfvo reference point -Interface and Information Model Specification".

[8]	GSMA Operator Platform – East-Westbound Interface APIs - https://www.gsma.com/futurenetworks/resources/platform-group-4-0-federation-api-1-0-0-yaml/

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[10]	3GPP TS 28.532: "Management and orchestration; Generic Management Service"

[11]	3GPP TR 28.817: "Management and orchestration; Study on access control for management service"

[12]	3GPP TR 28.824: "Management and orchestration; Study on network slice management capability exposure"

[13]	3GPP TR 23.700 98 v1.0.0: "Study on Enhanced architecture for enabling Edge Applications"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Related Work in Other SDOs

4.1 ETSI MEC

Multi-access Edge Computing (MEC) framework as specified in ETSI ISG MEC 003 [4] enables the implementation of MEC applications as software-only entities that run on top of a Virtualisation infrastructure, which is located in or close to the network edge.

Figure 4.1-1: Multi-access Edge Computing framework

Figure 4.1-1 shows the MEC framework as defined in [4], it mainly contains several components,

The MEC host is an entity that contains a MEC platform and a Virtualisation infrastructure which provides compute, storage, and network resources, for the purpose of running MEC applications.

The MEC platform is the collection of essential functionality required to run MEC applications on a particular Virtualisation infrastructure and enable them to provide and consume MEC services. The MEC platform can also provide services.

MEC applications are instantiated on the Virtualisation infrastructure of the MEC host based on configuration or requests validated by the MEC management.

The MEC system level management includes the MEC orchestrator as its core component, which has an overview of the complete MEC system. When relying on the NFV Orchestrator (NFVO) for resource orchestration and for orchestration of the set of MEC application VNFs, the MEC Orchestrator (MEO) is replaced by a MEC Application Orchestrator (MEAO), the architecture is described in clause 6.2.2 of [4].

The MEC host level management comprises the MEC platform manager and the Virtualisation infrastructure manager, and handles the management of the MEC specific functionality of a particular MEC host and the applications running on it.

In Annex C of 3GPP TS 23.558 [5], both EDGEAPP and ETSI MEC can provide support for hosting different edge applications, it further indicates that "Both EAS and MEC application are application servers and can provide similar application specific functionalities. EAS utilizes the services of EES as specified in this document whereas MEC application utilizes the services provided by MEC platform as specified in ETSI GS MEC 003 [4]. The EAS and MEC application can be collocated in an implementation.".

4.2 GSMA OPG

The Operator Platform (OP) is defined by GSMA OPG [2], it facilitates access to the Edge Cloud capability of an Operator or federation of operators and their partners.

The architecture scope under definition is shown below,

Figure 4.2-1: OP Roles and Interfaces Reference Architecture

This study is focused on the NBI requirements as defined in [2]. The NBI is the interface between the application provider and the Capabilities Exposure Role in the Operator Platform, it allows an OP to advertise the above cloud capabilities that it can provide to application providers. In addition, the NBI allows an application provider to reserve a set of resources or request an Edge Cloud service with the resources and features that they require and for the OP to accept or reject the request.

In this study, it is assumed that the application provider as described in [2] maps to ASP in TS 28.538 [6], and the Operator Platform as described in [2] maps to ECSP management system in TS 28.538 [6].

The following table provides the mapping of concepts (not exhaustive) defined in TS 28.538[6] with the concepts defined in GSMA OPG [2].

| GSMA[x] | ECM(TS 28.538) | Comment/Observations |
| --- | --- | --- |
| Edge Application | EAS | Edge Application and EAS are both referring to the application running in the edge. |
| Application Provider | Application Service Provider | Application Provider and Application Service Provider both referring to the application providers producing and requesting for the deployment of the applications. |
| Edge Cloud | EDN | The Applications are instantiated on an OP managed Edge Cloud/Cloudlet. The edge Cloud contain resources utilized to provide edge services. EAS are instantiated on an EDN. Since, the EAS are mapped to Applications; EDN can be mapped to Edge Cloud. |
| Operator Platform | ECSP Management System | Both Operator Platform (Capability Exposure Role) and the ECSP Management System are the entities which exposes interface and management service towards ASP. |
| Northbound Interface | Management services for Edge Computing lifecycle management | NBI maps to management service, enabling LCM for EAS, exposed towards ASP. Note: The term “NBI” does not relates to edge computing only, rather it is a generic term which can apply elsewhere. |

5	Issue investigations and potential solutions for alignment with ETSI MEC

5.1	Key Issue# 1: Management of collocated platforms for EES and MEC platform

5.1.1	Description

As described in [5], one of the deployment option is that the EES and MEC platform can be collocated in an implementation.

The EES instantiation procedure is described in clause 7.1.4 of TS 28.538 [6], which the PLMN operator or ECSP requests the EES instantiation via provisioning MnS to ECSP management system, the simplified procedure is show in Figure 5.1.1-1,

Figure 5.1.1-1: EES deployment procedure

The MEC platform as described in ETSI ISG MEC 003 [4] can be deployed as a VNF by using ETSI NFV MANO, the simplified procedure is show in Figure 5.1.1-2,

Figure 5.1.1-2: MEC platform deployment procedure

In the collocated scenario, the EES and MEC platform can be deployed in the same Physical/NFV Infrastructure by the same operator. The MEC platform can be deployed first, then the operator can deploy the EES at the same Physical/NFV Infrastructure where the MEC platform is deployed.

REQ-EES-deplyment-1: ECSP management system shall have the capability to instantiate the EES to a specific Physical/NFV Infrastructure that collocated with MEC platform.

### 5.1.2	Potential solutions

#### 5.1.2.1	Potential solution#1

In this option, a PLMN operator deploys both EES and MEC platform in the same EDN.

A NSD (network service descriptor) which contains both EES and MEC platform information (e.g. VnfdId) is used by the PLMN operatior. Within the NSD, the VnfProfiles (see clause 6.3.3 of ETSI GS NFV-IFA 014 [x]) of EES and MEC platform contain the same affinityOrAntiAffinityGroupId, which could be defined as the following,

affinityOrAntiAffinity: AFFINITY

scope: NFVI_NODE

The MEC platform can be deployed first, then for EES instantiation, ECSP management system uses the NSD as described above for requesting instantiation of EES VNF to be collocated with the MEC platform by interworking with ETSI NFV MANO.

#### 5.1.2.2	Solution evaluation

This solution does not have any impact on the current EES deployment procedure.

5.2	Key Issue# 2: Application lifecycle management

### 5.2.1	Description

3GPP TS 28.538 [6] specifies the EAS lifecycle management procedure by interworking with ETSI NFV MANO by invoking InstantiateNsRequest or UpdateNsRequest operation as specified in ETSI NFV IFA013 [7]. An NFV Network Service (NS) as defined by ETSI NFV is a composition of Network Functions (NF) (VNF or PNF).

With ETSI MEC [3], MEO (when ETSI NFV is not involved) or MEAO (when ETSI NFV is involved) provides the application instance lifecycle management interface, which is similar to VNF LCM interface. The consumer of MEO or MEAO does not need to take creation of NS into consideration when requesting creation of an application instance.

The main issue related to application LCM is the following:

1, when and how ECSP management system create an NS when receiving the creating EAS request from ASP.

2, whether the application instance lifecycle management interface produced by MEO or MEAO could be used for EAS lifecycle management.

### 5.2.2	Potential solutions

5.2.2.1	Potential solution#1

5.2.2.1.1	Description

This solution addresses the first issue for the application lifecycle management.

In this solution, ECSP management system automatically creates an NSD for EAS creation. The procedure can refer to Figure 7.1.2.1-1 in TS 28.538 [6]. For the same ASP, ECSP management system could use a single NS for all its EAS VNFs instantiation. For example, after step 7 in Figure 7.1.2.1-1 in TS 28.538 [6], when the same ASP requests to deploy another EAS, ECSP management system could decide to use the same NS as created in step 4, and invokes UpdateNsRequest operation with NFVO for instantiating a new EAS VNF instance.

5.2.2.1.2	Solution evaluation

This solution does not impact the current EAS deployment procedure, creation of NSD is only the internal procedure in ECSP management system.

5.2.2.2	Potential solution# mapping application lifecycle management operation produced by MEO/MEAO with EAS LCM requirements

5.2.2.2.1	Description

This solution addresses the second issue for the application lifecycle management.

The application lifecycle management interface produced by MEO/MEAO is described in clause 6.3.1 in ETSI GS MEC 010-2 [3], which contains the following operations:

Create application instance identifier.

Application instantiation.

Application instance terminate.

Delete application instance identifier.

Query application instance information.

Change application instance state.

Query application lifecycle operation Status.

Subscribe to notifications relating to application lifecycle management.

The EAS lifecycle management requirements are described in clause 5.1.14 in 3GPP TS 28.538 [6], the below table describes the mapping between EAS lifecycle management requirement and appropriate application lifecycle management operation produced by MEO/MEAO.

Table 5.2.2.2.1-1: Mapping between EAS lifecycle management requirement and application lifecycle management operation produced by MEO/MEAO

| Requirement label | Description | Related use case(s) | Mapping with operation produced by MEO/MEAO |
| --- | --- | --- | --- |
| REQ-EAS-INST-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the deployment of EAS based on the given deployment requirements. | EAS Deployment | Can be achieved by Application instantiation operation |
| REQ-EAS-INST-FUN-2 | Generic Provisioning MnS Producer should have the capability to deploy EAS at a suitable EDN which can support the EAS requirements e.g. serving location, required latency, affinity/anti-affinity with other EAS, service continuity. | EAS Deployment | Can be achieved by virtualComputeDescriptor, osContainerDescriptor, virtualStorageDescriptor, locationConstraints information included in Application instantiation operation. |
| REQ-EAS-INST-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of instantiation as the response to the deployment request. | EAS Deployment | Can be achieved by Query application lifecycle operation Status operation. |
| REQ-EAS-INST-FUN-4 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g. success, failure) of instantiation operation. | EAS Deployment | Can be achieved by Subscribe to notifications relating to application lifecycle management |
| REQ-EAS-TERM-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the termination of the EAS VNF instance. | EAS Termination | Can be achieved by Delete application instance identifier operation |
| REQ-EAS-TERM-FUN-2 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of termination as the response to the termination request. | EAS Termination | Can be achieved by Query application lifecycle operation Status operation. |
| REQ-EAS-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g. success, failure) of termination operation. | EAS Termination | Can be achieved by Subscribe to notifications relating to application lifecycle management |
| REQ-EAS-QUERY-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to obtain the EAS instance information. | Query EAS information | Can be achieved by Query application instance information. |
| REQ-EAS-MOD-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the modification of the EAS instance. | EAS Modification | Can be achieved by Change application instance state operation. |

This solution proposes the EAS lifecycle management requirments to be achieved by application lifecycle management operation produced by MEO/MEAO based on Table 5.2.2.2.1-1.

5.2.2.2.2	Solution evaluation

This solution is based on the mapping of EAS lifecycle management requirements with application lifecycle management operations produced by MEO/MEAO as described in Table 5.2.2.2.1-1. This solution requires ECSP management system acts as the consumer of application lifecycle management operation produced by MEO/MEAO, no required changes for the architecture of ECSP management system.

6	Issue investigations and potential solutions for alignment with GSMA OPG

6.1	Key Issue# 1: Issue for resource reservation

### 6.1.1	Description

As described in clause 2.2.2.3 GSMA OPG [2], resource reservation is one of its NBI requirements, it indicates that,

- The OP (Operator Platform) shall enable Application Providers to express the resource (e.g., compute, networking, storage, acceleration) requirements that the Application Provider wants to be guaranteed.

- The NBI shall allow an Application provider to request a set of resources to be booked.

- The NBI allows an Application Provider to reserve resources ahead of the application onboarding and unrelated to any specific application, only related to the Application Provider themselves. The NBI allows an Application Provider to consume the reserved resources when onboarding a new application, creating the association between the resources and the application (resources allocation). The NBI allows an Application Provider to delete their reservation.

TS 28.538 [6] defines the lifecycle management procedure for EAS with ETSI NFV MANO, while the resource reservation related to the ASP requirements is not specified in [6].

It is important to investigate the method for ASP to request the resource reservation.

REQ-resource-reservation-1: ECSP management system shall have a capability allowing ASP to request resource reservation related to virtualisation resources (e.g., compute, networking, storage, acceleration).

REQ-resource-reservation-2: ECSP management system shall have a capability to reserve a set of resources for ASP unrelated to any specific application.

Editor’s note: Resource reservation is FFS

### 6.1.2	Potential solutions

6.1.2.1	Potential solution#1

In this solution, a new IOC called ResourceReservationJob is suggested to be sent to ECSP management system for resource reservation. Figure 6.1.2.1-1 depicts the main steps for ASP asking for resource reservation.

1. ECSP management system receives a resource reservation job creation request (createMOI operation for creation ResourceReservationJob), the ResourceReservationJob contains the following attributes, but not limited to,

…- ocation at which the resource are to be reserved;

- compute resource for reservation;

- storage resource for reservation;

- networking resource for reservation;

- GPU, Hardware accelerators resource for reservation;

…- expiration time

2. ECSP management system sends the ResourceReservationJob creation response to ASP for the received Job DN.

3. ECSP management system creates the ResourceReservationJob instance and configures the attribute from the request and ECSP management system starts the executing the resource reservation process.

4. ECSP management system request resource reservation (See clause 7.3.3 in ETSI GS NFV-IFA 013 [7] by interworking with ETSI NFV MANO.

5. Response with the progress of the requested resource reservation.

6. ECSP management system sends notification of the progress of the requested resource reservation job.

7. ECSP management system sends the final notification with the reservation status.

6.1.2.2	Solution evaluation

This solution introduces a new ResourceReservationJob IOC which can be used by ASP to request resource reservation. ECSP management system needs to support ResourceReservationJob IOC and create a ResourceReservationJob instance based on the request from ASP. The information includes in ResourceReservationJob IOC could be location information, expiration time, reservation statues and virtualized resource information required for reservation, including compute, storage, networking, GPU and Hardware accelerators resource.

6.2.	Key Issue#2: Issue for federation management

### 6.2.1	Description

GSMA requirements related to federation management are divided into E/WBI requirements and Federation Manager Role requirements, which are introduced in clause 5.1.2 and clause 5.2.3 of [2] respectively.

-	Based on the description in clause 5.1.2 of [2]:

The Federation Management functionality within the Operator Platform (OP) enables it to interact with other OP instances, often in different geographies, thereby providing access for the Application Providers to a larger footprint of Edge Clouds, a more extensive set of subscribers and multiple Operator capabilities.

This Key issue is about enabling two OPs establishing federation relationship between each other and sharing available location(s) at which the edge services are provided, resource available at each location, federation expiry etc.

The OP which initiates federation relationship is called leading/originated OP. The OP which receives federation relationship request is called partner OP.

The following functionalities are enabled with established federation relationship:

- Federated EAS resource reservation management: This is intended for an OP to reserve resources for an application provider, with the other OP, when the application provider initiate the reservation using NBI.

- Federated EAS deployment and termination: This will be used by an OP to instantiate an application to edge clouds of other OP as requested by application provider over NBI.

- EDN sharing: This is intended for OPs to share EDN among each other.

Based on the description in clause 5.2.3 of [2]:

GSMA E/WBI API definition [8] provides information on various API and associated parameters and data models to achieve GSMA federation requirements.

Table 6.2.1-1 concludes the Federation Manager Role requirements and related E/WBI APIs, and shows the comparison with 3GPP SA5 management capabilities. The requirement of Settlement is set for charging system and is therefore not shown in table below.

Table 6.2.1-1

| GSMA Federation Manager Role requirement and corresponding API | GSMA E/WBI API | 3GPP management capability | Gap analysis |
| --- | --- | --- | --- |
| Federation and platform interconnection – General: This capability is achieved by the federation E/WBI interface; to interconnect OPs belonging to different operators, enterprises or others. The communication between federated entities shall support a distributed tracking mechanism that allows end-to-end tracking across these federated entities.(see clause 5.2.3.1.1 of [2]) | East/West Bound Interface Management API | MnS for federation requirements is FFS. | GSMA defines East/West Bound Interface Management API (clause 3.1.1 of [8]) and OP data model (clause 3.1.1.7 of [8]) to realize the handshake between two Ops, while 3GPP TR 28.824 [12] has discussed the MnS consumer (i.e. API invoker) registration mechanism by reusing CAPIF. There could be an enhancement in 3GPP on (external) MnS consumer management to catch OP features after evaluating the East/West Bound Interface Management API. |
| Federation and platform interconnection - Authentication/authorisation: Federating OPs are likely to belong to different entities in different security domains. Therefore, the capability to exchange authentication and authorisation between federated OPs is required. (see clause 5.2.3.1.2 of [2]) | No API definition for Authentication/ authorisation | Access control between MnS producer and MnS consumer (TS 28.533 [9]) | GSMA E/WBI API definition [8] doesn’t define API for Authentication/authorisation purpose. 3GPP describes the Access control between MnS producer and MnS consumer in clause 4.9 of TS 28.533 [9]. TR 28.817 [11] and TR 28.824 [12] discussed the access control for (external) MnS consumers, of which the normative work could be used to satisfy this requirement. SA5 may actively coordinate the outcome of access control with GSMA and evaluate whether the work can be reused by GSMA. |

Editor’s Note: Interaction with SA6 is necessary to ensure separation of concerns related with Federation solutions.

### 6.2.2	Requirements

REQ-FUN-Federation-1: ECSP management system shall have a capability to establishing federation relationship with the (external) MnS consumer (e.g. partner operator platforms).

REQ-FUN-Federation-2: ECSP management system shall enable federation relationship to include appropriate information including (not limited to) location(s) at which the edge services are provided, resource available at each location, federation expiry.

### 6.2.3		Potential Solutions

TBD

6.3.	Key Issue#3: EDN Sharing

Once the federation relationship is established, the OPs can share the EDN available in their network with each other. An OP can select some of the EDN from the offered list and register that with the other OP. This will imply that the OP may use the edge resources available in the registered EDN only.

This is crucial to investigate in the method and solutions used for EDN sharing management. It is to be discussed whether it require enhancement to edge NRM and the procedures defined in TS 28.538.

### 6.3.1	Requirements

REQ-FUN-NBI-1: ECSP management system shall enable sharing of available EDN with other ECSP management system.

REQ-FUN-NBI-1: ECSP management system shall disable sharing of available EDN with other ECSP management system.

### 6.3.2	 Potential Solutions

TBD

6.4.	Key Issue#4: Federated EAS resource reservation management

This is intended for an OP to reserve resources for an application provider, with other OP, when the application provider initiate the reservation using NBI. The application provider shall be able to request reservation of resources with partner OP on per location basis. The partner OP shall be able to reserve resources for a given application provider from the allocated quota for the leading OP. The partner OP may have allocated the pool of resources for the leading OP during the EDN registration process. Once the resource reservation request is approved by the partner OP, a resource pool identifier is provided to the leading OP to refer to specific resource pool for application provider. Application provider can use the identifier while instantiating the application to indicate which resource are to be used when deploying applications in partner OP zones.

This is crucial to investigate in the method and solutions used for resource reservation management. It is to be discussed whether it require enhancement to edge NRM and the procedures defined in TS 28.538.

### 6.4.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability enabling ASP to request reservation of resources with other ECSP management system on per EDN basis.

REQ-FUN-NBI-1: ECSP management system shall have a capability enabling ASP to request deletion and updation of the reservation made with other ECSP management system.

### 6.4.2 	Potential Solutions

TBD

6.5.	Key Issue#5: Federated EAS deployment and termination

E/WBI shall control the launch and termination of applications on a partner OP. This will be used by a leading OP to instantiate an application to edge clouds (EDN) of partner OP as requested by application provider over NBI. A leading OP shall make the application instantiation result available on the NBI interface. Partner OP shall also provide the application instance status over E/WBI to leading OP which leading OP may expose to application providers on NBI.

This is crucial to investigate in the method and solutions used for application management on other OPs. It is to be discussed whether it require enhancement to edge NRM and the procedures defined in TS 28.538.

### 6.5.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability enabling ASP to request deployment of EAS on the edge network of other ECSP Management system.

### 6.5.2 	Potential Solutions

TBD

6.6	Key Issue# 6: Virtual Infrastructure Resource Requirements

As described in clause 5.1.1.2.2 GSMA OPG [2], the OP shall support the Infrastructure resource requirements from the Application providers. This includes CPU, memory, storage, hypervisor and networking definition used by the application. In relation to this, an ASP can provide required infrastructure resources for an EAS while requesting to deploy the same.

### 6.6.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability allowing EAS to declare its requirements on the virtual infrastructure.

6.7	Key Issue# 7: Querying Available Edge Services

- As described in Clause 5.1.1.3 of GSMA OPG[2], OP shall provide the following capabilities to the Application Provider: An OP shall offer a range of quality policies so that an Application Provider can choose the performance that their application requires. These policies are defined based on objectively measured end-to-end parameters that include performance aspects of both the network and the Cloudlet, such as latency and packet loss (measured as average statistics).

- The NBI shall enable a request-response mechanism through which the Application Provider can state a geographical point where a typical user could be and get informed of the mean latency performance expected.

- The OP shall describe the capabilities of the Edge Cloud including the geographical zones where it is provided and the type and “granularity” of edge cloud and network service (typically generic Compute, memory, storage, and specialised compute, such as GPU and future resource types).

The above requirements is calling for a mechanism using which the available edge service and their characteristics (e.g. available SLS, infrastructure resources) are described. This information could be anchored at a particular location i.e. edge service and their characteristics available at a particular location.

### 6.7.1	Requirements

REQ-FUN-PLOC-1: ECSP management system shall have a capability allowing ASP to query the available edge services locations and their capabilities including available SLA and infrastructure resources.

### 6.7.2	Potential Solution

The solution requires introduction of the following information in the EdgeDataNetwork IOC.

- Geographical location

- Available compute resources

- Available SLS

The Edge PoP can be queried by ASP to know what edge resources are available in what locations. ASP can then take a informed decision on where he was the EAS to be deployed.

6.8	Key Issue# 8: SLA Requirements Key Issue

As described in clause 5.1.1.2.2 GSMA OPG [2], the OP shall support the SLA requirements from the Application providers. A SLA description allows an Application Provider to describe the physical constraints in an edge network that should be met for the application to run successfully and provide a correct Quality of Experience (QoE) for the end-user at the UE. The SLA requirements include the following properties:

- Bandwidth, bidirectional data rate between UE and backend application.

- Latency, the round trip delay between UE and backend application.

In relation to this, the ASP can declare such requirements for an EAS. The EAS need to be deployed at an EDN which can satisfy these requirements.

### 6.8.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability allowing EAS to declare its SLA requirements, in terms of bandwidth, latency and jitter, for the data connection with the UE.

REQ-FUN-NBI-1: ECSP management system shall be able to select an EDN based on the SLA requirements of an EAS.

### 6.8.2 	Potential Solution

The solution requires adding QoS requirements in EASRequirements IOC. This will enable ASP to provide expected QoS from the edge infrastructure. The following need to be added in the EASREquirements IOC

- Bandwidth: bidirectional data rate between UE and EAS. This will be ascertained in mbps.

- Latency: the round trip delay between UE and EAS. This will be calculated in millisecond.

- Jitter: variance of round-trip delay between UE and backend application

6.9	Key Issue# 9: EAS relocation policies Key Issue

As described in clause 5.2.2.3 of GSMA OPG [2], because of the OP’s measurements or knowledge, or hints from the application about performance degradations, the OP may decide that a different edge compute resource can better host the Edge Application. The EAS relocation trigger from OP are related with lifecycle management of its edge compute resources (for example, the overload of an edge compute resource, a failure or planned maintenance, a new or expanded edge compute resource, an issue with the network for its edge compute resource). The OP should be cognisant of the policy indication from the Application Provider about its sensitivity to a change of the edge compute resource hosting the Edge Application.

As described in clause 5.1.1.2.2 GSMA OPG [2], the NBI allows an Application Provider to specify their support for a stateful or stateless Edge Application, i.e. whether the Edge Application can be moved from one edge compute resource to another and this with or without prior notification.

The Application Provider shall be able to indicate the following policies:

- Its Edge Application cannot be moved from one edge compute resource to another;

- Its Edge Application can be moved from one edge compute resource to another, without any notification;

- Its Edge Application can be moved from one edge compute resource to another with prior notification.

TS 28.538 does not support this yet. This requires an EAS to be moved from one (source) EDN to another (target) EDN. This will involve terminating EAS from the source EDN and then instantiating EAS on the target EDN. This involves the following issues to be addressed

- How the ASP will provide the policies

- How the existing application context is retained from source to target EAS.

### 6.9.1 	Requirements

REQ-FUN-MOB-1: ECSP management system shall have a capability allowing EAS to declare its mobility policies indicating whether EAS can be moved from one EDN to another

REQ-FUN-MOB-2: ECSP management system shall have a capability allowing EAS to declare its mobility policies indicating whether EAS can be moved from one EDN to another, without any notification.

### 6.9.2 	Potential Solution

The solution requires adding required EAS mobility requirements. This will dictate if the EAS can be moved from one EDN to another. If it can be moved, can it be moved without any prior notification. It is proposed to add the following in EASREequirements IOC

- EAS mobility: Boolean attribute stating where EAS can be moved or not.

6.10	Key Issue# 10: Application Resource Catalogue

- As described in Clause 3.5.1.3 of GSMA OPG [2], OP shall provide ASP with application resource catalogue with all the available application and there characteristics. The following requirements relates to application resource catalogue in GSMA. The NBI shall allow applications providers to access the resource catalogue. The Resource catalogue shall consider local resources (CPU, memory, storage).

- The OP shall offer Application Providers a registry to store their application images and update or delete them. The registry may be centralised or distributed, depending upon the Application Provider’s needs to reduce boot time and recovery

TS 28.538 does not address the requirements related with application resource catalogue. An ASP can first query the catalogue to know what all applications are already available and if there is one which can satisfy ASPs requirements.

### 6.10.1 	Requirements

REQ-FUN-PLOC-1: ECSP management system should enable ASP to access the application (EAS) resource catalogue containing the available applications and there characteristics.

### 6.10.2 	Potential Solution

The solution requires a capability where ASP can upload and manage all the artifacts (e.g. image) related with the application with the ECSP/3GPP management system. The uploaded applications can be queried to know the details of available application inventory. All these functionalities can be fulfilled by the VNF Onboarding mechanism defined by ETSI NFV ISG. Hence, it is recommended to refer to the relevant specifications from ETSI NFV ISG to describe a solution of the use case.

7	Conclusion and recommendation

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022.04 | SA5#142e | S5-222103 |  |  |  | Initial version of TR28.903 | 0.0.0 |
| 2022.04 | SA5#142e | S5-222329 S5-222330 | - | - | - | Add scope Add skeleton | 0.1.0 |
| 2022.05 | SA5#143e | S5-223267 | - | - | - | Add background information of ETSI MEC | 0.2.0 |
| 2022.07 | SA5#144e | S5-224438 S5-224439 S5-224440 S5-224441 |  |  |  | adding key issue for collocated management with MEC adding key issue for application lifecycle management adding key issue for resource reservation adding background information for GSMA OPG | 0.3.0 |
| 2022.08 | SA5#145e | S5-225872 S5-225873 S5-225874 |  |  |  | add potential solution for EES and MEP collocation add potential solution for application LCM update background information for GSMA OPG | 0.4.0 |
| 2023.03 | SA5#147 | S5-232860 S5-232861 S5-232862 S5-232863 S5-232864 S5-232865 S5-233046 |  |  |  | E/WBI related Key Issues Add KI for Federation Management NBI Related Key Issues NBI Related Solutions add potential solution for resource reservation GSMA OP and ECM Concept Mapping add potential solution for application LCM with MEC | 0.5.0 |