3GPP TSG-SA5 Meeting #148e	S5-233445rev1

e-meeting, 17-25 April 2023

- Source:	Huawei Technologies

- Title:	pCR TR 28.843 Addition of solution about Independent vertical charging architecture

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Discussion

This is a pCR to TR 28.843 introducing the solution about Independent vertical charging architecture.

# 2	References

[1]	3GPP TR 28.843 "Study on Structure of Charging for Verticals ".

# 3	Rationale

This pCR proposes to introduce the solution about Independent vertical charging architecture in TR 28.843 "Study on Structure of Charging for Verticals".

# 4	Detailed propsosal

Propose to incorporate the following change into the TR 28.843 [1].

First changes

## 7.X	Solution #x: Network slice Tenant charging architecture

### 7.X.1	Solution Description

#### 7.X.1.1	General

This soluion #x resolves the key issues X. Based on the network slice tenant charging, the separated UE converged charging and Network Slice Tenant converged charging are present.

#### 7.X.1.1	Solution

Figure 7.X.1.1-1: NS Tenant charging architecture option

Two types of subscribers are considered in the context of Network Slice:

-	the Tenant the Network Slice is assigned to, which is referred to as "NS-Tenant", which can be considered as the charged party of B2B.

-	the individual end-user "UE" served under the Network Slice, which can be considered as the charged party of B2C.

Each type of subscriber is handled under a separated Converged Charging System (CCS): "Tenant CCS" and "UE CCS" respectively.

"UE CCS" performs charging functionalities for individual UEs (i.e. B2C), and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF hosting individual UE account balance, provisioned with all necessary information related to subscribed-to services the UE has paid for.

- The subscription description includes in particular the subscribed NSSAI (set of S-NSSAI(s)).

"Tenant CCS" performs charging functionalities for NS-Tenant (i.e. B2B), and includes in particular:

-	CHF exposing Charging service(s) associated to NS-Tenant.

-	ABMF hosting NS-Tenant account balance, provisioned with all necessary information related to the Network Slice matching SLS requirements the NS-Tenant has paid for,

-	The subscription description includes in particular the list of service profiles subscribed-to by the NS-Tenant, each service profile with assigned group of S-NSSAI(s).

The Nchf exposed by the CHF (Tenant CCS) is defined for enabling two cases:

-	Association between individual UE charging and NS tenant charging. This association is achieved by UE CCS responsible for individual UE charging consuming the Nchf exposed by the Tenant CCS. The charging for individual UE charging may be impacted by the charging for the NS tenant charging.

-	Charging for the NS Tenant charging only.

Editor’s Note: The basically deployment options described in the current documentation for Vertical (e.g. B2B) charging should clarified, such the NF to both B2B and B2C CHFs.

End of changes