- 3GPP TSG SA WG5 Meeting #148e	S5-233488rev3

- Electronic, 17-25 April 2023 	revision of S5-20xabc

- Source:	CATT

- Title:	pCR Add charging scenarios and key issues for satellite backhaul on converged charging for data connection

- Document for:	Approval

Agenda Item:	7.5.6

# 1	Discussion

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.844: "Study on charging aspects of satellite in 5GS".

# 3	Rationale

This pCR proposes to introduce charging modes to TR 28.844 "Study on charging aspects of satellite in 5GS".

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.844[1].

1st change

# 5	Charging scenarios and key issues for SNPN

## 5.X	Topic X: Converged charging for data connectivity in satellite backhaul

### 5.X.1	General description and assumptions

In the satellite backhaul scenario, multiple satellite backhaul categories referring to the type of the satellites have been introduced in clause 4.X.

Based on the satellite backhaul architecture depicted in clause 4.X, if the UE accesses a gNB that uses satellite backhaul, the charging party and charged party can be:

Charged party: Satellite service customer access a gNB which is using satellite backhaul link to CN.

Charging party: MNO who provides the satellite communication service to the Satellite service customer.

If the MNO needs to lease the satellite from other company, the charging party and charged party can be:

Charged party: MNO who using the satellite from the Satellite Service Provider

Charging party: Satellite Service Provider who provides the satellite service to MNO

The potential charging requirements for this Use Case is: REQ-CH_ SATBH_DC-01.

5.X.2	Potential charging requirements

The following are potential high-level charging requirements for satellite in 5GS, derived from the requirements in TS 23.501 [2], TS 23.502 [3] and TS 23.503[5].

REQ-CH_ SATBH_DC-01: The 5GS should support collecting charging information for satellite backhaul service.

5.X.3	Key issues

#### 5.X.3.1	Key issue #X.1: Charging events and charging information required

This key issue is for investigating how to support the satellite backhaul service charging considering REQ-CH_ SATBH_DC-01. This investigation covers the following:

-	identification of the triggers for charging events for satellite backhaul;

-	identification and classification of the charging information for satellite backhaul;

5.X.3.2	Key issue #X.2:NF/Service suitable to provide charging information

This key issue is for investigating how to support the satellite backhaul service charging considering REQ-CH_ SATBH_DC-01, REQ-CH_ SATBH_DC-02 and REQ-CH_ SATBH_DC-03. This investigation covers the following:

-	determination of which NF in the 5G system are suitable to provide the charging information to support the satellite backhaul;

End of changes