- 3GPP TSG-SA5 Meeting #148-e	S5-233358

- e-meeting, 17-25 April 2023

- Source:	Huawei

- Title:	pCR 28.903 add conclusion and recommendation for SLA related issue

- Document for:	Approval

- Agenda Item:	6.9.6.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.903-050 “Study on alignment with ETSI MEC for Edge computing management”

# 3	Rationale

This pCR suggests to update description of key issue#8: SLA Requirements Key Issue and provide conclusion and recommendation.

# 4	Detailed proposal

It is proposed to update description of key issue#8: SLA Requirements Key Issue and provide conclusion and recommendation in TR 28.903-050 [1].

1st change

- 6.8	Key Issue# 8: SLA Requirements Key Issue

As described in clause 5.1.1.2.2 GSMA OPG [2], the OP shall support the SLA requirements from the Application providers. A SLA description allows an Application Provider to describe the physical constraints in an edge network that should be met for the application to run successfully and provide a correct Quality of Experience (QoE) for the end-user at the UE. The SLA requirements include the following properties:

- Bandwidth, bidirectional data rate between UE and backend application.

- Latency, the round trip delay between UE and backend application.

In relation to this, the ASP can declare such requirements for an EAS. The EAS need to be deployed at an EDN which can satisfy these requirements.

- 6.8.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability allowing EAS to declare its SLA requirements, in terms of bandwidth and latency, for the data connection with the UE.

REQ-FUN-NBI-1: ECSP management system shall be able to select an EDN based on the SLA requirements of an EAS.

- 6.8.2 	Potential Solution

The solution requires adding SLA requirements in EASRequirements IOC. This will enable ASP to provide expected SLA from the edge infrastructure. The following need to be added in the EASREquirements IOC

- Bandwidth: bidirectional (in and out) data rate between UE and EAS.

- Latency: the round trip delay between UE and EAS.

2nd change

- 7	Conclusion and recommendation

## 7.x	Conclusion and recommendation for alignement with GSMA OPG

- 7.x.x	Issue#8: SLA Requirements key issue

It is recommended to introduce the Bandwidth and Latency in the EASRequirements IOC for supporting SLA requirements when deploying a EAS. New performance measurements and KPIs related to Bandwidth and Latency between UE and EAS may need to be added.

The detailed solution see clause 6.8.2.

End of change