TM Forum <<Document Type>>

TR294B Model Connection to 3GPP TS 28.541 - Intent Extension Model v1.0.0 (Draft)

[Subtitle]

<<document number>>

| Maturity Level: Alpha, Beta, General Availability (GA) | Team Approved Date: DD-MMM-YYYY |
| --- | --- |
| Release Status: Pre-production | Approval Status: Team Approved |
| Version [add version number] | IPR Mode: RAND |

# Notice

Copyright © TM Forum 2023. All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published, and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this section are included on all such copies and derivative works. However, this document itself may not be modified in any way, including by removing the copyright notice or references to TM FORUM, except as needed for the purpose of developing any document or deliverable produced by a TM FORUM Collaboration Project Team (in which case the rules applicable to copyrights, as set forth in the TM FORUM IPR Policy, must be followed) or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by TM FORUM or its successors or assigns.

This document and the information contained herein is provided on an “AS IS” basis and TM FORUM DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY OWNERSHIP RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Direct inquiries to the TM Forum office:

181 New Road, Suite 304

Parsippany, NJ 07054, USA

Tel No.  +1 862 227 1648

TM Forum Web Page: www.tmforum.org

# Table of Contents

Notice	3

Table of Contents	4

1.	Executive Summary	4

2.	Introduction	4

3.	Motivation and Background	5

4.	Notation and Namespaces	6

5.	Vocabulary Specification	7

5.1.	Individuals	7

5.2.	Classes	7

5.3.	Properties	11

6.	Model Usage and Examples	12

| - Maturity Level: | - Team Approved Date: dd-Mmm-yyyy |
| --- | --- |
| - Release Status: | - Approval Status: |
| - Version | - IPR Mode: |

# Executive Summary

3GPP TS 28.541 is an intent extension model that introduces vocabulary for using the artifacts that are defined in the 3GPP technical specification 28.541.

# Introduction

This document is one in a suite of intent extension models that are specific to 3GPP. In particular, it defines the artifacts used to specify different types of network resources in 5G. Some of these artifacts can be used for creating radio network (RAN) and service intents as defined in the model TR294A.

Figure 0.1: Relationship of TR294B with other models from TM Forum

# Motivation and Background

TR293 describes the connector model for federation of non-RDF based models that are defined by different SDOs. Using this model, it is possible to create intent extension models that are specific to relevant domains or use cases. One such non-RDF based model is defined in 3GPP technical specification 28.541, i.e., 5G Network Resource Model (NRM); Stage 2 and Stage 3. This model contains definitions of different types of network resources in 5G. This includes RAN, Core, as well as network slicing in 5G. Some of the definitions in this model pertain to the artifacts needed for creation of RAN and service intents as defined in TR294A. However, since this model is not based on RDF, hence TR293 needs to be used for doing model federation and creating these intents in RDF format.

This intent extension model facilitates the creation of RAN and service intents by defining different artifacts that are specified in 3GPP TS 28.541. All the formalities required for model connection are done as part of this model, and anyone who wishes to create a RAN or service intent can just directly use the classes, individuals, and properties defined in this model by making a reference to its IRI.

Note that this is an evolving document, and as of now it does not define all the artifacts included in the 3GPP specification. But instead, it only includes the definitions of artifacts that are deemed necessary for creating RAN and service intents. The definitions of further artifacts will be added later on in this document if needed in some other use cases.

# Notation and Namespaces

The intent extension model for 3GPP TS 28.541 is defined in a namespace under the TM Forum domain. It depends on the following models and uses the respective namespaces:

| - Model | - Prefix | - Namespace | - Published by |
| --- | --- | --- | --- |
| - 3GPP TS 28.541 - Intent Extension Model | - 3gpp_28541 | - http://tio.models.tmforum.org/tio/v1.0.0/3GPP-TS-28541/ | - TM Forum |
| - Connector Model | - conn | - http://tio.models.tmforum.org/tio/v1.0.0/ConnectorModel/ | - TM Forum |
| - Intent Common Model | - icm | - http://tio.models.tmforum.org/tio/v2.0.0/IntentCommonModel/ | - TM Forum |
| - W3C RDF Schema 1.1 | - rdfs | - http://www.w3.org/2000/01/rdf-schema# | - W3C |
| - XML Schema | - xsd | - http://www.w3.org/2001/XMLSchema# | - W3C |

The proposed prefix label for 3GPP TS 28.541 intent extension model is "3gpp_28541".

This model uses connector model for referring to the artifacts defined in the specification document.

The Intent Common Model (ICM) and Resource Description Framework Schema (RDFS) provide the base model stack for defining this model. For example, numerical quantities are derived from the definition of "Quantity" in ICM, and classes are derived from the definition of "Class" in RDFS.

This model uses "string", "integer", and "float" datatypes that are defined in XML Schema.

# Vocabulary Specification

This section describes all the classes and properties that are specified in the intent extension model for 3GPP TS 28.541. Moreover, the class individuals or instances used in the model are also described.

## Individuals

The model defines individual of class conn:ModelConnector which is defined in the TR293 connector model. This individual contains all the details about the 3GPP TS 28.541 model.

| - Individual of Class conn:ModelConnector: | - 3gpp_28541:ModelConn_3GPP28541 |
| --- | --- |
| - Description: | - This describes the details about 3GPP TS 28.541 model, e.g., publisher, label, version, description etc. |

## Classes

The model contains the following classes:

| - Class: | - 3gpp_28541:CoverageArea |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:CoverageArea specifies the coverage area of a network slice, i.e., the geographic region where a 3GPP communication service is accessible. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - coverageArea |

| - Class: | - 3gpp_28541:NRTAC |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:NRTAC holds the identity of the common Tracking Area Code (TAC) for the Public Land Mobile Networks (PLMNs). |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - nRTAC |

| - Class: | - 3gpp_28541:PLMNId |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:PLMNId holds the identity of a PLMN which is a combination of Mobile Country Code (MCC) and Mobile Network Code (MNC). |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - pLMNId |

| - Class: | - 3gpp_28541:CoverageAreaTAList |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:CoverageAreaTAList specifies the list of Tracking Areas for a network slice. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - coverageAreaTAList |

| - Class: | - 3gpp_28541:DLThptPerUE |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:DLThptPerUE specifies the downlink data rate supported by a network slice per UE. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | conn:ModelElementConnector - 3gpp_28541:XLThpt |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - dLThptPerUE |

| - Class: | - 3gpp_28541:ULThptPerUE |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:ULThptPerUE specifies the uplink data rate supported by a network slice per UE. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | conn:ModelElementConnector - 3gpp_28541:XLThpt |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - uLThptPerUE |

| - Class: | - 3gpp_28541:XLThpt |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:XLThpt specifies the uplink or downlink data rate (guaranteed or maximum or both). |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - XLThpt |

| - Class: | - 3gpp_28541:DLLatency |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:DLLatency specifies the required downlink packet transmission latency (in milliseconds) through the RAN, Core Network (CN), and Transport Network (TN) part of the 5G network, and is used to evaluate utilization performance of the end-to-end network slice. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | conn:ModelElementConnector - icm:Quantity |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - dLLatency |

| - Class: | - 3gpp_28541:ULLatency |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:ULLatency specifies the required uplink packet transmission latency (in milliseconds) through the RAN, Core Network (CN), and Transport Network (TN) part of the 5G network, and is used to evaluate utilization performance of the end-to-end network slice. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | conn:ModelElementConnector - icm:Quantity |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - uLLatency |

| - Class: | - 3gpp_28541:MaxNumberofUEs |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:MaxNumberofUEs specifies the maximum number of concurrent UEs supported by a network slice. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - maxNumberofUEs |

| - Class: | - 3gpp_28541:ActivityFactor |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:ActivityFactor specifies the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs that are exchanging data with the network. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | conn:ModelElementConnector - icm:Quantity |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - activityFactor |

| - Class: | - 3gpp_28541:UESpeed |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:UESpeed specifies the maximum speed (in km/hour) supported by a network slice at which a defined QoS can be achieved. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | conn:ModelElementConnector - icm:Quantity |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - uESpeed |

| - Class: | - 3gpp_28541:UEMobilityLevel |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:UEMobilityLevel specifies the mobility level of UEs accessing a network slice. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - uEMobilityLevel |

| - Class: | - 3gpp_28541:ResourceSharingLevel |
| --- | --- |
| - Definition: | - An instance of class 3gpp_28541:ResourceSharingLevel specifies whether the resources to be allocated to a network slice may be shared with another network slice. |
| - Instance of: | - rdfs:Class |
| - Subclass of: | - conn:ModelElementConnector |
| - In Model: | - 3gpp_28541:ModelConn_3GPP28541 |
| - Model Element Label: | - resourceSharingLevel |

## Properties

The model contains the following properties:

| - Property: | - 3gpp_28541:coverageAreaName |
| --- | --- |
| - Definition: | - This property describes the name of a coverage area. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:CoverageArea |
| - Range: | - xsd:string |

| - Property: | - 3gpp_28541:nRTACValue |
| --- | --- |
| - Definition: | - This property describes the value of common TAC for the PLMNs. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:NRTAC |
| - Range: | - xsd:integer |

| - Property: | - 3gpp_28541:pLMNIdString |
| --- | --- |
| - Definition: | - This property gives the string representation of the identity of a PLMN. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:PLMNId |
| - Range: | - xsd:string |

| - Property: | - 3gpp_28541:coverageAreaTAListEntry |
| --- | --- |
| - Definition: | - This property specifies an entry in the CoverageAreaTAList. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:CoverageAreaTAList |
| - Range: | - xsd:integer |

| - Property: | - 3gpp_28541:guaThpt |
| --- | --- |
| - Definition: | - This property describes the guaranteed data rate. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:XLThpt |
| - Range: | - xsd:float |

| - Property: | - 3gpp_28541:maxThpt |
| --- | --- |
| - Definition: | - This property describes the maximum data rate. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:XLThpt |
| - Range: | - xsd:float |

| - Property: | - 3gpp_28541:3GPPNoOfUEs |
| --- | --- |
| - Definition: | - This property specifies the maximum number of UEs for 3GPP access. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:MaxNumberofUEs |
| - Range: | - xsd:integer |

| - Property: | - 3gpp_28541:non3GPPNoOfUEs |
| --- | --- |
| - Definition: | - This property specifies the maximum number of UEs for non-3GPP access. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:MaxNumberofUEs |
| - Range: | - xsd:integer |

| - Property: | - 3gpp_28541:uEMobilityValue |
| --- | --- |
| - Definition: | - This property specifies the value of mobility for UEs. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:UEMobilityLevel |
| - Range: | - xsd:string |

| - Property: | - 3gpp_28541:resourceSharingLevel |
| --- | --- |
| - Definition: | - This property specifies the value of resource sharing allowed for a network slice. |
| - Instance of: | - rdf:Property |
| - Domain: | - 3gpp_28541:ResourceSharingLevel |
| - Range: | - xsd:string |

# Model Usage and Examples

Example 1: Specifying a RAN intent with all the relevant parameters defined in 3GPP TS 28.312 and TS 28.541 specification documents

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> . @prefix icm: <http://tio.models.tmforum.org/tio/v2.2.0/IntentCommonModel#> . @prefix iv: <http://tio.models.tmforum.org/tio/v1.1.0/IntentValidity/> . @prefix xsd: <http://www.w3.org/2001/XMLSchema#> . @prefix 3gpp_28312: <http://tio.models.tmforum.org/tio/v1.0.0/3GPP-TS-28312#> . @prefix 3gpp_28541: <http://tio.models.tmforum.org/tio/v1.0.0/3GPP-TS-28541#> . # --- Intent -------------------- :RAN_SubNet_Intent a icm:Intent ; rdfs:comment "Intent for creation of RAN SubNetwork" ; icm:hasExpectation :propertyExpectation ; . # --- Class instances -------------------- :CoverageAreaPolygon1 a 3gpp_28312:CoverageAreaPolygon ; rdfs:member :CoverageArea1 ; rdfs:member :CoverageArea2 ; . :CoverageArea1 a 3gpp_28541:CoverageArea ; 3gpp_28541:coverageAreaName "City X South" ; . :CoverageArea2 a 3gpp_28541:CoverageArea ; 3gpp_28541:coverageAreaName "City Y East" ; . :CoverageAreaTAC1 a 3gpp_28312:CoverageAreaTAC ; rdfs:member :NRTAC1 ; rdfs:member :NRTAC2 ; . :NRTAC1 a 3gpp_28541:NRTAC ; 3gpp_28541:nRTACValue 1234 ; . :NRTAC2 a 3gpp_28541:NRTAC ; 3gpp_28541:nRTACValue 5678 ; . :PLMN1 a 3gpp_28312:PLMN ; rdfs:member :PLMNId1 ; rdfs:member :PLMNId2 ; . :PLMNId1 a 3gpp_28541:PLMNId ; 3gpp_28541:pLMNIdString "001-002" ; . :PLMNId2 a 3gpp_28541:PLMNId ; 3gpp_28541:pLMNIdString "001-007" ; . :NRFqBand1 a 3gpp_28312:NRFqBand ; rdfs:member "n1" ; rdfs:member "n7" ; rdfs:member "n98" ; . :RAT1 a 3gpp_28312:RAT ; rdfs:member "UTRAN" ; rdfs:member "EUTRAN" ; rdfs:member "NR" ; . # --- Target -------------------- :TargetResource a icm:IndividualTarget ; icm:intersection ( [ icm:elementsCondition ( [ icm:elementType 3gpp_28312:RANSubNetwork ] icm:hasProperty :CoverageAreaPolygon1 ) ] [ icm:elementsCondition ( [ icm:elementType 3gpp_28312:RANSubNetwork ] icm:hasProperty :CoverageAreaTAC1 ) ] [ icm:elementsCondition ( [ icm:elementType 3gpp_28312:RANSubNetwork ] icm:hasProperty :PLMN1 ) ] [ icm:elementsCondition ( [ icm:elementType 3gpp_28312:RANSubNetwork ] icm:hasProperty :NRFqBand1 ) ] [ icm:elementsCondition ( [ icm:elementType 3gpp_28312:RANSubNetwork ] icm:hasProperty :RAT1 ) ] ) ; . # --- Expectation -------------------- :propertyExpectation a icm:PropertyExpectation ; icm:target :TargetResource ; icm:allOf [ rdfs:member :weakRSRPRatioProperty ; rdfs:member :lowSINRRatioProperty ; rdfs:member :aveULRANUEThptProperty ; rdfs:member :aveDLRANUEThptProperty ; rdfs:member :lowULRANUEThptRatioProperty ; rdfs:member :lowDLRANUEThptRatioProperty ; ] ; . # --- Properties -------------------- :weakRSRPRatioProperty a icm:PropertyParameter ; iv:hasValidity :weakRSRPValidity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:WeakRSRPRatio ] ] [ icm:value "60"^^xsd:integer ] ) ; . :weakRSRPValidity a iv:Validity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:WeakRSRPThreshold ] ] [ icm:value "70.8"^^xsd:float ] ) ; . :lowSINRRatioProperty a icm:PropertyParameter ; iv:hasValidity :lowSINRValidity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:LowSINRRatio ] ] [ icm:value "75"^^xsd:integer ] ) ; . :lowSINRValidity a iv:Validity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:LowSINRThreshold ] ] [ icm:value "80"^^xsd:integer ] ) ; . :aveULRANUEThptProperty a icm:PropertyParameter ; icm:atLeast ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:AveULRANUEThpt ] ] [ icm:value "205"^^xsd:integer ] ) ; . :aveDLRANUEThptProperty a icm:PropertyParameter ; icm:atLeast ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:AveDLRANUEThpt ] ] [ icm:value "380"^^xsd:integer ] ) ; . :lowULRANUEThptRatioProperty a icm:PropertyParameter ; iv:hasValidity :lowULRANUEThptValidity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:LowULRANUEThptRatio ] ] [ icm:value "40"^^xsd:integer ] ) ; . :lowULRANUEThptValidity a iv:Validity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:LowULRANUEThptThreshold ] ] [ icm:value "120.7"^^xsd:float ] ) ; . :lowDLRANUEThptRatioProperty a icm:PropertyParameter ; iv:hasValidity :lowDLRANUEThptValidity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:LowDLRANUEThptRatio ] ] [ icm:value "55"^^xsd:integer ] ) ; . :lowDLRANUEThptValidity a iv:Validity ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:LowDLRANUEThptThreshold ] ] [ icm:value "225.4"^^xsd:float ] ) ; .

Example 1 shows an example of RAN intent with all the relevant parameters (including optional ones). All these parameters are defined in intent extension model for 3GPP TS 28.312, however some parameters are dependent on the ones defined in intent extension model for 3GPP TS 28.541. Hence, in this document, an example is given that shows how both models relate together and help in defining a complete RAN intent. At the same time, it also shows the usage of the model 3GPP TS 28.541, i.e., the topic of this section.

For the sake of readability, this intent example is divided into different sections which are explained in more detail in Section 5 of TR294A (intent extension model for 3GPP TS 28.312). It is also worth noting here that the above example does not have any managed object connector meaning that it does not use already existing artifacts stored in some other managed system. Instead, the values of all artifacts are locally defined in the intent itself.

Example 2: Specifying a service intent with all the relevant parameters defined in 3GPP TS 28.312 and TS 28.541 specification documents

@prefix rdfs: <http://www.w3.org/2000/01/rdf-schema#> . @prefix icm: <http://tio.models.tmforum.org/tio/v2.2.0/IntentCommonModel#> . @prefix iv: <http://tio.models.tmforum.org/tio/v1.1.0/IntentValidity/> . @prefix itv: <http://tio.models.tmforum.org/tio/v1.1.0/IntentTemporalValidity/> . @prefix xsd: <http://www.w3.org/2001/XMLSchema#> . @prefix geo: <http://www.w3.org/2003/01/geo/wgs84_pos#> . @prefix 3gpp_28312: <http://tio.models.tmforum.org/tio/v1.0.0/3GPP-TS-28312#> . @prefix 3gpp_28541: <http://tio.models.tmforum.org/tio/v1.0.0/3GPP-TS-28541#> . # --- Intent -------------------- :Service_Intent a icm:Intent ; rdfs:comment "Intent for creation of service instance" ; icm:hasExpectation :propertyExpectation ; iv:hasValidity :serviceStartTimeValidity , :serviceEndTimeValidity , :ueMobilityLevelValidity , :resourceSharingLevelValidity ; . # --- Intent Validities -------------------- :serviceStartTimeValidity a itv:TemporalValidity ; t:after [ a 3gpp_28312:ServiceStartTime ; t:inXSDDateTimeStamp 2022-12-01T09:00:00+01:00 ; ] ; . :serviceEndTimeValidity a itv:TemporalValidity ; t:before [ a 3gpp_28312:ServiceEndTime ; t:inXSDDateTimeStamp 2022-12-01T17:00:00+01:00 ; ] ; . :ueMobilityLevelValidity a itv:Validity ; icm:subjectString [ icm:latestValueOf 3gpp_28312:UEMobilityLevel ] ; icm:stringMatches "stationary" ; . :resourceSharingLevelValidity a itv:Validity ; icm:subjectString [ icm:latestValueOf 3gpp_28312:ResourceSharingLevel ] ; icm:stringMatches "non-shared" ; . # --- Class instances -------------------- :EdgeIdentificationLoc1 # assuming that edge data network id is not known a 3gpp_28312:EdgeIdentificationLoc ; geo:lat "55.701" ; geo:long "12.552" ; . :CoverageAreaTA1 a 3gpp_28312:CoverageAreaTA ; 3gpp_28541:coverageAreaTAListEntry 1234 ; 3gpp_28541:coverageAreaTAListEntry 5678 ; . # --- Target -------------------- :TargetResource a icm:IndividualTarget ; icm:intersection ( [ icm:elementsCondition ( [ icm:elementType 3gpp_28312:RANSubNetwork ] icm:hasProperty :EdgeIdentificationLoc1 ) ] [ icm:elementsCondition ( [ icm:elementType 3gpp_28312:RANSubNetwork ] icm:hasProperty :CoverageAreaTA1 ) ] ) ; . # --- Expectation -------------------- :propertyExpectation a icm:PropertyExpectation ; icm:target :TargetResource ; icm:allOf [ rdfs:member :dLThptperUEProperty ; rdfs:member :uLThptperUEProperty ; rdfs:member :dLLatencyProperty ; rdfs:member :uLLatencyProperty ; rdfs:member :maxNumberofUEsProperty ; rdfs:member :activityFactorContextProperty ; rdfs:member :uESpeedContextProperty ; ] ; . # --- Properties -------------------- :dLThptperUEProperty a icm:PropertyParameter ; icm:atLeast ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:DLThptperUE ] ] [ icm:value "140.7"^^xsd:float ] ) ; . :uLThptperUEProperty a icm:PropertyParameter ; icm:atLeast ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:ULThptperUE ] ] [ icm:value "82.5"^^xsd:float ] ) ; . :dLLatencyProperty a icm:PropertyParameter ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:DLLatency ] ] [ icm:value "10"^^xsd:integer ; icm:unit80000 "ms" ; ] ) ; . :uLLatencyProperty a icm:PropertyParameter ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:ULLatency ] ] [ icm:value "14"^^xsd:integer ; icm:unit80000 "ms" ; ] ) ; . :maxNumberofUEsProperty a icm:PropertyParameter ; icm:atMost ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:MaxNumberofUEs ] ] [ icm:value "1000"^^xsd:integer ] ) ; . :activityFactorContextProperty a icm:PropertyParameter ; icm:exactly ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:ActivityFactorContext ] ] [ icm:value "0.4"^^xsd:float ] ) ; . :uESpeedContextProperty a icm:PropertyParameter ; icm:exactly ( [ icm:latestValueOf [ icm:valueSeriesOf 3gpp_28312:UESpeedContext ] ] [ icm:value "100"^^xsd:integer ; icm:unit80000 "km/h" ; ] ) ; .

Example 2 shows an example of service intent with all the relevant parameters (including optional ones), i.e., similar to the example of RAN intent. Details about different sections of this intent can be found in Section 5 of TR294A (intent extension model for 3GPP TS 28.312).