3GPP TSG-SA5 Meeting #148e 	S5-233301

Electronic meeting, Online, 17 -25 April 2023

# Source:	Ericsson LM

# Title:	DP Use of AMF charging profile

# Document for:	Discussion

Agenda Item:	7.3

# 1	Decision/action requested

Agree on a way forward for AMF charging profile usage.

# 2	References

[1]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2".

# 3	Rationale

## 3.1	Background

In TS 32.256 [1] clause 5.2.1.2.2 it is stated: “An AMF charging profile is defined to allow AMF determination of applicable trigger conditions for each functionality (i.e Registration, N2 connection, Location Reporting)”

In TS 32.256 [1] clause 5.2.1.2.2 it is stated: “The AMF charging profile is per UE and per AMF.”

In TS 32.256 [1] clause 5.2.1.2.2 it is stated: “The default AMF charging profile is provided by the Charging Characteristics which is described in Annex A.”

In TS 32.256 [1] Annex A it is stated: The Charging Characteristics parameter consists of a string of 16 bits designated as Behaviours (B), freely defined by Operators, as shown in TS 32.298 [51]. Each bit corresponds to an index to specific charging behaviour which is configured on a per operator basis and active when the bit is set to "1" value.

The following Table A.1.1 provides an example of Charging Characteristics for AMF:

Table A.1.1: Example of Charging Characteristics behaviours for AMF

|  |  | Registration charging |  | N2 connections charging |  | Location reporting charging |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Behaviour Index | Primary and Secondary CHF addresses | Active | Charging scenario | Active | Charging scenario | Active | Charging scenario | Level |
| 0 | URI 1 URI 2 | Yes | ECUR | No | - | No | - |  |
| 1 | URI 1 URI 2 | Yes | IEC | Yes | PEC | Yes | PEC | TAI, Cell |
| 2 | URI 1 URI 2 | Yes | PEC | No | - | Yes | PEC | Cell |
| 3 | URI 1 URI 2 | No | - | Yes | PEC | Yes | PEC | TAI |
| … | … | … | … | … | … | … | … | … |

“

In TS 32.256 [1] clause 6.1.1.2:

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] In case Subscriber Identifier (i.e. SUPI) is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Charging Identifier | - | This field is not applicable. |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57] |
| One-time Event Type | OC | Described in TS 32.290 [57] |
| Notify URI | - | This field is not applicable. |
| Service Specification Information | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G connection and mobility specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57]. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| AMF Charging Profile | OC | This field holds the applicable AMF Charging Profile |
| Registration Charging Information | OM | This field holds the registration specific information described in clause 6.2.1.2 |
| N2 connection Charging Information | OM | This field holds the N2 connection specific information described in clause 6.2.1.3 |
| Location Reporting Charging information | OM | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

In TS 32.256 [1] clause 6.1.1.3:

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | This field is not applicable. |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G connection and mobility specific triggers described in clause 5.2.1. |
| Multiple Unit Information | OC | This field is applicable for ECUR. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | OM | Described in TS 32.290 [57] |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |
| AMF Charging Profile | OC | This field holds the AMF Charging Profile to be applied |
| Location Reporting Charging information | Oc | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

In TS 32.291 [1] clause 7.4:

Table 7.4-1: Bindings of 5G 5G connection and mobility CDR field, Information Element and Resource Attribute

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
|  |  | ChargingDataRequest |
| Registration Charging Information | Registration Charging Information | /registrationChargingInformation |
| Registration message type | Registration message type | /registrationChargingInformation/registrationMessagetype |
| User Information | User Information | /registrationChargingInformation/userInformation |
| User Identifier | User Identifier | /registrationChargingInformation/userInformation/servedGPSI |
| User Equipment Info | User Equipment Info | /registrationChargingInformation/userInformation/servedPEI |
| unauthenticatedFlag | unauthenticatedFlag | /registrationChargingInformation/userInformation/unauthenticatedFlag |
| Roamer In Out | Roamer In Out | /registrationChargingInformation/userInformation/roamerInOut |
| User Location Information | User Location Information | /registrationChargingInformation/userLocationinfo |
| PSCell Information | PSCell Information | /registrationChargingInformation/pSCellInformation |
| UE Time Zone | UE Time Zone | /registrationChargingInformation/uetimeZone |
| RAT Type | RAT Type | /registrationChargingInformation/rATType |
| 5GMM Capability | 5GMM Capability | /registrationChargingInformation/5gMMCapability |
| MICO Mode Indication | MICO Mode Indication | /registrationChargingInformation/mICOModeIndication |
| SMS Supported Indication | SMS Supported Indication | /registrationChargingInformation/smsIndication |
| TAI List | TAI List | /registrationChargingInformation/taiList |
| Service Area Restrictions | Service Area Restrictions | /registrationChargingInformation/serviceAreaRestriction |
| Requested NSSAI | Requested NSSAI | /registrationChargingInformation/requestedNSSAI |
| Allowed NSSAI | Allowed NSSAI | /registrationChargingInformation/allowedNssai |
| Rejected NSSAI | Rejected NSSAI | /registrationChargingInformation/rejectedNSSAI |
| NSSAI mapping list | NSSAI mapping list | /registrationChargingInformation/nSSAIMapList |
| AMF UE NGAP ID | AMF UE NGAP ID | /registrationChargingInformation/amfUeNgapId |
| RAN UE NGAP ID | RAN UE NGAP ID | /registrationChargingInformation/ranUeNgapId |
| RAN Node Id | RAN Node Id | /registrationChargingInformation/ranNodeId |
| N2 Connection Charging Information | N2 Connection Charging Information | /n2ConnectionChargingInformation |
| N2 Connection message type | N2 Connection message type | /n2ConnectionChargingInformation/n2ConnectionMessageType |
| User Information | User Information | /registrationChargingInformation/userInformation |
| User Identifier | User Identifier | /n2ConnectionChargingInformation/userInformation/servedGPSI |
| User Equipment Info | User Equipment Info | /n2ConnectionChargingInformation/userInformation/servedPEI |
| unauthenticatedFlag | unauthenticatedFlag | /n2ConnectionChargingInformation/userInformation/unauthenticatedFlag |
| Roamer In Out | Roamer In Out | /n2ConnectionChargingInformation/userInformation/roamerInOut |
| User Location Information | User Location Information | /n2ConnectionChargingInformation/userLocationinfo |
| PSCell Information | PSCell Information | /n2ConnectionChargingInformation/pSCellInformation |
| UE Time Zone | UE Time Zone | /n2ConnectionChargingInformation/uetimeZone |
| RAT Type | RAT Type | /n2ConnectionChargingInformation/rATType |
| AMF UE NGAP ID | AMF UE NGAP ID | /n2ConnectionChargingInformation/amfUeNgapId |
| RAN UE NGAP ID | RAN UE NGAP ID | /n2ConnectionChargingInformation/ranUeNgapId |
| RAN Node Id | RAN Node Id | /n2ConnectionChargingInformation/ranNodeId |
| Mobility Restrictions | Mobility Restrictions | /n2ConnectionChargingInformation/restrictedRatList /n2ConnectionChargingInformation/forbiddenAreaList /n2ConnectionChargingInformation/serviceAreaRestriction /n2ConnectionChargingInformation/restrictedCnList |
| Allowed NSSAI | Allowed NSSAI | /n2ConnectionChargingInformation/allowedNssai |
| RRC Establishment Cause | RRC Establishment Cause | /n2ConnectionChargingInformation/rrcEstCause |
| Location Reporting Charging Information | Location Reporting Charging Information | /locationReportingChargingInformation |
| N2 Connection message type | N2 Connection message type | /locationReportingChargingInformation/n2ConnectionMessageType |
| User Information | User Information | /locationReportingChargingInformation/userInformation |
| User Identifier | User Identifier | /locationReportingChargingInformation/userInformation/servedGPSI |
| User Equipment Info | User Equipment Info | /locationReportingChargingInformation/userInformation/servedPEI |
| unauthenticatedFlag | unauthenticatedFlag | /locationReportingChargingInformation/userInformation/unauthenticatedFlag |
| Roamer In Out | Roamer In Out | /locationReportingChargingInformation/userInformation/roamerInOut |
| User Location Information | User Location Information | /locationReportingChargingInformation/userLocationinfo |
| PSCell Information | PSCell Information | locationReportingChargingInformation/pSCellInformation |
| UE Time Zone | UE Time Zone | /locationReportingChargingInformation/uetimeZone |
| Presence Reporting Area Information | Presence Reporting Area Information | /locationReportingChargingInformation/presenceReportingAreaInformation |
| RAT Type | RAT Type | /locationReportingChargingInformation/rATType |
|  |  | ChargingDataResponse |
| Supported Features | - | /supportedFeatures- |
| Location Reporting Charging Information | - | /locationReportingChargingInformation |
| Location reporting message type | - | /locationReportingChargingInformation/locationReportingMessageType |
| Presence Reporting Area Information | - | /locationReportingChargingInformation /presenceReportingAreaInformation |

## 3.2 Evaluation

The use of the AMF charging profile in the current TS 32.256 [1] is unclear, and it cannot be mapped to any attribute in either Nchf or CDR. This means that it cannot be used in the current specification, and since it is linked to charging characteristics according to TS 32.256 [1] clause 5.2.1.2.2 even if there is no mention of it in annex A, it would be better to use the charging characteristics directly instead of going through the AMF charging profile.

## 3.3 Conclusion

Remove the AMF charging profile and describe the use of charging characteristics better.

# 4	Detailed proposal

Update TS 32.256 [1] according to the recommendation in clause 3.3.