3GPP TSG-SA5 Meeting #148e 	S5-233326

Electronic meeting, Online, 17 -25 April 2023

# Source:	Ericsson, Deutsche Telekom

# Title:	Add network slice expectation definition alternative

# Document for:	Approval

Agenda Item:	6.7.4

# 1	Decision/action requested

The group is asked to discuss and agree clause 4.

# 2	References

[1]	3GPP TS 28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

[2]	3GPP TS 28.552 Management and orchestration; 5G performance measurements

[3]	3GPP TS 28.554 Management and orchestration; 5G end to end Key Performance Indicators (KPI)

[4]	3GPP TS 22.104 Service requirements for cyber-physical control applications in vertical domains

# 3	Rationale

The ServiceProfile, see reference [1], captures the requirements from an MnS consumer that are conveyed to the MnS producer in a request to allocate an NetworkSlice.

The requirements from an MnS consumer can be about functionality, capacity, performance monitoring, within a given context. The ServiceProfile is in a NetworkSlice scenario treated as a flat list of requirements while an Intent Expectation has a structure that gives meaning and context to the requirements, the meaning and context is captured by the naming of the dataTypes included in an Intent Expectation.

The following dataTypes can be used; ObjectContext, ExpectationTarget, ExpectationContext. The current ServiceProfile attributes, see reference [1], are shown in Table 3.1 split between mandatory and optional. These attributes are mapped to Intent DataTypes which is shown in Table 3.2 with for each attribute the mapping and reason.

Table 3.1 ServiceProfile attributes [1]

| Mandatory attributes | Optional attributes |  |  |
| --- | --- | --- | --- |
| plmnInfoList | maxNumberofUEs | uLThptPerSlice | survivalTime |
| sST | coverageArea | uLThptPerUE | radioSpectrum |
|  | dLLatency | dLMaxPktSize | reliability |
|  | uLLatency | uLMaxPktSize | maxDLDataVolume |
|  | uEMobilityLevel | maxNumberofPDUSessions | maxULDataVolume |
|  | networkSliceSharingIndicator | kPIMonitoring | nBIoT |
|  | availability | userMgmtOpen | synchronicity |
|  | delayTolerance | v2XCommModels | positioning |
|  | dLDeterministicComm | termDensity | sliceSimultaneousUse |
|  | uLDeterministicComm | activityFactor | energyEfficiency |
|  | dLThptPerSlice | uESpeed | nssaaSupport |
|  | dLThptPerUE | jitter | n6Protection |

Table 3.2 Mapping table of ServiceProfile attributes to Intent

| ServiceProfile attribute | Intent construct | Reason |
| --- | --- | --- |
| pLMNId | Object context | When given the PLMN as context, the producer can use an existing or create a new NetworkSlice for that PLMN. |
| sST |  | When given the SST as context, the producer can use an existing or create a new NetworkSlice for that SST. |
| coverageArea |  | When given the coverageArea as context, the producer can use an existing or create a new NetworkSlice for that coverageArea. |
| uLLatency | Expectation target | Expectation from consumer on characteristics and the requirement fulfilment is measurable, see reference [2], [3] |
| dLLatency |  | Expectation from consumer on characteristics and the requirement fulfilment is measurable, see reference [2], [3] |
| uLThptPerUE |  | Expectation from consumer on characteristics and the requirements fulfilment is measurable, see reference [2], [3] |
| dLThptPerUE |  | Expectation from consumer on characteristics and the requirements fulfilment is measurable, see reference [2], [3] |
| uLThptPerSlice |  | Expectation from consumer on characteristics and the requirements fulfilment is measurable, see reference [2], [3] |
| dLThptPerSlice |  | Expectation from consumer on characteristics and the requirements fulfilment is measurable, see reference [2], [3] |
| availability |  | Expectation from consumer on characteristics and the requirements fulfilment is measurable, see reference [3] Editor’s Note: Currently there are no availability KPIs specified in TS 28.554. However, an Editor’s note in TS 28.554 states it will be there in the future. |
| reliability |  | Expectation from consumer on characteristics and the requirements fulfilment is measurable, see reference [3] |
| energyEfficiency |  | Expectation from consumer on characteristics and the requirements fulfilment is measurable, see reference [3]. |
| Editor’s Note: The use and mapping of the remaining ServiceProfile attributes in Intent is FFS |  |  |

The ServiceProfile attributes that are mapped to the ExpectationTarget datatype, are those that can represent the characteristics of communication services as defined clause in 5 of TS 22.104, see reference [4].

It is proposed to add this mapping as alternative under solution for “

# 4	Detailed proposal

### 6.2.x	Alternative 2: Modelling service and slice profile attributes as intent expectation for network slice

#### 6.2.x.1	ExpectationObject

Following are the specific allowed values to be used when implementing the IntentExpectation for a network slice.

Table 6.2.x.1-1

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | NetworkSlice |
| objectInstance (CM) | DN of the NetworkSlice |

Following attributes in ServiceProfile and SliceProfile can be modelled as ObjectContexts

Table 6.2.x.1-2

| ServiceProfile and SliceProfile attribute | ObjectContext attribute |
| --- | --- |
| coverageArea | coverageAreaPolygonContext, coverageAreaTAContext |
| plmnId | plmnId |
| sST | sST |

Editor’s Note: Whether any of the remaining attributes in ServiceProfile and SliceProfile are mapped to Table 6.2.x.1-2 is FFS

#### 6.2.x.2	ExpectationTargets

Following attributes in ServiceProfile and SliceProfile can be modelled as ExpectationTargets

Table 6.2.x.1-3

| ServiceProfile and SliceProfile attribute | ExpectationTarget attribute |
| --- | --- |
| uLLatency | uLLatency |
| dLLatency | dLLatency |
| uLThptPerUE | uLThptPerUE |
| dLThptPerUE | dLThptPerUE |
| uLThptPerSlice | uLThptPerSlice |
| dLThptPerSlice | dLThptPerSlice |
| availability | availability |
| reliability | reliability |
| energyEfficiency | energyEfficiency |

Editor’s Note: Whether any of the remaining attributes in ServiceProfile and SliceProfile are mapped to Table 6.2.x.1-3 is FFS

#### 6.2.x.3	ExpectationContext

Editor’s Note: The use of the ServiceProfile and SliceProfile attributes in ExpectationContext is FFS