3GPP TSG-SA5 Meeting #148e 	S5-233327

Electronic meeting, Online, 17 -25 April 2023

# Source:	Ericsson, Deutsche Telekom

# Title:	Use case for network slice intent

# Document for:	Approval

Agenda Item:	6.7.4

# 1	Decision/action requested

The group is asked to discuss and agree clause 4.

# 2	References

[1]	TS 28.312 Management and orchestration; Intent driven management services for mobile networks

[2]	TS 28.531 Management and orchestration; Provisioning

[3]	TS 28.536 Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3

# 3	Rationale

There is no text in the study report to describe the benefits of using an intent interface as specified in TS 28.312, see reference [1], for network slice management as opposed to using the interface specified in TS 28.531, see reference [2] and the interface specified in TS 28.536, see reference [3]. It is proposed to add a use case description capturing the benefits for a consumer to interact with a NetworkSlice producer through an intent-based interface.

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements"

[3] 	3GPP TS 28.531: "Management and orchestration; Provisioning"

[4]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[5]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

[x]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3"

…

Second change

## 5.A	Use case for network slice intent

### 5.A.1	Description

One of the drivers of adoption of 5G in the Telecoms industry is support for network slicing. The management solution for network slicing enables an operator to provision a network slice, to do fault supervision and performance assurance of a network slice.  Over time more features have been specified to make network slice management more efficient and effective, one example is the ability to create and monitor closed control loops that automatically adjust the network resources to assure that the goals specified in a service profile or slice profile are continuously met without manual intervention. The granularity of this solution is that of ServiceProfile/NetworkSlice and SliceProfile/NetworkSliceSubnet, the fulfilment information is reported on the granularity of a ServiceProfile/SliceProfile. An MnS consumer must specify and configure the closed control loop and the measurements and actions, before it can monitor the fulfilment of the goals, the solution assumes that the associated NetworkSlice or NetworkSliceSubnet are in operation.

- 	Currently the standards support an MnS consumer to provision a NetworkSlice or NetworkSliceSubnet using the mechanism in TS 28.531, see reference [3], and to assure the performance using the mechanism in TS 28.536, see reference [x], this means that a consumer must handle two separate requests to provision and assure a NetworkSlice. With the introduction of intent, the consumer can combine the provisioning and assurance requirements in a single request for an intent with a network slice expectation.

- 	While the object in case of using the intent interface is the NetworkSlice or NetworkSliceSubnet, the fulfilment information can be reported at the level that matches the requirements of the MnS consumer.

- 	For an MnS consumer it is easier to express the network slice expectations including requirements, goals and constraints using the data structures used for intent.

-	An MnS consumer does not have to have the knowledge about the relations between NetworkSlice provisioning and assurance, or between NetworkSliceSubnet provisioning and assurance.

### 5.A.2	Details

Editor’s Note: the content of this clause is FFS

### 5.A.3	Potential requirements

REQ-Intent_NetworkSlice-FUN-x1: The intent management service producer shall be able to provision and assure network slice intent expectations.

REQ-Intent_NetworkSlice-FUN-x2: The intent management service producer shall be able to fulfil network slice requirements, goals, and constraints.

REQ-Intent_NetworkSlice-FUN-x3: The intent management service producer shall be able to provision and assure network slice subnet intent expectations.

REQ-Intent_NetworkSlice-FUN-x4: The intent management service producer shall be able to fulfil network slice subnet requirements, goals, and constraints.

End of changes