3GPP TSG-SA5 Meeting #148e 	S5-233379

Electronic meeting, Online, 17-25 April 2023

# Source:	MATRIXX Software

# Title:	pCR 28.203 Introduce triggers

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

This pCR is to introduce triggers.

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.203 "Charging management; Network Slice Admission Control charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR is to introduce triggers.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.203 [2]:

First change

#### 5.2.1.2	Applicable triggers in the NSACF

##### 5.2.1.2.1	General

When a charging event is issued towards the CHF, it includes details of charging information such as S-NSSAI.

Each trigger condition (i.e. chargeable event) defined for the Network Slice Admission Control converged charging functionality with the associated behaviour when they are met, is specified this document, and the basic trigger mechanism is specified in the 3GPP TS 32.290 [x].

A Charging Data Request [Initial] is invoked by the NSACF towards the CHF when one of the charging session initiation trigger is met, and no charging session exists.

A Charging Data Request [Termination] is invoked by the NSACF towards the CHF when the last charging session termination trigger is met.

A Charging Data Request [Update] is invoked by the NSACF towards the CHF, when any trigger which is not a charging session termination trigger is met or a charging session termination trigger which is not the last one is met, and the charging session exists.

Upon the first Charging Data Response [Initial], the NSACF is optionally provided with NSACF charging profile overriding the default triggers configured in the Charging Characteristics described in Annex A.

The triggers remain active until they are updated or disabled by NSACF charging profile supplied by subsequent Charging Data Response [Initial/Update/Termination/Event] from the CHF.

The NSACF is configured with maximum numbers for registered UEs and PDU sessions, referred-to as "NSACF max Nb of Reg UEs" and "NSACF max Nb of PDU sessions" respectively, which cannot be exceeded by CCS determined maximum numbers "CCS max Nb of Reg UEs" and "CCS max Nb of PDU sessions" respectively. Within these "CCS max Nb of Reg UEs" and "CCS max Nb of PDU sessions" limits, quota of registered UEs and PDU sessions can be gradually allocated by CHF quota management functionality.

Table 5.2.1.2.1.1 summarizes the set of default trigger conditions which shall be supported by the NSACF when charging is active for the corresponding NSACF functionality, with the associated Charging Data Request message sent from NSACF towards the CHF.

Table 5.2.1.2.1.1: Default trigger conditions in NSACF

| Trigger conditions | CHF allowed to enable and disable |  |  |  |  | Message |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of registered UEs |  |  |  |  |  |  |  |
| Nb of Reg UEs threshold reached for initial | Yes |  |  |  |  | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Initial] or Charging Data Request [Update] |  |
| Nb of Reg UEs threshold crossed upwards | Yes |  |  |  |  | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |  |
| Nb of Reg UEs threshold crossed downwards |  | Yes |  | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |  |  |
| Nb of Reg UEs threshold reached for termination | Yes |  |  |  |  | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Termination] or Charging Data Request [Update] |  |
| Quota of Nb of Reg UEs reached | Yes | Charging Data Request [Update] with a possible request quota |
| Number of established PDU Sessions |  |  |  |  |  |  |  |
| Nb of PDU sessions threshold reached for initial | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Initial] or Charging Data Request [Update] |
| Nb of PDU sessions threshold crossed upwards | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Nb of PDU sessions threshold crossed downwards | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Nb of PDU sessions threshold reached for termination | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Termination] or Charging Data Request [Update] |
| Quota of Nb of PDU sessions reached | Yes | Charging Data Request [Update] with a possible request quota |

End of change