# 3GPP TSG-SA5 Meeting #145e	S5-233475

# 15 August to 24 August 2022, E-meeting

# Source:	China Mobile

# Title:	pCR 28.912 issue#4.11 Generic information model definition enhancement

# Document for:	approval

# Agenda Item:	6.7.3.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.912 “Study on enhanced intent driven management services for mobile networks ” V1.2.0 (2023-03)

3	Rationale

- The targetCondition attribute in ExpectationTarget<<dataType>> and the contextCondition attribute in Context<dataType>> can only be used to represent simple conditions, and cannot be used to represent complex conditions composed of multiple conditions in logical relationships. Therefore, it is proposed to enhance it with additional condition combination mechnism.

4	Detailed proposal

Start of Modified

4.11	Issue#4.11: Enhancement of generic Information model definition

4.11.1	Description

In TS 28.312 [2], the generic intent information model (including the Intent, IntentExpectation, ExpectationObject, ExpectationTarget, Context and Fulfilment) are defined. Following aspects for generic intent information model need to be enhanced:

- Enhancement Aspect 1: the intent activate/de-activate intent management capabilities are described in clause 4.2.2 in TS 28.312 [2], which enable the MnS consumer to request to suspend an intent on the MnS producer side or cancel the suspension for a suspended intent on the MnS producer side. The intent is suspended represents the state that the intent shall not be considered for fulfilment. However, currently there is no mechanism defined to enable the MnS consumer to request to suspend an intent on the MnS producer or cancel the suspension for a suspended intent on the MnS producer.

- Enhancement Aspect 2: the attribute "fulfilmentStatus" is defined to indicate whether the intent is fulfilled or not fulfilled. However, the observation period for the intent fulfilment evaluation is not defined.

- Enhancement Aspect 3: the attribute targetCondition in ExpectationTarget<<dataType>> and the attribute contextCondition in Context<dataType>> currently only support the AND relationship between conditions, not the OR relationship between conditions.

4.11.2	Potential solutions

Regarding the Enhancement Aspect 1, it proposes to add attribute "intentAdminState" in Intent <<IOC>>, the allowed value can be "ACTIVATED" and "DE-ACTIVATED". In case MnS consumer want to suspend an intent, MnS consumer can request MnS producer to configure attribute "intentAdminState" with value "De-ACTIVATED ". In case MnS consumer want to resume an intent on the MnS producer side when the intent is suspended, MnS consumer can request MnS producer to configure attribute "intentAdminState" with value " ACTIVATED ".

Regarding the Enhancement Aspect 2, it proposes to reuse attribute "observationTime" defined in clause 4.5.1to represent the period for evaluating the fulfilment information for corresponding Expectation Targets, IntentExpectation and Intent. The fulfilment information will be observed from the start of each observation period, then evaluated and derived at the end of each observation period. MnS producer configures the attributes related to intent fulfilment information at the end of each observation period also.

Regarding the Enhancement Aspect 3, it proposes to define "condition" as a common conditional expression information element, which supports both conditional expressions of simple numerical relationships and conditional expressions combined with different logical relationships such as AND and OR. Replace the original targetValueRange attribute and targetCondition attribute in ExpectationTarget<<dataType>>, as well as the original contextValueRange attribute and contextCondition attribute in Context<dataType>>, with the newly defined Condition<<dataType>>below.

The attributes of Condition<<dataType>> shall follow the indications provided in table 4.11.2-1.

Table 4.11.2-1: Attributes of Condition<<dataType>>

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conditionId | M | T | T | F | F |
| conditionName | M | T | T | F | F |
| operator | M | T | T | F | F |
| conditionValue | CM | T | T | F | F |
| conditionList | CM | T | T | F | F |

And the attribute definition of Condition<<dataType>> shall follow the indications provided in table 4.11.2-2.

Table 4.11.2-2 Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| conditionId | It describes identifier of this condition. allowedValues: Not Applicable | type: Identifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conditionName | It describes name of the condition. allowedValues: Not Applicable | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| operator | It describes conditional expressions allowedValues: If conditionValue exists, represents the specific requirements for condition. VALUES: - greater than - equal to - less than - not equal to - one of - some of - maximum value - minimum value - median - credibility If conditionList exists, representing the relationship between all conditions. VALUES: - and - or | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conditionValue | For a simple condition, it describes the specific value that composes the condition. - allowedValues: Not Applicable | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conditionList | For a composite condition, identifies the list of conditions that compose the specific condition. allowedValues: Not Applicable | type: Identifier multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

In ExpectationTarget<<dataType>>, remove the targetValueRange attribute and targetCondition attribute, and add the targetConditions attribute, whose type is the newly added condition<dataType>>.

In Context <<dataType>>, remove the contextValueRange attribute and contextCondition attribute, and add the contextConditions attribute, whose type is the newly added condition<dataType>>.

End of Modified