| 3GPP TR 28.863 V0.6.0 (2023-4) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on Key Quality Indicators (KQIs) for 5G service experience (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Issues and potential solutions	7

4.1	Issue # 1: Definition of KQI	7

4.1.1	Description	7

4.1.1.1	3GPP TR 21.905	7

4.1.1.2	3GPP TR 32.862	7

4.1.1.3	ETSI GS F5G 005	8

4.1.1.4	Background description of 5G service experience related studies in ITU-T	8

4.1.1.4.1	ITU-T SG12	8

4.1.1.4.2	ITU-T SG9	9

4.1.2	Potential solutions	9

4.1.2.a	Potential solution #<a>: <Potential Solution a Title>	9

4.1.2.a.1	Introduction	9

4.1.2.a.2	Description	9

4.1.3	Conclusion - Impact on normative work	9

4.2	Issue # 2: Scenarios for 5G KQI	9

4.2.1	Description	9

4.2.2	Potential solutions	9

4.2.2.1	Potential solution #1: Scenarios for 5G KQI	9

4.2.2.1.1	Introduction	9

4.2.2.1.2	Description	9

4.2.3	Conclusion - Impact on normative work	9

4.3	Issue # 3: KQIs for Video Uploading	10

4.3.1	Description	10

4.3.2	Potential solutions	10

4.3.2.a	Potential solution #<a>: <Potential Solution a Title>	10

4.3.2.a.1	Introduction	10

4.3.2.a.2	Description	10

4.3.3	Conclusion - Impact on normative work	10

4.4	Issue # 4: KQIs for Remote Controlling	10

4.4.1	Description	10

4.4.2	Potential solutions	10

4.4.2.a	Potential solution #<a>: <Potential Solution a Title>	10

4.4.2.a.1	Introduction	10

4.4.2.a.2	Description	10

4.4.3	Conclusion - Impact on normative work	10

4.5	Issue # 5: KQIs for Cloud VR	10

4.5.1	Description	10

4.5.1.1	General	10

4.5.1.2	Background	11

4.5.1.2.1	ITU SG9-TD896 GEN	11

4.5.1.2.2	ITU SG12 G.QoE-VR	11

4.5.1.3	Use case	12

4.5.1.3.1	Use case for stalling of 5G KQIs	12

4.5.2	Potential solutions	12

4.5.2.1	Potential solution #1: KQIs for Cloud VR	12

4.5.2.1.1	Introduction	12

4.5.2.1.2	Description	12

4.5.3	Conclusion - Impact on normative work	12

4.6	Issue # 6: Relation of KQI with the SLS requirements	12

4.6.1	Description	12

4.6.2	Potential solutions	12

4.6.2.a	Potential solution #<a>: <Potential Solution a Title>	12

4.6.2.a.1	Introduction	12

4.6.2.a.2	Description	12

4.6.3	Conclusion - Impact on normative work	13

Annex X (informative): Change history	14

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document investigates the definition and scenarios for KQI. The KQIs and the evaluation of the KQIs for each of the identified scenarios will be investigated. It also provides the analysis for the relation of KQI with the SLS requirements.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 32.862: "Study on Key Quality Indicators (KQIs) for service experience"

[3] 	ETSI GS F5G 005: "F5G High-Quality Service Experience Factors Release #1"

[4]	ITU-T Y.1540: Internet protocol data communication service - IP packet transfer and availability performance parameters

[5]	ITU-T P.1201: Parametric non-intrusive assessment of audiovisual media streaming quality

[6]	ITU-T P.1202: Parametric non-intrusive bitstream assessment of video media streaming quality

[7]	ITU-T P.1203: Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport

[8]	ITU-T P.1204: Video quality assessment of streaming services over reliable transport for resolutions up to 4K

[9]	ITU-T GSTR-5GQoE: Study of existing QoE requirements for 5G services

[10]	ITU-T P.10/G.100 (11/2017): Vocabulary for performance and quality of service.

[11]	ITU-T G.1035: Influencing factors on quality of experience for virtual reality services

[12]	Draft ITU-T P.1320 (ex P.QXM): QoE assessment of eXtended Reality (XR) meetings

[13]	Draft ITU-T G.1036 (ex G.QoE-AR): Quality of experience (QoE) influencing factors for augmented reality (AR) services

[14] 	Draft ITU-T G.QoE-VR (G.QoE-VR) "Influencing Factors on Quality of Experience (QoE) for Virtual Reality Services"

[15]		ITU-T SG 9 TD 896-GEN: "Output - Draft New Recommendation J. Cloud-VR "E2E Network Requirements of Cloud-VR Services"

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Key Quality Indicator: For purposes of TR 28.863, the term is defined to be an indicator which reflects the end-to-end service experience as perceived by the end user. It may be derived from a combination network layer measurements, service layer measurements and includes qualitative as well as quantitative components.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

KQI	Key Quality Indicators

4	Issues and potential solutions

4.1	Issue # 1: Definition of KQI

4.1.1	Description

In this clause, the background survey of the definition of KQI is provided. The TS/TR related to KQI will be listed here as the basis of the study.

4.1.1.1	3GPP TR 21.905

In TR 21.905 [1], no definition or abbreviation of KQI is provided.

4.1.1.2	3GPP TR 32.862

TR 32.862 [2] gives a description of KQI and some related survey on that:

The introduction of Key Quality Indicators (KQIs), which reflect the end-to-end service performance and quality, could be used to better represent real customer experience. NGMN NGCOR has defined principles for KQIs which could be used to evaluate the customer experience for fixed and mobile networks. It is proposed that SA5 takes the responsibility to study how to standardize the related service KQIs providing a standardised common base for end-to-end measurements and to fulfil the NGCOR requirements. KQIs have also been defined by ETSI (TS 102.250), 3GPP SA4 (TR 26.944), NGMN (P-SERQU) and QuEST Forum should also be considered.

The intention of KQI for the service experience is to try to reflect the customer experience with indicators reflecting them for example how many stalls occur while watching a movie, time delay before seeing the first screen of a movie. With this information, KQI could closely reflect the customer experience. The KPIs defined in the current specifications, e.g. 3GPP TS 32.450, are not differentiated by services, they only measure the bearer.

Editor’s note: It is still under investigation if there is an existing definition of KQI in other WGs or standard organizations.

4.1.1.3	ETSI GS F5G 005

ETSI GS F5G 005 [3] gives a description of KQI:

The Key Quality Indicators (KQI) are composed by the QoS metrics, which have the largest impact on QoE, namely

user centric and service specific quality patterns that directly influence the user perception for each service category.

QoS is a measure of the performance of networked services at the network or application level. QoS also refers to a set

of techniques that enable the network administrator to manage the network performance differentiating between

different users. QoS metrics may include network layer measurements such as packet loss, delay or jitter or application

level measurements such as video frame loss, frame freezing, image distortion.

4.1.1.4	Background description of 5G service experience related studies in ITU-T

4.1.1.4.1	ITU-T SG12

There are some studies related to KQIs, or relevant to, 5G service experience, with below a list of related ITU-T Recommendations and other output documents such as Technical Reports by ITU-T SG12, as well as work items under study.

There are also the different prediction models for speech and video quality available from SG12’s work (e.g. P.1201, P.1202, P.1203, P.1204 for video), which can directly provide KQI related information.

List of related in-force ITU-T Recommendations and Technical Reports are given according to video streaming and cloud VR:

Video streaming related documents:

–	ITU-T Y.1540 [4]: Internet protocol data communication service - IP packet transfer and availability performance parameters

–	ITU-T P.1201 [5]: Parametric non-intrusive assessment of audiovisual media streaming quality

–	ITU-T P.1202 [6]: Parametric non-intrusive bitstream assessment of video media streaming quality

–	ITU-T P.1203 [7]: Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport

–	ITU-T P.1204 [8]: Video quality assessment of streaming services over reliable transport for resolutions up to 4K

–	ITU-T GSTR-5GQoE [9]: Study of existing QoE requirements for 5G services

–	ITU-T P.10/G.100 (11/2017) [10]: Vocabulary for performance and quality of service.

Cloud VR related documents:

–	ITU-T G.1035 [11]: Influencing factors on quality of experience for virtual reality services

Consented draft ITU-T Recommendations:

–	Draft ITU-T P.1320 (ex P.QXM) [12]: QoE assessment of eXtended Reality (XR) meetings

–	Draft ITU-T G.1036 (ex G.QoE-AR) [13]: Quality of experience (QoE) influencing factors for augmented reality (AR) services

–	Draft ITU-T G.QoE-VR (G.QoE-VR) [14] "Influencing Factors on Quality of Experience (QoE) for Virtual Reality Services" (for consent)

4.1.1.4.2	ITU-T SG9

Cloud VR related documents:

–	ITU-T SG 9 TD 896-GEN [15]: "Output - Draft New Recommendation J. Cloud-VR "E2E Network Requirements of Cloud-VR Services"

4.1.2	Potential solutions

4.1.2.a	Potential solution #1: Definition of KQI

4.1.2.a.1	Introduction

In 5G there are some new services defined in 3GPP. The new services of video uploading (video streaming uplink), remote control and cloud VR have requirements e.g. related to latency, reliability and UL throughput for 5G networks or the combination of them. The metrics of network performance are defined as Key Performance Indicators (KPIs) for the 5G network, defined in TS28.554 [17]. However, it is not enough to only define separate network performance indicators. The ultimate goal of network management of services is to provide a good service experience to the user. Network resources are limited. Only focusing on the network management may not ensure optimal service experience. User experience-driven management may be enabled with key quality indicators (KQIs) which are used to better represent a user’s experience.

KQIs are important for service management. In user experience-driven management, KQIs may be used to propose service management objectives and express experience optimization requirements. Experience optimization requirements may be mapped to the network, and are ultimately achieved through network resource assurance and network performance optimization.

4.1.2.a.2	Description

KQI (Key Quality Indicator): An indicator which reflects the end-to-end service experience as perceived by the end user. It may be derived from a combination network layer measurements, service layer measurements and includes qualitative as well as quantitative components.

#### 4.1.2.b	Potential solution # 2: Difference of KPI, KQI and QoE

TS28.554 [17] specifies end-to-end Key Performance Indicators (KPIs) for the 5G network and network slicing.

KQI (Key Quality Indicator) is defined to be a measure of the experience perceived by the user.

In ITU-T P.10/G.100 [10] the definition of QoE is given:

Quality of experience (QoE)

The degree of delight or annoyance of the user of an application or service.

QoE is defined as the degree of delight or annoyance of the user of an application or service. It includes the complete end-to-end system effects (client, terminal, network, services infrastructure, etc.) and may be influenced by user expectations and context.

From the above definition of QoE we see that QoE is an overall value to reflect the degree of the user experience.

There are also measurements in SA4 that may be used to estimate QoE. These measurements are collected by the client and may be reported to the OAM or QoE server in the QoE metrics reporting. How to make use of these QoE metrics is not specified. One difference is that the QoE measurements are collected from the client side for all of the aspects of the media e.g. the buffer, play list, MPD information besides network aspects. KQIs may be calculated from KPIs, or calculated based on some of the collected QoE metrics. QoE metrics may be one component of KQI.

4.1.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

4.2	Issue # 2: Scenarios for 5G KQI

4.2.1	Description

In 5G there are various new services. Different services bring different service experience. The service experience of video uploading requires good quality of video and high efficiency of uploading; while for AR/VR requires good interaction additionally. For different service different KQIs are required to express the different service experience.  It is important to select the most typical services and investigate the KQIs for such services.

4.2.2	Potential solutions

4.2.2.1	Potential solution #1: Scenarios for 5G KQI

4.2.2.1.1	Introduction

This clause provides the proposed scenarios for 5G KQI.

4.2.2.1.2	Description

Video uploading is widely used in e.g. transportation, smart city; remote controlling is important for industry automation and telemedicine; Cloud VR is also expected to be widely used in daily life. In this document the 5G KQIs will be described for the following scenarios:

- Video uploading

- Remote controlling

- Cloud VR

4.2.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

4.3	Issue # 3: KQIs for Video Uploading

4.3.1	Description

The scenario of video uploading means the video streaming uplink. In TS26.247 [16] the QoE metrics for video streaming are defined which could be used as the source of the KQIs of video uploading.

Video uploading is widely used in transportation, smart city, telemedicine and industry automation. In these scenarios, a large number of videos are captured and uploaded to the server and are required to be low latency and High-definition. There are various requirements for the service experience of video uploading. For example, real-time video transmission is required in industrial control scenarios such as ports and mines, which requires short video transmission delay; in addition, for transmission of high-definition video, the high video transmission rate is required.

KQIs for video uploading is important for the management: In user experience-driven management, KQIs are used to declare management objectives of video uploading and indicate whether user experience need to be optimized.

4.3.2	Potential solutions

4.3.2.a	Potential solution #<a>: <Potential Solution a Title>

4.3.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

4.3.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

4.3.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

4.4	Issue # 4: KQIs for Remote Controlling

4.4.1	Description

Editor’s note: This clause provides a description of KQIs for Remote Controlling.

4.4.2	Potential solutions

4.4.2.a	Potential solution #<a>: <Potential Solution a Title>

4.4.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

4.4.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

4.4.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

4.5	Issue # 5: KQIs for Cloud VR

4.5.1	Description

4.5.1.1	General

Cloud VR (Cloud Virtual Reality) is a new cloud computing and cloud rendering technology for VR services, as describe in ITU SG9-TD896 GEN. It applies to scenarios such as VR games, VR 360-degree video, VR live broadcast, giant-screen cinema, and VR education. With fast and stable transport networks, VR content is stored and rendered in the cloud, and video and audio outputs are coded, compressed, and transmitted to user terminals. The large-scale deployment of Cloud VR services requires the joint effort from the enterprises to address E2E quality management and monitoring. Therefore, the development of measurable and manageable key quality indicators will be a key basis for operator to ensure the satisfaction of consumers. There are already some standards for studying the KQIs for Cloud VR. In this clause, the background and the use case of the KQI of the cloud VR are provided.

4.5.1.2	Background

4.5.1.2.1	ITU SG9-TD896 GEN

It describes the service indicators based on the experience evaluation factors of VR in ITU SG9-TD896 GEN, the experience evaluation factors of VR include sense of reality, interaction, and pleasure. The sense of reality depends on resolution, colour depth, frame rate, and encoding compression technologies. Cloud VR implements computing and rendering on the cloud, any latency from remote processing compromises the sense of immersion and imagination. The sense of pleasure depends on the smoothness of VR services.

The Cloud VR includes the service indicators:

| Factors | service indicators |
| --- | --- |
| Sense of reality | Resolution |
|  | Frame rate |
|  | Field of View (FoV) |
|  | Color depth |
|  | Coding compression |
| Sense of interaction | Initial buffer time |
|  | Motion sensory conflict |
|  | Operation latency |
| Sense of pleasure | Stalling(time and duration percentage) |
|  | Artrafacts(times and duration/area percentage) |
|  | Effective frame rate in gaming |

4.5.1.2.2	ITU SG12 G.QoE-VR

In ITU SG12 G.QoE-VR, it describes that the VR QoE is assessed by the level of immersion provided by the VR system combined with the quality of the interaction between the user and the VR environment, and the level of immersion can be determined by the quality of the immersive media and the presentation quality delivered by the device. It also summarizes some key influences factors that impact quality of the immersive media, presentation quality, and interaction quality.

It includes the key influences factors:

| Category | Key influence factors |
| --- | --- |
| Quality of Immersive media | Degrees of freedom (DoF) |
|  | Video Three-dimensionality |
|  | Video Field of View (FoV) |
|  | Video Spatial resolution in pixels per degree (PPD) |
|  | Video Frame-rate |
|  | Video Compression |
|  | Audio Three-dimensionality |
|  | Audio Channels |
|  | Audio sample rate |
|  | Audio Compression |
| Presentation quality | Playback quality(e.g. smoothness and number of freezes, video/audio quality change) |
|  | Audio and video spatial alignment. |
|  | Audio and video synchronization. |
| Interaction quality | Intractability with objects in the VR environment |
|  | Response time between human action and adaptation in sound and display |
|  | Spatial precision between human action and adaptation in sound (3D audio) and visual information |

4.5.1.3	Use case

4.5.1.3.1	Use case for stalling of 5G KQIs

Stalling is an important indicator for end users to perceive the smoothness of streaming media when using Cloud VR services (e.g. VR games). In the process of VR video, there will be a certain amount of buffer. Because the download rate of the network is not enough to meet the output requirements of video coding quality, there will be no data in the buffer for playback, which will cause stalling and affect the end users’ experience. Therefore, This KQI is of great importance to estimate the service quality.

4.5.2	Potential solutions

4.5.2.1	Potential solution #1: KQIs for Cloud VR

4.5.2.1.1	Introduction

In this potential solution, KQIs for Cloud VR is provided.

4.5.2.1.2	Description

Stalling frequency (StallingFre)

Stalling frequency indicates the number of times frame stalling occurs in a unit of time during video playback. The calculation the video stalling frequency is as follows: The number of stalling is described in TS26.247 [16], the video stalling frequency is the number of video stalling times during video playback devided by the video playback duration. The unit is times/minute. The KQI type is Float.

Stalling duration (StallingDur)

Stalling duration indicates the duration of the stalling. The stalling start time and stalling end time is described in TS26.247 [16]. 3GPP management system collects the stalling start time and the stalling end time. The formula for calculating the stalling duration is the stalling end time minus the stalling start time. The KQI unit is ms and the KQI type is Float.

4.5.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

4.6	Issue # 6: Relation of KQI with the SLS requirements

4.6.1	Description

Editor’s note: This clause provides a description of the relation of KQI with the SLS requirements.

4.6.2	Potential solutions

4.6.2.a	Potential solution #<a>: <Potential Solution a Title>

4.6.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

4.6.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

4.6.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5-142e | S5-222673 | - | - | - | TR 28.863-0.0.0 initial skeleton | 0.0.0 |
| 2022-04 | SA5-142e | S5-222674 S5-222109 S5-222675 |  |  |  | Update to implement the agreed pCRs in SA5#142e: S5-222674 pCR TR28.863 Skeleton proposal for TR28.863 S5-222109 pCR TR 28.863 add scope S5-222675 pCR TR 28.863 Key Issue # 2 Scenarios for 5G KQI | 0.1.0 |
| 2022-05 | SA5-143e | S5-223741 |  |  |  | Update to implement the agreed pCRs in SA5#143e: S5-223741 pCR TR 28.863 Key Issue #1 Definition of KQI | 0.2.0 |
| 2022-08 | SA5-145e | S5-225778 S5-225869 |  |  |  | Update to implement the agreed pCRs in SA5#145e: S5-225778 pCR TR 28.863 Issue#1 Background description of KQI definition S5-225869 pCR TR 28.863 Key Issue # 1: add a description of KQI in ETSI GS F5G 005 | 0.3.0 |
| 2022.11 | SA5-146e | S5-226391 S5-226390 S5-226389 |  |  |  | Update to implement the agreed pCRs in SA5#146e: S5-226391 pCR TR 28.863 Key Issue # 5: KQIs for cloud VR- the background survey of the Cloud VR S5-226390 pCR TR 28.863 Key Issue # 5: KQIs for cloud VR- use case of the Cloud VR S5-226389 pCR TR 28.863 Key Issue #5 KQIs for Cloud VR- the solution of KQIs for cloud VR | 0.4.0 |
| 2023.3 | SA5-147 | S5-232866 |  |  |  | Update to implement the agreed pCRs in SA5#147: S5 232866 pCR TR 28.863 Issue 3 Video uploading description | 0.5.0 |
| 2023.4 | SA5-148e | S5-233469 |  |  |  | Update to implement the agreed pCRs in SA5#148e: S5-233469 pCR TR 28.863 Issue#1 definition of KQI | 0.6.0 |