3GPP TSG-SA5 Meeting #150 	S5-235096

# Göteborg, Sweden, 21-25 August 2023

# Source:	Ericsson

# Title:	Discussion paper on support for V2X in service profile

# Document for:	Approval

Agenda Item:	6.3.9

# 1	Decision/action requested

The group is asked to endorse the detailed proposal in section 4.

# 2	References

[1]	3GPP TS 28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

[2]	GSMA NG.116 Generic Network Slice Template, Version 8.0, 27 January 2023.

# 3	Rationale

A service profile represents the properties of the network slice related requirements that should be supported by a NetworkSlice instance in a 5G network. The service profile is defined as a dataType and the attributes represent the properties of the network slice related requirements. One of the requirements from an NSC to an NSP could be that the NetworkSlice supports V2X.

To indicate support for V2X as described in GST, see reference [2] section 3.4.35, the dataType V2XCommMode has been defined in TS 28.541, see reference [1] clause 6.3.16. This dataType combines servAttrCom with V2XMode. The dataType servAttrCom finds its origin in the GST and gives additional information about the attribute (categories, tagging and exposure), see reference 1 clause 6.3.6. The attribute V2XMode indicates if a V2XMode is supported and if it is which mode. The definitions of the V2X attributes from TS 28.541 are shown in Table 3.1..

Table 3.1: Definitions of v2XCommMode [1]

| Attribute Name |  | Documentation and Allowed Values |  | Properties |  |
| --- | --- | --- | --- | --- | --- |
| v2XCommModels |  | An attribute specifies whether or not the V2X communication mode is supported by the network slice. |  | type: V2XCommMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |  |
| V2XCommMode.v2XMode |  | An attribute specifies whether or not the V2X communication mode is supported by the network slice. allowedValues: "NOT SUPPORTED", "SUPPORTED BY NR". |  | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |  |

Observation: According to the definition the attribute v2XCommMode is a single instance of the dataType V2XCommMode, the attribute name v2XCommModels does not reflect this. It may give the impression that it is a list which it is not.

Conclusion: Based on the observation, the group concludes that it may be beneficial to change the attribute name v2XCommModels to v2XCommMode.

When the group endorse the conclusion the CR’s shown in Table 3.2 are needed:

Table 3.2: Overview of CR’s needed to support conclusion

| TS | baseline | Proposal | CR category | Contribution for #150 |
| --- | --- | --- | --- | --- |
| 28.541 | Rel-16 | Correct attribute name for indication of v2XCommMode | F | S5-235078 |
| 28.541 | Rel-17 | Correct attribute name for indication of v2XCommMode | A | S5-235079 |
| 28.541 | Rel-18 | Correct attribute name for indication of v2XCommMode | A | S5-235080 |

# 4	Detailed proposal

The group is asked to endorse the observation and the conclusion in section 3, which is shown in italic below here.

Conclusion: Based on the observation, the group concludes that it may be beneficial to change the attribute name v2XCommModels to v2XCommMode.