3GPP TSG-SA5 Meeting #150	S5-235158

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0191 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.531 Add UML source code for Network Slice LCM procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UML source code for the Network Slice LCM procedures defined in clause 7.2 is available in Annex C.2. This CR adds the UML source code to Annex C for the procedures in clause 7.3 to 7.7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding UML for the Network Slice LCM procedures in Annex C |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UML source code for Network Slice LCM procedures not available for any corrections. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.a (new), C.b (new), C.c (new), C.d (new), C.e (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

Annex C (informative):
Plant UML source code

# C.1	General

This annex contains the PlantUML source code for procedures defined in clause 7 of the present document.

# C.2	Procedure of network slice instance allocation

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSMS_C" as NSMS_Consumer

participant "\tNSMS_P\t\t" as NSMS_Provider

alt NSMS_C sends AllocateNsi request

NSMS_Consumer -> NSMS_Provider: 1a-1. AllocateNsi(ServiceProfile) Request

NSMS_Provider -> NSMS_Provider: 1a-2. Creates NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1a-3. AllocateNsi(ServiceProfileId) Response

NSMS_Provider --> NSMS_Consumer: 1a-4. notifyMOICreation(\n\tobjectClass=NetworkSliceController, \n\tobjectInstance=DN of NetworkSliceController MOI,...)

else NSMS_C sends createMOI request

NSMS_Consumer -> NSMS_Provider: 1b-1. createMOI(\n\t managedObjectClass=NetworkSliceController, \n\t attributeListIn=inputServiceProfile,...) Request

NSMS_Provider -> NSMS_Provider: 1b-2. Creates NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1b-3. createMOI() Response

end alt

NSMS_Provider -> NSMS_Provider: 2. Decides to create a new NetworkSlice MOI\

\n\t or use an existing NetworkSlice MOI.

alt Using an existing NetworkSlice MOI

NSMS_Provider -> NSMS_Provider: 3a-1. Identifies the NetworkSlice MOI to re-use

NSMS_Provider -> NSMS_Provider: 3a-2. Configures NetworkSlice MOI with \n DN of NetworkSliceController MOI

NSMS_Provider -> NSMS_Provider: 3a-3. Configures NetworkSliceController MOI with \n DN of NetworkSlice MOI

opt If the existing NetworkSlice MOI needs to be modified

ref over NSMS_Provider

3a-4. Procedure of Network Slice Instance Modification

end ref

end opt

else Creating a new NetworkSlice MOI

NSMS_Provider -> NSMS_Provider: 3b-1. Derives the network slice subnet related requirements

ref over NSMS_Provider

4b-2. Procedure of Network Slice Subnet Instance Allocation

end ref

|||

NSMS_Provider -> NSMS_Provider: 3b-3. Creates NetworkSlice MOI and \n configures NetworkSlice MOI with \nDN of NetworkSliceController MOI and \nDN of NetworkSliceSubnet MOI

NSMS_Provider -> NSMS_Provider: 3b-4. Configures NetworkSliceController MOI with \n DN of NetworkSlice MOI

|||

end alt

NSMS_Provider -> NSMS_Provider: 4. Updates attributes of NetworkSliceController MOI\nindicating completion of the procedure

group Monitoring network slice allocation progress and result. \nThis can be happened anytime after step NetworkSliceController is created (i.e., step 1a-2 or step 1b-2)

alt obtain the progress and result by subscribe-notification method

NSMS_Provider -> NSMS_Consumer: 5a. Notify network slice allocation progress and result \n (DN of NetworkSliceController MOI and its attributes)

else obtain the progress and result by query method

NSMS_Consumer -> NSMS_Provider: 5b-1. Request to query the \n NetworkSliceController information (DN of NetworkSliceController)

NSMS_Provider -> NSMS_Consumer: 5b-2. Response with the NetworkSliceController \n information (DN of NetworkSliceController and values of its attribute)

end alt

@enduml

# C.a	Procedure of Network Slice Subnet Instance Allocation

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSSMS_C" as NSSMS_Consumer

participant "NSSMS_P" as NSSMS_Provider

'alt CRUD operations not visible to the consumer

alt NSSMS_C sends AllocateNssi request

NSSMS_Consumer -> NSSMS_Provider: 1a-1. AllocateNssi(SliceProfile) Request

NSSMS_Provider -> NSSMS_Provider: 1a-2. Creates NetworkSliceSubnetController MOI

NSSMS_Provider --> NSSMS_Consumer: 1a-3. AllocateNssi(SliceProfileId) Response

NSSMS_Provider -> NSSMS_Consumer: 1a.4. notifyMOICreation(\n\tobjectClass=NetworkSliceSubnetController, \n\tobjectInstance=DN of NetworkSliceSubnetController MOI,...)

else NSSMS_C sends createMOI request

NSSMS_Consumer -> NSSMS_Provider: 1b-1. createMOI(\n\t managedObjectClass=NetworkSliceSubnetController, \n\t attributeListIn=inputSliceProfile,...) Request

NSSMS_Provider -> NSSMS_Provider: 1b-2. Creates NetworkSliceSubnetController MOI

NSSMS_Provider --> NSSMS_Consumer: 1b-3. createMOI() Response

end

ref over NSSMS_Provider

2. Procedure of feasibility check and reservation of network slice subnet

end ref

NSSMS_Provider -> NSSMS_Provider: 3. Decides to create a new NetworkSliceSubnet MOI\

\n\t or use an existing NetworkSliceSubnet MOI.

alt Using an existing NetworkSliceSubnet MOI

NSSMS_Provider -> NSSMS_Provider: 4a-1. Identifies the NetworkSliceSubnet MOI to re-use

|||

NSSMS_Provider -> NSSMS_Provider: 4a-2. Configures NetworkSliceSubnet MOI with DN of NetworkSliceSubnetController MOI

NSSMS_Provider -> NSSMS_Provider: 4a-3. Configures NetworkSliceSubnetController MOI with\nDN of NetworkSliceSubnet MOI

opt If the existing NetworkSliceSubnet MOI needs to be modified

|||

ref over NSSMS_Provider

4a-4. Procedure of Network Slice Subnet Instance Modification

end ref

end opt

else Using a new NetworkSliceSubnet MOI

NSSMS_Provider -> NSSMS_Provider: 4b-1. Creates an MOI of NetworkSliceSubnet and \nconfigures NetworkSliceSubnet MOI with DN of NetworkSliceSubnetController MOI

NSSMS_Provider -> NSSMS_Provider: 4b-2. Configures NetworkSliceSubnetController MOI with\nDN of NetworkSliceSubnet MOI

NSSMS_Provider -> NSSMS_Provider: 4b-3. Derives the requirements for corresponding\n network slice subnet constituent(s) and transport network

opt If the NetworkSliceSubnet MOI to be created contains virtualization part (i.e., VNF or VL)

|||

ref over NSSMS_Provider

4b-4. NS instantiation procedure

end ref

NSSMS_Provider -> NSSMS_Provider: 4b-5. Configures the NetworkSliceSubnet MOI attribute nsInfo

end opt

loop 4b-6. for each constituent NetworkSliceSubnet MOI

|||

ref over NSSMS_Provider

4b-6a. Procedure of Network Slice Subnet Instance Allocation

end ref

NSSMS_Provider -> NSSMS_Provider: 4b-6b. Configures the NetworkSliceSubnet MOI \n attribute networkSliceSubnetRef with the DN constituent NetworkSliceSubnet

end loop

loop 4b-7. for each constituent Network Function

|||

ref over NSSMS_Provider

4b-7a. \tProcedure of NF instance creation or

\t\tProcedure of NF instance modification

end ref

NSSMS_Provider -> NSSMS_Provider: 4b-7b. Configures the NetworkSliceSubnet MOI \n attribute managedFunctionRef with the DN constituent ManagedFunction

end loop

loop for each transport network requirements

|||

ref over NSSMS_Provider

4b-8. Procedure of TN coordination

supporting network slicing

|||

end ref

end loop

end alt

|||

NSSMS_Provider -> NSSMS_Provider: 5. Configures attributes of NetworkSliceSubnetController MOI\nindicating completion of the procedure

group Monitoring network slice subnet allocation progress and result. \nThis can be happened anytime after step NetworkSliceSliceController is created (i.e., step 1a-2 or step 1b-2)

alt obtain the progress and result by subscribe-notification method

NSSMS_Provider -> NSSMS_Consumer: 6a. Notify network slice subnet allocation progress and result \n (DN of NetworkSliceSliceController MOI and its attributes)

else obtain the progress and result by query method

NSSMS_Consumer -> NSSMS_Provider: 6b-1. Request to query the \n NetworkSliceSubnetController information (DN of NetworkSliceSliceController)

NSSMS_Provider -> NSSMS_Consumer: 6b-2. Response with the NetworkSliceSubnetController \n information (DN of NetworkSliceSliceController and values of its attribute)

end alt

@enduml

# C.b	Procedure of Network Slice Instance Deallocation

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSMS_C" as NSMS_Consumer

participant "NSMS_P" as NSMS_Provider

alt NSMS_C sends DeallocateNsi request

NSMS_Consumer -> NSMS_Provider: 1a-1. DeallocateNsi(networkSliceDN, serviceProfileId) Request

NSMS_Provider -> NSMS_Provider: 1a-2. Identifies the DN of NetworkSlice MOI to be deleted.

NSMS_Provider -> NSMS_Provider: 1a-3. Updates NetworkSlice MOI to remove the \nreference to DN of NetworkSliceController

NSMS_Provider -> NSMS_Provider: 1a-4. Deletes NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1a-5. DeallocateNsi() Response

else NSMS_C sends deleteMOI request

NSMS_Consumer -> NSMS_Provider: 1b-1. deleteMOI(baseObjectInstance=DN of NetworkSliceController) Request

NSMS_Provider -> NSMS_Provider: 1b-2. Identifies the DN of NetworkSlice MOI \nfrom the attribute networkSliceRef and \nvalue of attribute serviceProfileId from \nthe NetworkSliceController MOI to be deleted.

NSMS_Provider -> NSMS_Provider: 1b-3. Updates NetworkSlice MOI to remove the \nreference to DN of NetworkSliceController

NSMS_Provider -> NSMS_Provider: 1b-4. Deletes NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1b-5. deleteMOI() Response

end

NSMS_Provider -> NSMS_Provider: 2. Decides to delete or modify NetworkSlice MOI

alt Delete NetworkSlice MOI identified in step 1b-2. \t

NSMS_Provider -> NSMS_Provider: 3a-1. Deletes NetworkSlice MOI

ref over NSMS_Provider

3a-2. Procedure of network slice subnet instance deallocation

end ref

else Modify NetworkSlice MOI identified in step 1b-2.

|||

ref over NSMS_Provider

3b-1. Procedure of Network Slice Subnet Instance Modification

end ref

NSMS_Provider -> NSMS_Provider: 3b-2. Modify NetworkSlice MOI

end

@enduml

# C.c	Procedure of network slice subnet instance deallocation

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSSMS_C" as NSSMS_Consumer

participant "NSSMS_P" as NSSMS_Provider

'participant "NSSMS_Provider" as NSSMS_Consumer

alt NSSMS_C sends DeallocateNssi request

NSSMS_Consumer -> NSSMS_Provider: 1a-1. DeallocateNssi(networkSliceSubnetDN, sliceProfileId) Request

NSSMS_Provider -> NSSMS_Provider: 1a-2. Identifies the DN of NetworkSliceSubnet MOI to be deleted

NSSMS_Provider -> NSSMS_Provider: 1a-3. Updates NetworkSliceSubnet MOI attribute networkSliceSubnetControllerRef \n\tto remove DN of NetworkSliceSubnetController

NSSMS_Provider -> NSSMS_Provider: 1a-4. Deletes NetworkSliceSubnetController MOI

NSSMS_Provider --> NSSMS_Consumer: 1a-5. DeallocateNssi() Response

else NSSMS_C sends deleteMOI request

NSSMS_Consumer -> NSSMS_Provider: 1b-1. deleteMOI(baseObjectInstance=DN of NetworkSliceSubnetController) Request

NSSMS_Provider -> NSSMS_Provider: 1b-2. Identifies the DN of NetworkSliceSubnet MOI \nfrom the attribute networkSliceSubnetRef and \nvalue of attribute sliceProfileId from the \nNetworkSliceSubnetController MOI to be deleted

NSSMS_Provider -> NSSMS_Provider: 1b-3. Updates NetworkSliceSubnet MOI attribute networkSliceSubnetControllerRef \n\tto remove DN of NetworkSliceSubnetController

NSSMS_Provider -> NSSMS_Provider: 1b-4. Deletes NetworkSliceSubnetController MOI

NSSMS_Provider --> NSSMS_Consumer: 1b-5. deleteMOI() Response

end

NSSMS_Provider -> NSSMS_Provider: 2. Decides to delete or modify NetworkSliceSubnetSubnet MOI

alt 3a. Delete the NetworkSliceSubnetSubnet MOI identified in step 1b-2.

opt If the NetworkSliceSubnet MOI consists of constituent NetworkSliceSubnetSubnet MOIs

loop for each constituent NetworkSliceSubnetSubnet MOI

|||

ref over NSSMS_Provider

3a-1. Procedure of network slice subnet instance deallocation

end ref

end loop

end opt

opt If the NetworkSliceSubnet MOI consists of NFs

loop for each constituent ManagedFunction MOI

|||

ref over NSSMS_Provider

3a-2. Procedure of NF instance deletion or

\t\tProcedure of NF instance modification

end ref

end loop

end opt

opt If the NetworkSliceSubnet MOI contains TN part

|||

ref over NSSMS_Provider

3a-3. Procedure of TN coordination supporting network slicing

end ref

end opt

opt If the NetworkSliceSubnet MOI contains virtualized part

|||

ref over NSSMS_Provider

3a-4. NS Termination procedure

end ref

end opt

NSSMS_Provider -> NSSMS_Provider: 3a-5. Deletes NetworkSliceSubnet MOI

else 3b. Modify the NetworkSliceSubnetSubnet MOI identified in step 1b-2.

opt If the NetworkSliceSubnet MOI consists of constituent NetworkSliceSubnetSubnet MOIs

loop for each constituent NetworkSliceSubnetSubnet MOI

|||

ref over NSSMS_Provider

3b-1. Procedure of network slice subnet instance modification

end ref

end loop

end opt

NSSMS_Provider -> NSSMS_Provider: 3b-2. Modifies NetworkSliceSubnet MOI

end

@enduml

# C.d	Procedure of Network Slice Instance Modification

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSMS_C" as NSMS_Consumer

participant "NSMS_P" as NSMS_Provider

alt NSMS_C sends new network slice related requirements in NetworkSlice MOI

NSMS_Consumer -> NSMS_Provider: 1a. modifyMOIAttributes(\n\t baseObjectInstance=NetworkSlice, \n\t modificationList=serviceProfileList,...) request

else NSMS_C sends new network slice related \n requirements in inputServiceProfile

NSMS_Consumer -> NSMS_Provider: 1b-1. modifyMOIAttributes(\n\t baseObjectInstance=NetworkSliceController, \n\t modificationList=inputServiceProfile,...) request

NSMS_Provider -> NSMS_Provider: 1b-2. Updates NetworkSliceController MOI attributes\n with new inputServiceProfile along with progress and state information.

NSMS_Consumer <-- NSMS_Provider: 1b-3. modifyMOIAttributes(modificationListOut=...) response

end alt

ref over NSMS_Provider

2. Procedure of feasibility check and reservation of network slice

end ref

NSMS_Provider -> NSMS_Provider: 3. Decomposes the request to network slice subnet requirements.

ref over NSMS_Provider

4. Procedure of network slice subnet instance modification

end ref

NSMS_Provider -> NSMS_Provider: 5. Updates NetworkSlice MOI attributes (serviceProfileList)

opt if NSMS_C sent new network slice related requirements in NetworkSlice MOI (in step 1a)

Note over NSMS_Provider: NSMS_P sends response for modifyMOIAttributes request received in step 1a.

NSMS_Consumer <-- NSMS_Provider: 6a. modifyMOIAttributes(modificationListOut=...) response

else if NSMS_C sent new network slice related requirements in NetworkSliceController MOI (in step 1b-1)

NSMS_Provider -> NSMS_Provider: 6b. Updates attributes of NetworkSliceController MOI \nindicating completion of the procedure

end opt

@enduml

# C.e	Procedure of Network Slice Subnet Instance Modification

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSSMS_C" as NSSMS_Consumer

participant "NSSMS_P" as NSSMS_Provider

alt NSSMS_C sends new network slice subnet related requirements in NetworkSliceSubnet MOI

NSSMS_Consumer -> NSSMS_Provider: 1a. modifyMOIAttributes(\n\t baseObjectInstance=NetworkSliceSubnet, \n\t modificationList=sliceProfileList,...) request

else NSSMS_C sends new network slice subnet related requirements in NetworkSliceSubnetController MOI

NSSMS_Consumer -> NSSMS_Provider: 1b-1. modifyMOIAttributes(\n\t baseObjectInstance=NetworkSliceSubnetController, \n\t modificationList=inputSliceProfile,...) request

NSSMS_Provider -> NSSMS_Provider: 1b-2. Updates NetworkSliceSubnetController MOI attributes\n with new inputSliceProfile along with progress and state information.

NSSMS_Consumer <-- NSSMS_Provider: 1b-3. modifyMOIAttributes(modificationListOut=...) response

end alt

ref over NSSMS_Provider

2. Procedure of feasibility check and reservation of network slice subnet

end ref

NSSMS_Provider -> NSSMS_Provider: 3. Decomposes the modification request for each constituent MOIs.

loop for each constituent NetworkSliceSubnet MOIs

|||

ref over NSSMS_Provider

4a. Procedure of network slice subnet instance modification

end ref

end loop

loop for each constituent ManagedFunction MOIs

|||

ref over NSSMS_Provider

4b. Procedure of NF instance creation or

\t\tProcedure of NF instance modification

end ref

end loop

opt The NSSI contains virtualized parts

|||

ref over NSSMS_Provider

4c. NS instance scaling/updating/instantiation

procedures

end ref

end opt

opt The NSSI contains TN parts

'|||

ref over NSSMS_Provider

4d. Procedure of TN coordination

supporting network slicing

end ref

end opt

NSSMS_Provider -> NSSMS_Provider: 5. Updates NetworkSliceSubnet MOI attributes

opt if NSSMS_C sent new network slice related requirements in NetworkSliceSubnet MOI (in step 1a)

Note over NSSMS_Provider: NSSMS_P sends response for modifyMOIAttributes request received in step 1a.

NSSMS_Consumer <-- NSSMS_Provider: 6a. modifyMOIAttributes(modificationListOut=...) response

else if NSSMS_C sent new network slice related requirements in NetworkSliceSubnetController MOI (in step 1b-1)

NSSMS_Provider -> NSSMS_Provider: 6b. Updates attributes of NetworkSliceSubnetController MOI \nindicating completion of the procedure

end opt

@enduml

End of change