3GPP TSG-SA5 Meeting #150 	S5-235172

# Goteborg, Sweden, 21 Aug 2023 – 25 Aug 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Rel-18 pCR 28.844 Converged Charging Sattellite backhaul Solution Update

# Document for:	Approval

Agenda Item:	7.5.6

# 1	Decision/action requested

Approve new satellite backhaul use case.

# 2	References

[1]	3GPP TR 28.844 " Study on charging aspects of satellite in the 5G System (5GS)".

# 3	Rationale

Converged Charging Sattellite backhaul Solution Update which include Satellite Backhaul Information.

# 4	Detailed proposal

The following changes are proposed for TR 28.844[1].

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[5]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[6]	3GPP TR 23.700-27: "Study on 5G System with Satellite Backhaul".

[7]	3GPP TR 23.700-28: "Study on Integration of satellite components in the 5G architecture".

[8]	3GPP TR 23.737: "Study on architecture aspects for using satellite access in 5G".

[9]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[10]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2"

[11]	3GPP TS 32.290: " Services, operations and procedures of charging using Service Based Interface (SBI) "

Second change

### 5.2.4	Possible solutions

#### 5.2.4.1	Solution #2.1: SMF Charging Trigger Function (CTF) based solution for satellite backhaul charging

##### 5.2.4.1.1	General description

This solution which relying on CHF/5G Converged Charging System for satellite backhaul charging, addresses the Key Issue#2.1, Key Issue#2.2 and Key Issue#2.X.

During the PDU Session establishment procedure, if the AMF is aware that a satellite backhaul is used towards 5G AN, the AMF may report this to SMF. If AMF is aware that satellite backhaul category changes (e.g. at handover), the AMF reports the current satellite backhaul category and indicates the satellite backhaul category change to SMF.

If dynamic satellite backhaul is used, i.e. capabilities (latency and/or bandwidth) of the satellite backhaul change over time due to e.g. use of varying inter-satellite links as part of backhaul, the AMF notifies the SMF of the corresponding dynamic satellite backhaul category to serve the PDU Session. Then, the SMF informs this to PCF. The PCF may trigger QoS monitoring as specified in TS 23.501 [2].

Since the SMF obtains the satellite backhaul categories or satellite backhaul category changes (e.g. at handover) from the AMF, the SMF can be used to provide the charging information to support the satellite backhaul charging.

##### 5.2.4.1.2	Architecture description

The converged charging architecture for SMF Charging Trigger Function (CTF) based solution as depicted in figure 5.2.4.1.2-1. SMF based solution-CTF and are proposed for the session based charging.

Figure 5.2.4.1.2-1: 5G data connectivity converged charging architecture defined in TS 32.255 [10]

##### 5.2.4.1.3	Charging procedures for sateliite backhaul

The figure 5.2.4.1.3-1 describes the high level charging procedure for SMF (CTF) Converged charging for satellite backhaul charging.

Figure 5.2.4.1.3-2: Message flow for satellite backhaul charging

1.	SMF received the PDU session establishment request including satellite backhaul category as specified in clause 4.3.2.2 TS 23.502 [3].

1ch-a. The SMF sends Charging Data Request [Initial, Requestd Quota] including satellite backhaul category to CHF for the satellite backhaul charging.

1ch-b. The CHF creates a CDR.

1ch-c. The CHF acknowledges by sending Charging Data Response [Initial, Granted Quota] to the SMF.

2.	SMF sends the satellite backhaul usage report request to UPF during N4 Session Establishment/Modification procedure, if the SMF obtains the satellite backhaul category or satellite backhaul category changes from AMF.

3.	UPF starts QFI related to the satellite backhaul counts and detects an event that the satellite backhaul usage report has to be reported.

4.	UPF/PSA UPF reports the satellite backhaul usage report to SMF.

4ch-a. The SMF sends Charging Data Request[Update, Requestd Quota] to the CHF including the satellite backhaul usage report (which can contain information on the start time and end time for its usage), satellite backhaul delay and it may include further information for Satellite Backhaul QoS (e.g. throughput, latency, jitter, error rate).

4ch-b. The CHF update a CDR.

4ch-c. The CHF acknowledges by sending Charging Data Response [Update, Granted Quota] to the SMF.

5.	SMF sends the granted quotas to the UPF/PSA UPF in the usage reporting response.

6.	UPF/PSA UPF reports the usage reporting of the last satellite backhaul traffic to SMF.

6ch-a. The SMF sends Charging Data Request [Termination, Used Units] to CHF including the satellite backhaul traffic usage report.

6ch-b. The CHF close a CDR.

6ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the SMF.

7.	SMF sends the usage reporting response to the UPF/PSA UPF.

##### 5.2.4.1.X	CDR Information Structure with Satellite Backhaul information

The Table 1 provides the information structure, as described in 3GPP TS 32.290 [11] which includes the attributes that shall be used for Satellite Backhaul.

Table 5.2.4.1.X-1: Common Data structure of Charging Data Request with Satellite Backhaul Identification(3GPP TS 32.290 [11] – Table 7.1)

| Information Element | Converged Charging Category | Offline Only Charging Category | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | This field identifies the charging session. |
| Subscriber Identifier | OM | OM | This field contains the identification of the individual subscriber that uses the requested service. |
| NF Consumer Identification | M | M | This is a grouped field which contains a set of information identifying the NF consumer of the charging service. |
| NF Functionality | M | M | This field contains the function of the node. |
| NF Name | OC | OC | This fields holds the name (i.e. UUID) of the NF consumer. At least one of the NF Address or NF Name shall be present. |
| NF Address | OC | OC | This field holds the address (i.e. IP address and/or FQDN) of NF consumer. At least one of the NF Address or NF Name shall be present. |
| NF PLMN ID | OC | OC | This field holds the PLMN ID of the network the NF consumer belongs to. |
| Satellite Backhaul Information | OC | OC | This field contain parameters that can be used to determine that a Satellite Backhaul has been used for the data traffic |
| Satellite Backhaul Category | OC | OC | Field contains information on the backhaul category that is used to support the Satellite PDU Session, possibly together with a starting time and/or an end time for the usage of the backhaul category |
| Observed Satellite backhaul delay | OC | OC | Optional information indicating the one way delay taken by data transfer through the Satellite backhaul, , possibly together with a starting time and/or an end time for the period of time when this Satellite backhaul delay applied |
| Satellite QoS | OC | OC | This field may be added optionally as a further enhancement to contain the result of the calculation of the following attributes (throughput), latency, jitter, and error rate) |
| Charging Identifier | OM | - | This field contains the charging identifier allowing correlation of charging information. Only applicable if not provided in the NF (CTF) consumer specific structure. |
| Invocation Timestamp | M | M | This field holds the timestamp of the charging service invocation by the NF consumer |
| Invocation Sequence Number | M | M | This field contains the sequence number of the charging service invocation by the NF consumer in a charging session. |
| Retransmission Indicator | OC | OC | This field indicates if included, this is a retransmitted request message. |
| One-time Event | OC | - | This field indicates, if included, that this is event based charging and whether this is a one-time event in that there will be no update or termination. |
| One-time Event Type | OC | - | This field indicated the type of the one time event, i.e. Immediate or Post event charging. |
| Notify URI | OC | - | This field contains URI to which notifications are sent by the CHF. The latest received value shall always be used at notifications. |
| Supported Features | OC | - | This field indicates the features supported by the NF consumer. |
| Service Specification Information | OC | - | This field identifies the technical specification for the service (e.g. TS 32.255) and release version (e.g. Release 16) that applies to the request. It is for information. |
| Triggers | OC | OC | This field identifies the event(s) triggering the request and is common to all Multiple Unit Usage occurrences. |
| Multiple Unit Usage | OC | OC | This field contains the parameters for the quota management request and/or usage reporting. It may have multiple occurrences. |
| Rating Group | M | M | This field holds the identifier of a rating group. |
| Requested Unit | OC | - | This field indicates that quota management is required, and may contain the amount of requested service units for a particular category. If it is included then it indicates that quota management is required for the Rating Group or, in the case of service level reporting, at least one of the services in the Rating Group. If it is not included and at least one Used Unit Container included have Qquota Management Indicator set to onlineONLINE then it indicates that this is the last request for the Rating Group or, in the case of service level reporting, all online charging services in the Rating Group. |
| Time | OC | - | This field holds the amount of requested time. |
| Total Volume | OC | - | This field holds the amount of requested volume in both uplink and downlink directions. |
| Uplink Volume | OC | - | This field holds the amount of requested volume in uplink direction. |
| Downlink Volume | OC | - | This field holds the amount of requested volume in downlink direction. |
| Service Specific Units | OC | - | This field holds the amount of requested service specific units. |
| Used Unit Container | OC | OC | This field contains the amount of used non-monetary service units measured up to the triggers and trigger timestamp. It may have multiple occurrences. |
| Service Identifier | OC | OC | This field holds the Service Identifier. |
| Quota management Indicator | OC | - | This field holds an indicator on whether the reported used units are with quota management control, without quota management control or with quota management control temporary suspended. If the field is not present, it indicates the used unit is without quota management applied. |
| Triggers | OC | Oc | This field holds reason for charging information reporting or closing for the used unit container. |
| Trigger Timestamp | OC | OC | This field holds the timestamp of the trigger. |
| Time | OC | OC | This field holds the amount of used time. |
| Total Volume | OC | OC | This field holds the amount of used volume in both uplink and downlink directions. |
| Uplink Volume | OC | OC | This field holds the amount of used volume in uplink direction. |
| Downlink Volume | OC | OC | This field holds the amount of used volume in downlink direction. |
| Service Specific Unit | OC | OC | This field holds the amount of used service specific units. |
| Event Time Stamps | OC | OC | This field holds the timestamps of the event reported in the Service Specific Units, if the reported units are event based. |
| Local Sequence Number | OM | OM | This field holds the container sequence number. |

End of changes