3GPP TSG-SA5 Meeting #150	S5-235188

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0446 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Addition of measurement for optimization of the SN initiated inter-SN CPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With 3GPP Rel-18 the SCG mobility was extended to Conditional PSCell Change (CPC). In case of CPC Execution to wrong PSCell related to SN initiated inter- SN CPC the root cause can be on Source SN (S-SN), target SN (T-SN) or in MN side. For SN initiated inter-SN CPC the Master Node (MN) inttiates SN Addition procedure to each candidate T-SN to prepare the list of candidate PSCells selected by the S-SN. After that MN provides CPC configuration to UE. For fast moving UEs when due to extensive delay in configuring the CPC to UE the list of candidate PSCells selected by the S-SN may be outdated. For proper identification of an issue in MN side it is important to measure “Average Time Interval for Preparation of the SN initiated inter-SN CPC” to optimize the list of candidate PSCells in S-SN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of measurements for “Rel-18 CR TS 28.552 Addition of measurement for “Average Time Interval for Preparation of the SN initiated inter-SN CPC” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator cannot properly optimize the list of candidate PSCells selected by S-SN for SN initiated inter-SN CPC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.x (new), A.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

###### 5.1.1.6.x Average Time Interval for Preparation of the SN initiated inter-SN CPC

###### a)  This measurement provides the average time interval needed for preparation of the SN initiated inter-SN   Conditional PSCell Change (CPC).

b)	CC

c)	Average of samples from the observation interval where each one represents interval between the point in time the list of candidate PSCells is selected by the Source SN (S-SN) for the candidate Target SN(s) (T-SN) which so followed with sending XnAP: S-NODE CHANGE REQUIRED Required message from the S-SN to MN and communicating CPC configuration to UE triggered on reception of the XnAP: S-NODE CHANGE CONFIRM from MN (chapter 10.5.2 in the 3GPP TS 37.340). The measurement is done in S-SN per the MN.

d)	Each measurement is a real representing the mean delay in 1 millisecond. The measurement is provided per S-SN and MN pair.

e)	Avg.Time.SNin.InterSNCPC

f)	GNBCUUPFunction Relation

g)	Valid for packet switched traffic.

h)	5GS.

End of Modified Section

2nd Modified Section

A.y Inter SN CPC preparation related measurements

With 3GPP Rel-18 the SCG mobility was extended to Conditional PSCell Change (CPC). In case of CPC Execution to wrong PSCell related to SN initiated inter- SN CPC the root cause can be on Source SN (S-SN), target SN (T-SN) or in MN side.

For SN initiated inter-SN CPC the Master Node (MN) inttiates SN Addition procedure to each candidate T-SN to prepare the list of candidate PSCells selected by the S-SN. After that MN provides CPC configuration to UE. For fast moving UEs when due to extensive delay in configuring the CPC to UE the list of candidate PSCells selected by the S-SN may be outdated.

For proper identification of an issue in MN side it is important to measure “Average Time Interval for Preparation of the SN initiated inter-SN CPC” to optimize the list of candidate PSCells in S-SN.

End of Modified Section