3GPP TSG-SA5 Meeting #150	S5-235241

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0449 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add MA PDU session creation measurements in HR roaming scenario for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd.,Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MA PDU sessions are created in two scerarios; Non-roaming/LBO-roaming and HR-roaming, and are created by (V-)SMF and H-SMF respectively. Therefore, it is necessary to monitor the performance of MA PDU session establishment in HR-roaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MA PDU session creation measurements in HR roaming scenario including: - Number of MA PDU session creation requests in HR roaming scenario - Number of successful MA PDU session creations in HR roaming scenario - Number of failed MA PDU session creations in HR roaming scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the performance of ATSSS and effectively solve the corresponding network problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.x(new), 5.3.1.y(new),5.3.1.z(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

5.3.1.x	Number of MA PDU session creation requests in HR roaming scenario

a)	This measurement provides the number of MA PDU sessions requested to be created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On receipt by the H-SMF from V-SMF of Nsmf_PDUSession_Create Request indicating an "MA PDU Request" (see TS 23.502 [7]). Each MA PDU session requested to be created is added to the relevant subcounter per S-NSSAI .

d)	Each subcounter is an integer value

e)	SM.MAPDUSessionCreationHRroam.SNSSAI

Where SNSSAI identifies the S-NSSAI;

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

5.3.1.y	Number of successful MA PDU session creations in HR roaming scenario

a)	This measurement provides the number of MA PDU sessions successfully created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On transmission by the H-SMF to V-SMF of Nsmf_PDUSession_Create Response indicating the requested MA PDU session creation is accepted (see TS 23.502 [7]). Each MA PDU session successfully created is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value

e)	SM.MAPDUSessionCreationHRroamSucc.SNSSAI

Where SNSSAI identifies the S-NSSAI;

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

5.3.1.z	Number of failed MA PDU session creations in HR roaming scenario

a)	This measurement provides the number of MA PDU sessions failed to be created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On transmission by the H-SMF to V-SMF of Nsmf_PDUSession_Create Response that indicates a failure or rejected MA PDU session creation (see TS 29.502 [14]). Each MA PDU session failed or rejected to be created is added to the relevant subcounter per rejection cause.

d)	Each subcounter is an integer value

e)	SM.MAPDUSessionCreationHRroamFail.cause

Where cause indicates the rejection cause for the MA PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

End of change