3GPP TSG-SA5 Meeting #150	S5-235258

# Goteborg, Sweden, 21 Aug 2023 – 25 Aug 2023                          Revision of S5-233253

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.257 | CR | 0007 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 32.257 Edge Infrastructure Usage Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Amdocs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_CH |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The mapping of edge infrastructure usage data volumes to Nchf Charging Data Request is unclear |  |  |  |  |  |  |  |  |
| Summary of change: |  | The data volumes for edge infrastructure resource usage to be specified in Edge Infrastructure Resource Usage Charging Information container instead of Multiple Unit Usage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There can be confusion on the implementation of EAS Infrastructure Usage Charging events between CHF and consumers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1.2, 6.1.2.1.2, 6.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.291 CR 0468 TS 32.298 CR 0931 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-233253 |  |  |  |  |  |  |  |  |

First change

##### 6.1.1.1.2	Charging Data Request message

Table 6.1.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the CEF as used for edge enabling infrastructure resource usage converged charging.

Table 6.1.1.1.2-1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | - | This field is not applicable. |
| Subscriber Identifier | - | This field is not applicable. |
| NF Consumer Identification | M | Described in 3GPP TS 32.290 [6]. |
| NF Functionality | M | Described in 3GPP TS 32.290 [6]. |
| NF Name | OC | Described in 3GPP TS 32.290 [6]. |
| NF Address | OC | Described in 3GPP TS 32.290 [6]. |
| NF PLMN ID | OC | Described in 3GPP TS 32.290 [6]. |
| Charging Identifier | OM | Described in 3GPP TS 32.290 [6]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [6]. |
| Invocation Sequence Number | - | This field is not applicable. |
| Retransmission Indicator | OC | Described in 3GPP TS 32.290 [6]. |
| One-time Event | OC | Described in 3GPP TS 32.290 [6]. |
| One-time Event Type | OC | Described in 3GPP TS 32.290 [6]. |
| Notify URI | - | This field is not applicable. |
| Supported Features | OC | Described in 3GPP TS 32.290 [6]. |
| Service Specification Information | OC | Described in 3GPP TS 32.290 [6]. |
| Triggers | OC | Described in 3GPP TS 32.290 [6]. |
| Multiple Unit Usage | - | This field is not applicable. |
| EAS ID | OM | This field holds the EAS ID, see 3GPP TS 23.558 [9]. |
| EDN ID | OM | This field holds the DN of EdgeDataNetwork MOI, see 3GPP TS 28.538 [12]. |
| EAS Provider Identifier | OM | The identifier of the ASP that provides the EAS, see 3GPP TS 23.558 [9]. |
| Edge Enabling Infrastructure Resource Usage Charging Information | OM | This field holds the for edge enabling infrastructure resource usage charging specific information described in clause 6.1.2.1.2. |

Second change

##### 6.1.2.1.2	Definition of edge enabling infrastructure resource usage specific charging information

Specific charging information used for edge enabling infrastructure resource usage charging is provided within the Edge Enabling Infrastructure Resource Usage Charging Information.

The detailed structure of the Edge Enabling Infrastructure Resource Usage Charging Information can be found in table 6.1.2.1.2-1.

Table 6.1.2.1.2-1: Structure of Edge Enabling Infrastructure Resource Usage Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Mean Virtual CPU Usage | OM | This field holds the information of mean virtual CPU usage for the EAS, see VR.VCpuUsageMean in clause 5.7.1.1.1 of 3GPP TS 28.552 [13]. |
| Mean Virtual Memory Usage | OM | This field holds the information of mean virtual memory usage for the EAS, see VR.VMemoryUsageMean in clause 5.7.1.2.1 of 3GPP TS 28.552 [13]. |
| Mean Virtual Disk Usage | OM | This field holds the information of mean virtual disk usage for the EAS, see VR.VDiskUsageMean in clause 5.7.1.2.1 of 3GPP TS 28.552 [13]. |
| Incoming Data Volume | OM | This field holds the information of number of incoming bytes received by the EAS, see DataVolum.InBytesEAS in clause 5.7.2.1 of 3GPP TS 28.552 [13] |
| Outgoing Data Volume | OM | This field holds the information of number of outgoing bytes transmitted from the EAS, see DataVolum.OutBytesEAS in clause 5.7.2.2 of 3GPP TS 28.552 [13] |
| Duration Start Time | OM | This field holds the start time of the collection period, see 3GPP TS 28.550 [14]. |
| Duration End Time | OM | This field holds the end time of the collection period, see 3GPP TS 28.550 [14]. |

Third change

#### 6.1.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by CEF for edge enabling infrastructure resource usage converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.1.2.3-1 defines the basic structure of the supported fields in the Charging Data Request message for edge enabling infrastructure resource usage converged charging.

Table 6.1.2.3-1: Supported fields in Charging Data Request message

| Information Element | Functionality of CEF | Edge enabling infrastructure resource usage charging |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | - |
| Subscriber Identifier |  | - |
| NF Consumer Identification |  | E |
| NF Functionality |  | E |
| NF Name |  | E |
| NF Address |  | E |
| NF PLMN ID |  | E |
| Charging Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Sequence Number |  | - |
| Retransmission Indicator |  | E |
| One-time Event |  | E |
| One-time Event Type |  | E |
| Notify URI |  | - |
| Supported Features |  | E |
| Service Specification Information |  | E |
| Triggers |  | E |
| Multiple Unit Usage |  | - |
| EAS ID |  | E |
| EDN ID |  | E |
| EAS Provider Identifier |  | E |
| Edge Enabling Infrastructure Resource Usage Charging Information |  | E |

Table 6.1.2.3-2 defines the basic structure of the supported fields in the Charging Data Response message for edge enabling infrastructure resource usage converged charging.

Table 6.1.2.3-2: Supported fields in Charging Data Response message

| Information Element | Functionality of CEF | Edge enabling infrastructure resource usage charging |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Result |  | E |
| Invocation Result Code |  | E |
| Failed parameter |  | E |
| Failure Handling |  | E |
| Invocation Sequence Number |  | - |
| Session Failover |  | - |
| Supported Features |  | E |
| Triggers |  | - |
| Multiple Unit Information |  | - |
| Result Code |  | - |
| Rating Group |  | - |
| Granted Unit |  | - |
| Tariff Time Change |  | - |
| Time |  | - |
| Total Volume |  | - |
| Uplink Volume |  | - |
| Downlink Volume |  | - |
| Service Specific Units |  | - |
| Validity Time |  | - |
| Final Unit Indication |  | - |
| Time Quota Threshold |  | - |
| Volume Quota Threshold |  | - |
| Unit Quota Threshold |  | - |
| Quota Holding Time |  | - |
| Triggers |  | - |

End of changes