3GPP TSG-SA5 Meeting #150	S5-235274

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0072 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR for TS28.535 Fix invalid clause and wrong description of requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  | Date: |  | 2023-08-10 |
| Category: | A |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |

| Reason for change: | clause 4.1 is void in the specification, should be 4.2.4.2 |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Fix invalid clause and wrong description of requirements |  |  |  |
| Consequences if not approved: | Invalid reference causes confusing results |  |  |  |
| Clauses affected: | 4.2.1,6.2 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

Start of changes

### 4.2.1	Overview

For communication service assurance one can identify two interactions of management control loops:

1)	Between the CSC and the CSP: In this case, the CSC provides the requirements for an assured communication service to the CSP, the CSP provides the corresponding communication service, the CSP also provides feedback to the CSC. The CSP adjusts the resources used by a communication service or the CSC adjusts the SLS continuously to achieve the assured requirements.

2)	Between the CSP and the NSP: the communication service provided by CSP requires the network capabilities. For example, the CSP requires a certain network latency. The NSP management system adjusts the network or CSP adjusts the latency requirement continuously to satisfy the latency requirement.

Figure 4.2.1.1: Communication service closed control loop assurance

Figure 4.2.1.1 gives a high level description of interaction process involved in the management closed control loop.

Generally, the management control loop for CSA consists of the steps Monitoring, Analysis, Decision and Execution. The adjustment of the resources used for the communication service is completed by the continuous iteration of the steps in a management control loop. As described in clause 4.2.4.2, the management closed control loop for the resources used for the communication service is deployed in the preparation phase and takes effect during the preparation phase and operation phase.

Figure 4.2.1.2 shows the overall process of communication service assurance using a management control loop.

Figure 4.2.1.2: Management Control Loop

The Second Change

## 6.2	Requirements

REQ-CSA-CON-01 The 3GPP management system shall have the capability to take actions for a set of communication services serving certain group of UEs based on the target SLS.

REQ-CSA-CON-02 The 3GPP management system shall have the capability to collect service experience information.

REQ-CSA-CON-03 The 3GPP management system shall have the capability to analyse the performance information related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-04 The 3GPP management system shall have the capability to modify the configuration parameters related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-05	The 3GPP management system shall have the capability to collect NSI related data from one or more 5GC NF(s).

NOTE 1:	An example for NSI related data may be QoE data.

REQ-CSA-CON-06	The 3GPP management system shall have the capability to derive which communication service is associated to the QoE data from the collected NSI related QoE data.

NOTE 1a: A communication service in the 3GPP management system is identified by an S-NSSAI (the Slice/ServiceType, SST in the S-NSSAI identifies a communication service which can be detailed using the SliceDifferentiattor, SD), see TS 23.003 [10].

REQ-CSA-CON-07	The 3GPP management system shall have the capability to ascertain SLS breach.

REQ-CSA-CON-08	The 3GPP management system shall have the capability to perform the root cause analysis (e.g., identifying the underlying reason) for an SLS breach.

REQ-CSA-CON-09	The 3GPP management system shall have the capability to take corrective actions to ensure the target goal.

REQ-CSA-CON-10 The 3GPP management system shall have the capability to translate network slice requirements to cross domain network slice subnet SLS goal and single domain network slice subnet SLS goal.

REQ-CSA-CON-11 The 3GPP management system shall have the capability to collect single domain SLS analysis as input to cross domain SLS analysis.

REQ-CSA-CON-12	The 3GPP management system shall have the capability to allow its authorized consumer to control the SLS assurance (e.g. specify the SLS to be assured, enable/disable, specify the assurance time and update the SLS assurance requirements).

REQ-CSA-CON-13	The 3GPP management system shall have the capability to allow its authorized consumer to obtain the SLS assurance fulfilment status information.

NOTE 2:	The management system refers to the producer of management service for SLS assurance.

REQ-CSA-CON-14	The 3GPP management system shall have the capability to do network prediction (e.g. network resource usage and network performance) by analysing the network operation information in special scenarios.

REQ-CSA-CON-15	The 3GPP management system shall have the capability to take actions such as network configuration and perform network resource reallocation according to the network prediction results.

REQ-CSA-CON-16	The 3GPP management system shall have the capability to allow its authorized consumer to limit the set of action capabilities executable by an assurance closed loop.

NOTE: This use case is not supported.

REQ-CSA-CON-17	The 3GPP management system shall allow an authorized consumer to set a condition to enable/disable an ACCL.

NOTE: This use case is not supported.

REQ-LCM-CON-01 The 3GPP management system shall have the capability of lifecycle management of a closed control loop.

REQ-CSA-CON-18 The 3GPP management system shall have the capability to configure SLS assurance goals for the 5GC management domain and the NG-RAN management domain.

REQ-CSA-CON-19 The 3GPP management system shall have the capability to allow closed control loops in cross management domain to collect SLS assurance goal status of closed control loops in 5GC management domain and NG-RAN management domain.

REQ-CSA-CON-20	The 3GPP management system shall have the capability to allow an authorized consumer to enable or disable pause point(s) connected to actions on managed entities during the ACCL’s execution phase and prior to the actions’ execution.”

NOTE: This requirement is not supported.

REQ-CSA-CON-21 The 3GPP Management System shall have the ability to provide SLS assurance within a particular location.

End of changes