3GPP TSG-SA5 Meeting #150 	S5-235281

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0200 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance the description in clause 5.1.17 creation of a 3GPP NF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCVNF |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 28.834 has been appoved is 3GPP SA#100 meeting. It presents use cases and relevant solutions related to the management of cloud-native virtualized network function, in terms of life cycle management, PM, FM and CM of cloud native VNF. TS 28.531 is recommended to enhanced to support the LCM operation of cloud native VNF. For the procedure of 3GPP NF creation , because the 3GPP VNF(s) consitituting a NS should include both VM-based VNF(s) and cloud native VNF(s).the 3GPP management system shall have capabilities to interact with ETSI NFV MANO for creation of NF as both a containerised VNF and a virtual machine based VNF. Meanwhile, VNF package of the virtualized part of 3GPP NF is used for describing virtual machine resources of VM-based VNF or containerized resources of cloud native VNF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The assumptions is that 3GPP VNF(s) consitituting a NS can be the VM-based VNF(s), cloud native VNF(s) or both. VNF package of the virtualized part of 3GPP NF is used for describing virtual machine resources of VM-based VNF or containerized resources of cloud native VNF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is explicit lack of supporting cloud native VNF in usecases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specificationst |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.17	Creation of a 3GPP NF

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request creation of an instance of 3GPP NF. |  |
| Actors and Roles | An authorized consumer of the NF creation provisioning management service. |  |
| Telecom resources | VNF package(s) of the virtualized part of 3GPP NF used for description of virtual machine resources or containerized resources; ETSI NFV MANO system; NF provisioning management service producer. |  |
| Assumptions | The 3GPP VNF(s) consitituting a NS can be the VM-based VNF(s), cloud native VNF(s) or both. |  |
| Pre-conditions | The VNF package(s) of the virtualized part of 3GPP NF have been on-boarded to ETSI NFV MANO system. |  |
| Begins when | The authorized consumer needs to create a new instance of 3GPP NF (e.g. VM-based VNF, cloud native VNF). |  |
| Step 1 (M) | The authorized consumer requests the NF provisioning management service producer to create a new instance of 3GPP NF. |  |
| Step 2 (M) | The NF provisioning management service producer checks the subject 3GPP NF contains virtualized part and/or non-virtualized part. If it contains virtualized part, then the NF instantiation service producer performs the step 3 and 4 to instantiate the virtualized part of the subject 3GPP NF. How to instantiate the non-virtualized part of the subject 3GPP NF is out of scope of present specification. |  |
| Step 3 (M) | The NF provisioning management service producer interacts, or requests another NF provisioning management service producer to interact, with ETSI NFV MANO system to instantiate the VNF(s) that are realizing the virtualized part of subject 3GPP NF. |  |
| Step 4 (M) | If all of the contained parts (i.e., virtualized part and non-virtualized part if any) of the 3GPP NF have been successfully instantiated, the NF provisioning management service producer informs the consumer(s) (who have subscribed to the notifications for NF creation) that the instance of 3GPP NF has been created, and creates the MOI(s) for the subject 3GPP NF. |  |
| Step 5 (M) | Created MOI(s) may be maintained by a Management Function which has the NF provisioning management service or the 3GPP NF. When the MOI(s) is maintained by the 3GPP NF, the NF provisioning management service producer sends a request of creating the MOI(s) to the corresponding NF provisioning management service producers in the created NF. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The instance of 3GPP NF has been created, and the MOI(s) of the 3GPP NF have been created. |  |
| Traceability | REQ-PRO_NF-FUN-1, REQ-PRO_NF-FUN-2, REQ-PRO_NF-FUN-7 |  |

End of changes