3GPP TSG-SA5 Meeting #150	S5-235293

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0079 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR for TS28.550 editorial Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | editorial Corrections |  |  |  |  |  |  |  |  |
| Summary of change: |  | editorial Corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tow many typos |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.4.1,5.1.2.4.2,5.1.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

##### 5.1.1.4.1	Creation of threshold monitoring for NF performance measurements

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request creation of threshold monitoring for NF performance measurements. |  |
| Actors and Roles | An authorized consumer monitoring service for NF performance threshold monitoring. |  |
| Telecom resources | Producer of management service for NF performance threshold monitoring. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NF has been deployed. - The management service for NF performance threshold monitoring is in operation. - The authorized consumer of management service for NF performance threshold monitoring has subscribed to the threshold crossing notifications. |  |
| Begins when | The authorized consumer needs to create performance threshold monitoring for NF performance measurements. |  |
| Step 1 (M) | The authorized consumer requests the management service producer to create performance threshold monitoring for NF performance measurements. The request contains the threshold information with the conditions for triggering the threshold crossing notifications, and the information about the threshold monitoring notification target to receive the notifications. |  |
| Step 2 (O) | The service producer may reject the request, if - the NF performance measurements included in the threshold monitor are not being collected (e.g., by a measurement job or by NRM configurations); or - the NF performance measurements included in the threshold monitor are being collected but with a GP different from the monitoring GP of this request. |  |
| Step 3 (M) | The management service producer for NF performance threshold monitoring requests the NF(s) to monitor the performance measurements. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NF performance threshold monitoring has been created, and the target will receive the threshold crossing notifications when performance threshold is crossed or reached. |  |
| Traceability | REQ-THMS_NF-FUN-1, REQ-THMS_NF-FUN-3 |  |

The Second change

##### 5.1.2.4.2	Termination of threshold monitoring for NSSI performance measurements

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request termination of threshold monitoring for NSSI performance measurements. |  |
| Actors and Roles | An authorized consumer of management service for NSSI performance threshold monitoring. |  |
| Telecom resources | Producer of management service for NSSI performance threshold monitoring. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NSSI has been deployed. - The producer of management service for NSSI performance threshold monitoring is in operation. - The threshold monitoring for NSSI performance measurements has been created for the authorized consumer. |  |
| Begins when | The authorized consumer needs to terminate the performance threshold monitoring for NSSI performance measurements. |  |
| Step 1 (M) | The authorized consumer requests the management service producer to terminate the performance threshold monitoring for NSSI performance measurements. |  |
| Step 2 (M) | The management service producer stops the subject threshold monitoring, and requests the NSSI(s) to stop monitoring the performance measurements. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The threshold monitoring for NSSI performance measurements is stopped. |  |
| Traceability |  |  |

The Third change

##### 5.1.3.4.2	Termination of threshold monitoring for NSI performance measurements

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request termination of threshold monitoring for NSI performance measurements. |  |
| Actors and Roles | An authorized consumer of management service for NSI performance threshold monitoring. |  |
| Telecom resources | Producer of management service for NSI performance threshold monitoring. |  |
| Assumptions | N/A |  |
| Pre-conditions | - The 3GPP NSI has been deployed. - The producer of management service for NSI performance threshold monitoring is in operation. - The threshold monitoring for NSI performance measurements has been created for the authorized consumer. |  |
| Begins when | The authorized consumer needs to terminate the performance threshold monitoring for NSI performance measurements. |  |
| Step 1 (M) | The authorized consumer requests the management service producer to terminate the performance threshold monitoring for NSI performance measurements. |  |
| Step 2 (M) | The management service producer stops the subject threshold monitoring, and requests the NSI(s) to stop monitoring the performance measurements. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The threshold monitoring for NSI performance measurements is stopped. |  |
| Traceability |  |  |

End of changes