3GPP TSG-SA5 Meeting #150	S5-235353

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 ML inference history NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML inference history has been discussed and agreed (in TR 28.908). This CR is to normatively propose the solution for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | stage 2 solution has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the use case “ML inference history” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.V(new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 7.3.4	Information model definitions for ML inference phase

#### 7.3.4.1	Class diagram

##### 7.3.4.1.1	Relationships

@startuml TS 28.105

scale max 300 height

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class MLInference  <<InformationObjectClass>>

class MLEntity << InformationObjectClass >>

class MLInferenceHistoryRequest << InformationObjectClass >>

ManagedEntity "1" *-- "*" MLInference: <<names>>

MLInference "1" -r-> "*" MLEntity

MLInference "1" *-- "*" MLInferenceHistoryRequest: <<names>>

MLEntity "*" <--> "*" MLInferenceHistoryRequest

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

Figure 7.3.4.1-1: NRM fragment for ML InferenceHistory Control

##### 7.3.4.1.2	Inheritance

@startuml TS 28.105 figure 7x-2

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class MLInference << InformationObjectClass >>

class MLInferenceHistoryRequest << InformationObjectClass >>

ManagedFunction <|-- MLInference

Top <|-- MLInferenceHistoryRequest

@enduml

Figure 7.3.4.1.2-1: ML InferenceHistory Inheritance Relations

#### 7.3.4.2	Class definitions

7.3.4.2.1 	MLInference

###### 7.3.4.2.1.1	Definition

MLInference History is a capability for receiving requests and sending the related reports on a particular inference function or its associated ML entities. As such MLInference history should be an extension of  MLInference.

The MLInference) shall be associated with one or more MLModels, one or more MLInferenceHistoryRequests and one or more MLInferenceHistoryReport instances responsible for reporting on MLInferenceHistory.

7.3.4.2.1.2	Attributes

The MLInference IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLInferenceId | M | T | F | F | F |
| Attribute related to role |  |  |  |  |  |
| mLInferenceHistoryRequest | M | T | T | F | T |
| mLInferenceHistoryReport | M | T | F | F | F |

7.3.4.2.K 	MLInferenceHistoryRequest

###### 7.3.4.2.K.1	Definition

This IOC represents a job of requesting a ML inference history record.

For each request to collect ML Inference History, a consumer creates a new MLInferenceHistoryRequest on the MLInferenceHistory, i.e.,  MLInferenceHistoryRequest shall be an information object class that is instantiated for each request for ML Inference History

- Each MLInferenceHistoryRequest is associated to at least one MLEntity

- Each MLInferenceHistoryRequest may contain specific reporting requirements, e.g., for  ReportingPeriod that define how the MLInferenceHistory may report about the MLInferenceHistoryRequest

- The MLInferenceHistoryRequest may be associated with an MLInferenceHistoryRepor,  but it is not necessarily a one-to-one mapping between an instance of MLInferenceHistoryRequest and an MLInferenceHistoryReport since multiple requests could be mapped to a single reporting instance. As such the MLInferenceHistoryRequests could be managed independently of the MLInferenceHistoryReports. The association between MLInferenceHistoryRequest and its corresponding MLInferenceHistoryReport is written into the request by the  MLInference after determining the right MLInferenceHistoryReport to provide reports for the specific request.

7.3.4.2.K.2	Attributes

The MLInferenceHistoryRequest IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLInferenceHistoryRequestID | M | T | F | F | F |
| mLEntity | M | T | F | F | F |
| ReportingPeriod | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLInferenceHistoryReport | M | T | F | F | F |

## 7.4	DataType definition

##### 7.4.A.	MLInferenceHistoryReport <<dataType>>

###### 7.4.A.1	Definition

This dataType represents the properties of MLInferenceHistoryReport.

- MLInferenceHistory may generate one or more MLInferenceHistoryReports,

- Each MLInferenceHistoryReport is associated to one or more MLModels/MLEntitys since the MLInferenceHistoryReports must be associated to the MLInferenceHistoryReport instance which reports about MLInferenceHistoryRequests that themselves are associated to the MLModels/MLEntitys for which InferenceHistory is requested and/or reported.

7.4.A.2	Attributes

The MLInferenceHistoryReport includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLInferenceHistoryReportID | M | T | F | F | F |
| mLEntityList | M | T | F | F | F |
| mLInferenceHistoryRequests | O | T | F | F | F |
| mLinferenceHistoryResults | M | T | F | F | F |

##### 7.4.B 	MLOutcome << dataType >>

###### 7.4.B.1	Definition

This dataType represents the properties of a single MLInferenceOutcome.

7.4.B.2	Attributes

The MLInferenceHistoryReport includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLOutcomeID | M | T | F | F | F |
| mLOutcomeCapability | M | T | F | F | F |
| mLOutcomeContext | M | T | F | F | F |
| mLOutcomeType | M | T | F | F | F |
| mLOutcomeValue | M | T | F | F | F |

## 7.X.4	Attribute definitions

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLInferenceId | It indicates an identifier for a specific instantiated MLInference function instance. Allowed values: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLInferenceHistoryRequest | It indicates the MLInferenceHistory request associated with the MLInference instance. Allowed values: N/A. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLInferenceHistoryRequestID | It indicates the identifier of a specific MLInferenceHistoryRequest . Allowed values: N/A. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEntity | It indicates the identifier of the mlEntity on which inference history is requested. Allowed values: N/A. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| reportingPeriod | It indicates the time period in seconds between instance of reporting on inference history. Allowed values: N/A. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLInferenceHistoryReportID | It indicates the identifier of an MLInferenceHistoryReport instance. Allowed values: N/A. | type: DN (see TS 32.156 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLinferenceHistoryResults | It indicates the results that form the content of MLInferenceHistoryReport instance. Allowed values: N/A. | type: MLOutcome multiplicity: 1 …* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLOutcomeCapability | It indicates the specific ML capability that is being computed in an ML outcome. Allowed values: N/A. | type: MLCapability multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLOutcomeContext | It indicates the context within which the ML outcome was observed. Allowed values: N/A. | type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLOutcomeType | It indicates the data type of the outcomes for the ML capability that is being computed in an ML outcome Allowed values: - REAL - ENUM - STRING - BOOLEAN - DATE_TIME - GEOGRAPHIC_POLYGON - TIME_WINDOW - GEO_COORDINATE | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| mLOutcomeValue | It indicates the values for the ML capability that is being computed in an ML outcome The stringvalue shall be evaluated according to the MLinferenceOutcomeType . Allowed values: N/A. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |

End of Changes