3GPP TSG-SA5 Meeting #150 	S5-235436

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0005 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add solution for management of NPN service customer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This proposal satisfies the requirements from the existing use case - Management of NPN service customer in TS 28.557 clause 5.1.0.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed a procedure for management of tenant which plays the role of NPN service customer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no solution for the use case of management of NPN service customer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, new clause 6.1.x, new clause B.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	5G-ACIA White paper: "5G Non-Public Networks for Industrial Scenarios", July 31, 2019.

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[7]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[9]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[10]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[14]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[17]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[18]	5G-ACIA: Exposure of 5G Capabilities for Connected Industries and Automation Applications, https://5g-acia.org/whitepapers/exposure-of-5g-capabilities-for-connected-industries-and-automation-applications-2/.

[m]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

Next Change

## 6.1	Generic solutions for management of NPN

### 6.1.x	Solution for management of NPN service customer

The NPN service customer (i.e. NPN-SC) is a role defined for NPN management to represent the CSC (e.g. verticals) which consumes communication services for non-public use (see clause 4.2). While in 3GPP management system, tenant represents a group of MnS consumers associated with the management capabilities they are allowed to access and consume (see clause 4.8 of TS 28.533 [m]). Therefore, the management of NPN-SC, which focuses on the granular access control of NPN-SC, is achieved through the management of context information for tenants.

The creation of tenant context is finished during tenant registration procedure. The tenant registrates to NPN-SP/OP for two reasons:

- Enable the authentication and authorization to tenant.

- Enable the tenant to obtain the permitted MnS information (e.g., MnS type and MnS components).

Figure 6.1.x-1 shows the procedure of management of tenant.

Figure 6.1.x-1: Procedure of management of tenant

The pre-condition of the procedure is that the NPN-SC and NPN-SP/OP have reached a business agreement. Tenant provides the vertical information (e.g. human readable name of vertical, subscribed management capabilities exposed to vertical, etc.) to register a vertical to NPN-SP/NPN-OP through a tenant registration request message. This message may be interacted with BSS layer. After specifying the management capabilities for the tenant, BSS layer should forward them to OSS layer to authenticate and authorize the consumption of management capabilities and corresponding managed resources to the tenant.

- MnS provisioning consumer identifies the management capabilities that are allowed to be consumed by the tenant with the help of BSS layer.

- MnS provisioning consumer sends the createMOI request to MnS provisioning producer to create the instances for the identity, role and MnSs permissions of certain tenant, enabling OSS layer to manage tenant context information. For the definition of operation createMOI, see clause 11.1.1.1 of TS 28.532 [14].

- MnS provisioning producer executes the createMOI operations.

- MnS provisioning producer returns operation execution state (e.g., OperationSucceeded or OperationFailed) and the name/value pairs of MOIs’ attributes to access control MnS producer. The name/value pairs of MOIs’ attributes, as tenant context, include the identifier of tenant, credential, permitted MnSs, etc.

Next Change

Annex B (informative):
Plant UML source code

## B.y	Procedure for management of tenant

The following PlantUML source code is used to describe the procedure of management of the tenant, as depicted by Figure 6.1.x-1:

@startuml

skinparam NoteBackgroundColor White

participant "Provisioning MnS Consumer" as cmer

participant "Provisioning MnS Producer" as prov

rnote over cmer

1.Identify the management capabilities

that can be consumed by the tenant

endrnote

cmer -> prov: 2.create MOI for tenant management

prov -> prov: 3.execute the request

prov -> cmer: 4.return MOI creation results

@enduml

End of Change