3GPP TSG-SA5 Meeting #150 	S5-235462

Göteborg, Sweden, 21-25 August 2023

# Source:	Huawei

# Title:	Resolve editor's note in solution #4

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

Correction on the solutions description.

2	References

[1] 		TR 28.839: "Study on charging aspects for time sensitive networking".

3	Rationale

To address the editor’s note in solution #4, regarding the trigger used for UE CCS and TSN CCS.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.839 [1].

First change

7.4.2	Architecture description

Figure 7.4.2-1: SMF converged charging architecture

"UE CCS" performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

"TSN CCS" performs charging functionalities for TS-SC, and includes in particular:

-	CHF exposing Charging service(s) associated to the TS-SC.

Nchf interface is used for interaction between CHF (UE CCS) and CHF (TSN CCS).

Deployments can choose to collocate TSN CCS with UE CCS.

7.4.3.1	TSN Bridge/End station to TSN Bridge/End station of the Time Sensitive Communication

The figure 7.4.3.1-1 describes the high-level charging procedure for TSN Bridge/End station to TSN Bridge/End station of the Time Sensitive Communication based on SMF during the PDU session.

Figure 7.4.3.1-1: TSC for TSN Bridge/End station to TSN Bridge/End station

  - UE 1 and UE 2 performs the Time Sensitive Communications on the same 5GS bridge.

4. 	UPF sends the usage reporting for the Time sensitive communications.

5ch-a.	The SMF sends Charging Data Request [Initial] to the UE CHF for the TSN stream delivery with the 5GS Bridge id and related 5GS bridge information based on the trigger event specified in the TS 32.255[X].

5ch-b. The UE CHF creates a CDR.

5ch-c. The UE CHF acknowledges by sending Charging Data Response [Initial] to the SMF.

6ch-a.	If traffic data volume is related to the TSC, for example 5G Bridge ID,  the UE CHF sends Charging Data Request [Initial] to the TSN CHF for the TSN stream delivery with the 5GS Bridge id and related 5GS bridge information.

6ch-b. The TSN CHF creates a CDR for the start of the TSN stream.

6ch-c. The TSN CHF acknowledges by sending Charging Data Response [Initial] to the UE CHF.

For Time Sensitive Communications service, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [X].

TSN CCS or billing domain aggregates the volume usage per 5GS bridge via 5GS bridge Id for TSC based on the accumulation of usage reporting via the 5GS bridge.

End of change