3GPP TSG-SA5 Meeting #150 	S5-235535

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0458 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New performance measurements for the UE assistance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently there is no performance measure defined in TS 28.552 quantifying the utilization of this feature in both network and UE side. In this CR, it is proposed to define the number of RRCReconfiguration for configuring UE assistance information as well as number of received UE assistance information. This performance measure is used to derive UAI enablement and utilization per connection KPIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new performance measure capturing the number of RRCReconfiguration for configuring UE assistance information as well as number of received UE assistance information, where both performance measures are split into subcounters for each UAI feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this performance measure, average number of UAI enablement per connection and average number of UAI utilization per connection (TS 28.554 (CR0136) cannot be derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.y (new), 5.1.1.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 28.554 CR0136 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

###### 5.1.1.y	Number of RRCReconfiguration for configuring UE assistance information

a)	This measurement provides the number of RRCReconfiguration carrying the UAI configuration for an RRC connection established within the existing NRCellCU. The measurement is split into subcounters for each UAI configuration type.

b)	CC

c)	On transmission of RRCReconfiguration message to the UE carrying otherConfig (see TS 38.331 Clause 5.3.5.9), if the message contains 1) setup of a UAI configuration or 2) enablement of a UAI feature by setting the corresponding value to “true”, the corresponding subcounters will be incremented. The existing UAI features are include: overheatingAssistanceConfig, idc-AssistanceConfig, drx-PreferenceConfig, maxBW-PreferenceConfig, maxCC PreferenceConfig, maxMIMO-LayerPreferenceConfig,minSchedulingOffsetPreferenceConfig, releasePreferenceConfig, referenceTimePreferenceReporting, btNameList, wlanNameList, sensorNameList, obtainCommonLocation, sl-AssistanceConfigNR, ul-GapFR2-PreferenceConfig, musim-GapAssistanceConfig, musim-LeaveAssistanceConfig, successHO-Config, maxBW-PreferenceConfigFR2-2, maxMIMO-LayerPreferenceConfigFR2-2, minSchedulingOffsetPreferenceConfigExt, rlm-RelaxationReportingConfig, bfd-RelaxationReportingConfig, scg-DeactivationPreferenceConfig, rrm-MeasRelaxationReportingConfig, propDelayDiffReportConfig

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.OtherConfig.feature, where feature identifies the UAI feature which has been supported by the network as listed above.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the UAI enablement.

# >>>>NEXT CHANGE<<<<

###### 5.1.1.z	Number of received UE assistance information

a)	This measurement provides the number of UEAssistanceInformation messages received by the network for an RRC connection established within the existing NRCellCU (see TS 38.331 Clause 5.7.4.1). Note that for each RRC connection the network may receive multiple UAI messages for each feature. The measurement is split into subcounters for each UAI request.

b)	CC

c)	Upon reception of UEAssistanceInformation message by the network, if the message contains an update to a UAI feature, the corresponding subcounter will be incremented. The existing UAI features are as follows:

OverheatingAssistance related features:  reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedBW-FR2-2-DL,reducedBW-FR2-2-UL, reducedMIMO-LayersFR1-DL,reducedMIMO-LayersFR1-UL, reducedMIMO-LayersFR2-DL,reducedMIMO-LayersFR2-UL, reducedMIMO-LayersFR2-2-DL,reducedMIMO-LayersFR2-2-UL

Other UEAssistanceInformation related features:  idc-Assistance, drx-Preference, maxBW-Preference, maxCC-Preference, maxMIMO-LayerPreference, minSchedulingOffsetPreference, releasePreference, sl-UE-AssistanceInformationNR, referenceTimeInfoPreference, ul-GapFR2-Preference, musim-Assistance, overheatingAssistance, maxBW-PreferenceFR2-2, maxMIMO-LayerPreferenceFR2-2, minSchedulingOffsetPreferenceExt, rlm-MeasRelaxationState, bfd-MeasRelaxationState, nonSDT-DataIndication, scg-DeactivationPreference, uplinkData, rrm-MeasRelaxationFulfilment, propagationDelayDifference

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.UAI.feature, where feature denotes the corresponding UAI feature

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the UAI utilization.

# >>>>END OF CHANGES<<<<