3GPP TSG-SA5 Meeting #150	S5-235539

Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 28.623 | CR | 0262 | rev | - | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.623 add area scope in QMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in RAN3 TS 38.413 (R3-233430), the “UE Application Layer Measurement Configuration Information” message contains “CHOICE Area Scope of QMC”, that is, the RAN can collect QoE measurement data of a single UE in a certain area. Therefore, in the signalling based QMC procedure, the areascope should be included in QMCJob. As defined in RAN3 TS 38.413 (R3-233430), area scope include cell based, TA based and TAI based. Therefore, the definition of areascope should include these three types. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change Support Qualifier of areascope to “M” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would be unclear to perform signalling based QMC in specific area. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

D.2.14	module _3gpp-common-qmcjob.yang

<CODE BEGINS>

module _3gpp-common-qmcjob {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-qmcjob";

prefix "qmc3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-trace { prefix trace3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Quality of Experience Measurement Collection Job handling";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2023-02-14 { reference "CR-0234"; }

revision 2022-11-04 { reference "CR-0215"; }

revision 2022-08-17 { reference "CR-0185"; }

revision 2022-04-29 { reference "CR-0171"; }

revision 2022-03-22 { reference "draftCR"; }

revision 2022-01-06 { reference "CR-0142"; }

grouping QMCJobGrp {

leaf serviceType {

description "Specifies an end user service type for QoE measurements.";

mandatory true;

type enumeration {

enum DASH;

enum MTSI;

enum VR;

}

}

list areaScope {

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area where data shall be collected.

List of eNB/list of gNB/eNB/gNB for RLF or RCEF.

List of cells or Tracking Area for QMC.

Cell, TA, LA, RA are mutually exclusive.";

reference "Clause 5.10.2 of 3GPP TS 32.422";

uses trace3gpp:AreaScopeGrp;

}

leaf qoECollectionEntityAddress {

type inet:ip-address;

mandatory true;

description "Specifies the address to which the QMC records shall be

transferred.  Ipv4 or Ipv6 address(es) may be used.";

}

list pLMNTarget {

max-elements 1;

description "Defines the PLMN for which sessions shall be selected in

the network request session in case of area based QMC when several

PLMNs are supported in the RAN (this means that shared cells and not

shared cells are allowed for the specified PLMN. Furthermore, several

PLMNs can be used for not shared RAN cases as well as for shared RAN

cases.). Only the sessions may be selected where the PLMN that the

UE reports as selected PLMN is the same as the PLMN Target.";

uses types3gpp:PLMNId;

key "mcc mnc";

}

leaf qoETarget {

type string;

description "Specifies the target object of the QMC in case of

signalling based QMC.  The qoETarget attribute shall be able

to carry IMSI or SUPI.";

mandatory true;

}

leaf qoEReference {

type string;

mandatory true;

description "Identifies the QoE measurement collection job in the

Managed Elements and in the measurement collection entity.  The QoE

reference shall be globally unique therefore it is composed as follows:

MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC

activation request from the management system to identify one PLMN

containing the management system, and QMC ID is a 3 byte Octet String.

The QMC ID is generated by the management system or the operator.";

}

leaf jobId {

type string;

description "The jobId attribute presents the job identifier of a

QMCJob instance. The jobId can be used to associate  multiple QMCJob

instances. For example, it is possible to configure the same jobId

for multiple QMCJob instances required to produce the data

(e.g. Streaming services and MTSI reports) for a specific network

analysis.";

}

list sliceScope {

description "Contains a list of S-NSSAIs (Single Network Slice Selection

Assistance Information).  A Network Slice end to end is identified by

S-NSSAI.";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf qMCConfigFile {

type string;

description "Provides a reference to a file including the parameters for

configuration of application layer measurements, known as Container

for Application Layer Measurement Configuration.";

mandatory true;

}

list mDTAlignmentInformation {

key "idx";

min-elements 1;

max-elements 1;

description "This parameter indicates the MDT measurements with which

alignment of QoE measurement is required. This parameter is optional

and is valid for NR only.";

leaf idx { type uint32 ; }

uses trace3gpp:TraceReferenceGrp ;

}

leaf-list availableRANqoEMetrics {

type enumeration {

enum appLayerBufferLevelList;

enum playoutDelayForMedia;

enum Startup;

}

max-elements 2;

description "This parameter indicates available RAN visible QoE metrics

to the gNB. This parameter is optional and is valid for NR only.";

}

}

grouping QMCJobSubTree {

description "Helps with augmenting QMCJob into multple places.";

list QMCJob {

description "The QoE Measurement Collection provides capability for

collecting QoE information from:

- UEs which are in the specified area in case of Management Based

Activation or

- an individual UE in case of Signalling Based Activation.

The QoE Measurement Collection enables collection of application layer

measurements from the UE for specified end user service type.

The supported service types are:

- Streaming services, see TS 26.247

- MTSI services, see TS 26.114

- VR services, see TS 26.118

A QMCJob instance represents the job for collecting QoE measurements

according to the job parameters. For details of the QoE measurement

collection configuration parameters see clause 5 of TS 28.405.

QMCJob instance can be name-contained by SubNetwork or ManagedElement.

A QMC Job is activated by creating a QMCJob object instance in the

producer. For details of Management Based Activation of QoE Measurement

Collection see clause 4.5 and for details of Signalling Based

Activation of QoE Measurement Collection see clause 4.6 of TS 28.405.

The attributes pLMNTarget are only relevant when

Management Based Activation is used and the attribute qoETarget is

only relevant when Signalling Based Activation is used. All other

attributes are common for both Management Based Activation and

Signalling Based Activation.

When a MnS consumer wishes to deactivate a QMC Job, the MnS consumer

shall delete the corresponding QMCJob instance.

NOTE:  If the reporting is ongoing, when a request to delete a QMCJob

is received, the reporting does not end. The QMCJob instance is deleted,

when the last reporting for the QMC Job expires.

The jobId attribute presents the job identifier of a QMCJob instance.

jobId can be used to associate  multiple QMCJob instances. For example,

it is possible to configure the same jobId value for multiple QMCJob

instances required to produce the data (e.g. Streaming services and

MTSI reports) for a specific network analysis.

The QoE Measurement Collection provides capability for collecting QoE

information from:

- UEs which are in the specified area in case of Management Based

Activation or

- an individual UE in case of Signalling Based Activation.

The QoE Measurement Collection enables collection of application layer

from the UE for specified end user service type. The supported service

types are:

- Streaming services, see TS 26.247.

- MTSI services, see TS 26.114.

- VR services, see TS 26.118.

A QMCJob instance represents the job for collecting QoE measurements

according to the job parameters. For details of the QoE measurement

collection configuration parameters see clause 5 of TS 28.405. A QMCJob

instance can be name-contained by SubNetwork or ManagedElement.

A QMC Job is activated by creating a QMCJob object instance in the

MnS producer. For details of Management Based Activation of QoE

Measurement Collection see clause 4.5 and for details of Signalling

Based Activation of QoE Measurement Collection see

clause 4.6 of TS 28.405. The attributes pLMNTarget are

only relevant when Management Based Activation is used and the

attribute qoETarget is only relevant when Signalling Based Activation

is used. All other attributes are common for both Management Based

Activation and Signalling Based Activation.

When a MnS consumer wishes to deactivate a QMC Job, the MnS consumer

shall delete the corresponding QMCJob instance.

NOTE:  If the reporting is ongoing, when a request to delete a QMCJob

instance is received, the reporting does not end. The QMCJob instance

is deleted, when the last reporting for the QMC Job expires.

The jobId attribute presents the job identifier of a QMCJob instance.

The jobId can be used to associate  multiple QMCJob instances.

example, it is possible to configure the same jobId value for multiple

QMCJob instances required to produce the data (e.g. Streaming services

and MTSI reports) for a specific network analysis.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QMCJobGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses QMCJobSubTree;

}

augment "/me3gpp:ManagedElement" {

uses QMCJobSubTree;

}

}

<CODE ENDS>