3GPP TSG-SA5 Meeting #150 	S5-235579

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0106 | rev | - | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections in 3GPP TS 28.312 stage 3 OpenAPI section |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17/ Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Typos in Stage 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Typos in Stage 3 in 3GPP TS 28.312 stage 3 OpenAPI section |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spelling mistakes will not be corrected in the document |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2, 7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 7.2.2	OpenAPI document "TS28312_IntentNrm.yaml"

openapi: 3.0.1

info:

title: Intent NRM

version: 18.0.0

description: >-

OAS 3.0.1 definition of the Intent NRM

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------Definition of generic IOCs ----------#

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

Intent:

$ref: '#/components/schemas/Intent-Multiple'

Intent-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/IntentExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/RadioNetworkExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/ServiceSupportExpectation"

intentContexts:

type: array

items:

$ref: "#/components/schemas/IntentContext"

intentFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

#-------Definition of generic IOCs ----------#

#-------Definition of the generic IntentExpectation dataType ----------#

IntentExpectation:

description: >-

This data type is the "IntentExpectation" data type without specialisations

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

items:

$ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

$ref: "#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

required:

- expectationId

#-------Definition of the generic IntentExpectation dataType ----------#

#-------Definition of the generic ExpectationObject dataType ----------#

ExpectationObject:

description: >-

This data type is the "ExpectationObject" data type without specialisations

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork  #value for Radio Network Expectation--#

- Service_Support  #value for Service Support Expectation--#

- TBD    #-This will be added based on defined scenario specific intent expectation-#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

$ref: "#/components/schemas/ObjectContext"

#-------Definition of the generic ExpectationObject dataType ----------#

#-------Definition of the generic dataType --------------#

Condition:

type: string

enum:

- IS_EQUAL_TO

- IS_LESS_THAN

- IS_GREATER_THAN

- IS_WITHIN_RANGE

- IS_OUTSIDE_RANGE

- IS_ONE_OF

- IS_NOT_ONE_OF

- IS_EQUAL_TO_OR_LESS_THAN

- IS_EQUAL_TO_OR_GREATER_THAN

- IS_ALL_OF

FulfilStatus:

type: string

readOnly: true

enum:

- FULFILLED

- NOT_FULFILLED

NotFulfilledState:

type: string

readOnly: true

enum:

- ACKNOWLEDGED

- COMPLIANT

- DEGRADED

- SUSPENDED

- TERMINATED

- FULFILMENTFAILED

FulfilmentInfo:

type: object

properties:

fulfilStatus:

$ref: "#/components/schemas/FulfilStatus"

notFullfilledState:

description: ->

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

$ref: "#/components/schemas/NotFulfilledState"

notFulfilledReasons:

description: ->

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

type: string

readOnly: true

ExpectationVerb:

type: string

enum:

- DELIVER

- ENSURE

#-------Definition of the generic dataType --------------#

#-------Definition of the generic IntentContext dataType --------------#

IntentContext:

description: >-

This data type is the "IntentContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

#-------Definition of the generic IntentContext dataType --------------#

#-------Definition of the generic ExpectationTarget dataType----------#

ExpectationTarget:

description: >-

This data type is the "ExpectationTarget" data type without specialisations

type: object

properties:

targetName:

type: string

targetCondition:

$ref: "#/components/schemas/Condition"

targetValueRange:

type: number

targetContexts:

type: array

items:

$ref: "#/components/schemas/TargetContext"

TargetContext:

description: >-

This data type is the "TargetContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: number

#-------Definition of the generic ExpectationTarget  dataType----------#

#-------Definition of the generic ObjectContext dataType----------------#

ObjectContext:

description: >-

This data type is the "ObjectContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

#-------Definition of the generic ObjectContext dataType----------------#

#-------Definition of the generic ExpectionContext dataType----------------#

ExpectationContext:

description: >-

This data type is the "ExpectationContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

#-------Definition of the concrete ExpectationContext dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/Intent-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

2nd Change

### 7.2.3	OpenAPI document "TS28312_IntentExpectations.yaml"

openapi: 3.0.1

info:

title: Scenario specific Intent Expectations

version: 18.0.0

description: >-

OAS 3.0.1 definition of scenario specific Intent Expectations

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------Definition of the Scenario specific IntentExpectation dataType ----------#

RadioNetworkExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for radio network delivering and performance assurance

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/RadioNetworkExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/WeakRSRPRatioTarget"

- $ref: "#/components/schemas/LowSINRRatioTarget"

- $ref: "#/components/schemas/AveULRANUEThptTarget"

- $ref: "#/components/schemas/AveDLRANUEThptTarget"

- $ref: "#/components/schemas/LowULRANUEThptRatioTarget"

- $ref: "#/components/schemas/LowDLRANUEThptRatioTarget"

expectationContexts:

type: array

items:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

required:

- expectationId

ServiceSupportExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for service deployment

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/ServiceSupportExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/DLThptPerUETarget"

- $ref: "#/components/schemas/ULThptPerUETarget"

- $ref: "#/components/schemas/DLLatencyTarget"

- $ref: "#/components/schemas/ULLatencyTarget"

- $ref: "#/components/schemas/MaxNumberofUEsTarget"

- $ref: "#/components/schemas/ActivityFactorTarget"

- $ref: "#/components/schemas/UESpeedTarget"

- $ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/ServiceStartTimeContext"

- $ref: "#/components/schemas/ServiceEndTimeContext"

- $ref: "#/components/schemas/UEMobilityLevelContext"

- $ref: "#/components/schemas/ResourceSharingLevelContext"

- $ref: "TS28312_IntentNrm.yaml#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

required:

- expectationId

#-------Definition of the IntentExpectation dataType ----------#

#-------Definition of the scenario specific ExpectationObject dataType ----------#

RadioNetworkExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for RadioNetworkExpectation

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork #value for Radio Network Expectation--#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/CoverageAreaPolygonContext"

- $ref: "#/components/schemas/CoverageTACContext"

- $ref: "#/components/schemas/PLMNContext"

- $ref: "#/components/schemas/NRFqBandContext"

- $ref: "#/components/schemas/RATContext"

- $ref: "#/components/schemas/ObjectContext"

ServiceSupportExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for ServiceSupportExpectation

type: object

properties:

objectType:

type: string

enum:

- Service_Support #value for Service Support Expectation--#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/EdgeIdentifiticationIdContext"

- $ref: "#/components/schemas/EdgeIdentifiticationLocContext"

- $ref: "#/components/schemas/CoverageAreaTAContext"

- $ref: "#/components/schemas/ObjectContext"

#-------Definition of the ExpectationObject dataType ----------#

#-------Definition of the Scenario specific ExpectationTarget dataType----------#

WeakRSRPRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for WeakRSRPRatioTarget

type: object

properties:

targetName:

type: string

enum:

- WeakRSRPRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/WeakRSRPContext"

targetFulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

WeakRSRPContext:

description: >-

This data type is the "TargetContext" data type with specialisations for WeakRSRPContext

type: object

properties:

contextAttribute:

type: string

enum:

- WeakRSRPThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

LowSINRRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowSINRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowSINRRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowSINRContext"

targetFulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

LowSINRContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowSINRContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowSINRThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: integer

AveULRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveULRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveULRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

targetFulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

AveDLRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveDLRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveDLRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

targetFulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

LowULRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowULRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowULRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowULRANUEThptContext"

targetFulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

LowULRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowULRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowULRANUEThptThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: number

LowDLRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowDLRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowDLRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowDLRANUEThptContext"

targetFulfilmentInfo:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/FulfilmentInfo"

LowDLRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowDLRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowDLRANUEThptThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

DLThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- DlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/XLThpt"

ULThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- UlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/XLThpt"

DLLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- DlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ULLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- UlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

MaxNumberofUEsTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for MaxNumberofUEsTarget

type: object

properties:

targetName:

type: string

enum:

- maxNumberofUEs

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ActivityFactorTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ActivityFactorTarget

type: object

properties:

targetName:

type: string

enum:

- activityFactor

targetCondition:

type: string

enum:

- IS_EQUAL_TO

targetValueRange:

type: integer

UESpeedTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for UESpeedTarget

type: object

properties:

targetName:

type: string

enum:

- uESpeed

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

#-------Definition of the concrete ExpectationTarget  dataType----------#

#-------Definition of the concrete ObjectTarget dataType----------------#

ObjectContext:

description: >-

This data type is the "ObjectContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "TS28312_IntentNrm.yaml#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

CoverageAreaPolygonContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaPolygonContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaPolygon

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "#/components/schemas/CoverageArea"

CoverageArea:

type: string

CoverageTACContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageTACContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaTac

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28541_NrNrm.yaml#/components/schemas/NrTac"

PLMNContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for PLMNContext

type: object

properties:

contextAttribute:

type: string

enum:

- PLMN

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28623_ComDefs.yaml#/components/schemas/PlmnId"

NRFqBandContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for NRFqBandContext

type: object

properties:

contextAttribute:

type: string

enum:

- NRFqBand

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

RATContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for RATContext

type: object

properties:

contextAttribute:

type: string

enum:

- RAT

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

enum:

- UTRAN

- EUTRAN

- NR

EdgeIdentificationIdContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdentificationIdContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationId

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: array

items:

type: string

EdgeIdentificationLocContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdentificationLocContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationTarget

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: array

items:

type: string

CoverageAreaTAContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaTAContext

type: object

properties:

contextAttribute:

type: string

enum:

- coverageAreaTA

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "#/components/schemas/CoverageAreaTAList"

CoverageAreaTAList:

type: integer

#-------Definition of the scenario specific  ObjectTarget dataType----------------#

#-------Definition of the concrete ExpectationContext dataType----------------#

ServiceStartTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceStartTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceStartTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: string

ServiceEndTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceEndTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceEndTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: string

UEMobilityLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for UEMobilityLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- UEMobilityLevel

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/MobilityLevel"

ResourceSharingLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ResourceSharingLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- ResourceSharingLevel

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/SharingLevel"

#-------Definition of the concrete ExpectationContext dataType----------------#

End Change