3GPP TSG-SA5 Meeting #150	S5-235598

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0041 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.538 EAS Relocation Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EAS Relocation was studied in studfy phase. It is crucial to propose the same for TS. As described in clause 5.2.2.3 of GSMA OPG[2] , because of the OP’s measurements or knowledge, or hints from the application about performance degradations, the OP may decide that a different edge compute resource can better host the Edge Application. The EAS relocation trigger from OP are related with lifecycle management of its edge compute resources (for example, the overload of an edge compute resource, a failure or planned maintenance, a new or expanded edge compute resource, an issue with the network for its edge compute resource). The OP should be cognisant of the policy indication from the Application Provider about its sensitivity to a change of the edge compute resource hosting the Edge Application. As described in clause 5.1.1.2.2 GSMA OPG [2], the NBI allows an Application Provider to specify their support for a stateful or stateless Edge Application, i.e. whether the Edge Application can be moved from one edge compute resource to another and this with or without prior notification. The Application Provider shall be able to indicate the following policies: - Its Edge Application cannot be moved from one edge compute resource to another; - Its Edge Application can be moved from one edge compute resource to another, without any notification; - Its Edge Application can be moved from one edge compute resource to another with prior notification. TS 28.538 does not support this yet. This requires an EAS to be moved from one (source) EDN to another (target) EDN. This will involve terminating EAS from the source EDN and then instantiating EAS on the target EDN. This involves the following issues to be addressed - How the ASP will provide the policies How the existing application context is retained from source to target EAS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose EAS Relocation Requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x (New), 5.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Section

### 5.1.x	EAS Relocation

Due to operator measurements or knowledge, or hints from the application about performance degradations, the operator may decide that a different EDN can better host the EAS. The EAS relocation trigger from operator are related with lifecycle management of its edge compute resources (for example, the overload of an edge compute resource, a failure or planned maintenance, a new or expanded edge compute resource, an issue with the network for its edge compute resource). The ASP provides its policy indication regarding change of the edge compute resource hosting the Edge Application. There operator considers these policy while relocating EAS from one EDN to another.

The ASP indicates the following policies:

-	Its Edge Application cannot be moved from one edge compute resource to another;

-	Its Edge Application can be moved from one edge compute resource to another, without any notification;

-	Its Edge Application can be moved from one edge compute resource to another with prior notification.

When the policy is that a change of edge compute resource can be done with prior notification, the operator decides that a change of edge compute resource is needed and selects the new edge compute resource. In this case, the ASP chooses the exact timing of the move. The application layer functionalities will be responsible for transferring the application state from one edge compute resource to another.  If the ASP indicates that the EAS is not able to handle relocation, the operator can cancel the relocation procedure.

Next Modified Section

### 5.1.14	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-EAS-INST-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the deployment of EAS based on the given deployment requirements. | EAS Deployment |
| REQ-EAS-INST-FUN-2 | Generic Provisioning MnS Producer should have the capability to deploy EAS at a suitable EDN which can support the EAS requirements e.g. serving location, required latency, affinity/anti-affinity with other EAS, service continuity. | EAS Deployment |
| REQ-EAS-INST-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of instantiation as the response to the deployment request. | EAS Deployment |
| REQ-EAS-INST-FUN-4 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g. success, failure) of instantiation operation. | EAS Deployment |
| REQ-EAS-TERM-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the termination of the EAS VNF instance. | EAS Termination |
| REQ-EAS-TERM-FUN-2 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of termination as the response to the termination request. | EAS Termination |
| REQ-EAS-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g. success, failure) of termination operation. | EAS Termination |
| REQ-EAS-QUERY-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to obtain the EAS instance information. | Query EAS information |
| REQ-EAS-MOD-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the modification of the EAS instance. | EAS Modification |
| REQ-EES-INST-FUN-1 | Generic provisioning MnS producer should have the capability to instantiate the EES, as per request from authorized consumers. | EES Deployment |
| REQ-EES-INST-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of EES instantiation | EES Deployment |
| REQ-EES-INST-FUN-3 | Generic provisioning MnS producer should have the capability to relate instantiated EES with one or multiple served EAS(s). | EES Deployment |
| REQ-EES-TERM-FUN-1 | Generic provisioning MnS producer should have the capability to terminate the EES with the EES identifier, as per request from authorized consumers | EES Termination |
| REQ-EES-TERM-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of EES termination | EES Termination |
| REQ-EES-QUERY-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to obtain the EES instance information. | Query EES information |
| REQ-EES-MOD-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the modification of the EES instance. | EES Modification |
| REQ-EES-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of EES instantiation as the response to the deployment request. | EES Deployment |
| REQ-EES-TERM-FUN-4 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of EES termination as the response to the termination request. | EES Termination |
| REQ-ECS-INST-FUN-1 | Generic provisioning MnS producer should have the capability to instantiate the ECS, as per request from authorized consumers. | ECS Deployment |
| REQ-ECS-INST-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of ECS Instantiation. | ECS Deployment |
| REQ-ECS-INST-FUN-3 | Generic provisioning MnS producer should have the capability to relate instantiated ECS with one or multiple served EES(s). | ECS Deployment |
| REQ-ECS-TERM-FUN-1 | Generic provisioning MnS producer should have the capability to terminate the ECS with the ECS identifier, as per request from authorized consumers. | ECS Termination |
| REQ-ECS-TERM-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of ECS termination. | ECS Termination |
| REQ-ECS-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of ECS instantiation as the response to the deployment request. | ECS Termination |
| REQ-ECS-TERM-FUN-4 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of ECS termination as the response to the termination request. | ECS Termination |
| REQ-ECS-QUERY-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to obtain the ECS instance information. | Query ECS information |
| REQ-ECS-MOD-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the modification of the ECS instance. | ECS Modification |
| REQ-EAS-DF-FUN-1 | Performance assurance MnS producer should have a capability allowing an authorized consumer to request the collection of EAS discovery failure measurements that contain UE location and the requesting application characteristics information. | Instantiation triggered by EAS discovery failure |
| REQ-EAS-DF-FUN-2 | 3GPP Management system should have a capability allowing an EES to trigger or request for an EAS deployment. | Instantiation triggered by EAS discovery failure |
| REQ-EAS-REL-FUN-1 | Generic provisioning MnS producer shall have a capability allowing EAS to declare its mobility policies indicating whether EAS can be moved from one EDN to another. | EAS Relocation |
| REQ-EAS-REL-FUN-2 | Generic provisioning MnS producer shall have a capability allowing EAS to declare its mobility policies indicating whether EAS can be moved from one EDN to another, without any notification. | EAS Relocation |
| REQ-EAS-REL-FUN-3 | Generic provisioning MnS producer shall have a capability allowing EAS to declare its mobility policies indicating whether EAS can be moved from one EDN to another, with prior notification. | EAS Relocation |
| REQ-EAS-REL-FUN-4 | Generic provisioning MnS producer shall have a capability allowing ASP to reject the EAS relocation on receiving the relocation notification. | EAS Relocation |
| REQ-EAS-REL-FUN-5 | Generic provisioning MnS producer shall have a capability allowing scheduling of an EAS relocation. | EAS Relocation |

End Of Modified Section