3GPP TSG-SA5 Meeting #150	S5-235603

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0044 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.538 Federation Management Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Federation Managemnt was studies in the study phase. It is crucial to propose the same for TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UC and Requiremenst for Federation Management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (New), 3.1 and 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CRs 0042, 0043 and 0044 are related and impacting the same clause. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Section

## 5.x	Edge Federation Management

### 5.x.1	Description

This clause contains use cases associated with federation management.

### 5.x.x	Federation Management

The federation management functionality within the operator enables it to interact with other operator instances, often in different geographies, thereby providing access for the ASP to a larger footprint of EDN, a more extensive set of subscribers and multiple Operator capabilities. An operator initiates the establishment of federation relationship with another operator sharing available location(s) at which the edge services are provided, resource available at each location, federation expiry etc. The operator which initiates federation relationship is called Leading Operator (LO). The operator which receives federation relationship request is called Partner Operator (PO).

The federation relationship enables the following functionalities

-	Federated EAS resource reservation management: This is intended for an LO to reserve resources for an application provider, with the PO, when the application provider initiate the reservation using NBI.

-	Federated EAS deployment and termination: This will be used by an LO to instantiate an EAS deployment on EDN of LO as requested by application provider over NBI.

-	EDN sharing: This is intended for operator to share EDN among each other.

### 5.x.y	Requirements

Table 5.x.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-FED-FUN-1 | Generic Provisioning MnS shall have a capability to establishing federation relationship with the MnS consumer (e.g. partner operator platforms). | Federation Management |
| REQ-FED-FUN-2 | Generic Provisioning MnS shall enable federation relationship to include appropriate information including (not limited to) location(s) at which the edge services are provided, resource available at each location, federation expiry. | Federation Management |
| REQ-FED-FUN-3 | Generic Provisioning MnS shall have a capability to remove existing federation relationship with the MnS consumer(e.g. partner operator platforms). | Federation Management |

Next Modified Section

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Edge Computing: A concept, as described in 3GPP TS 23.501 [4], that enables operator and 3rd party services to be hosted close to the UE's access point of attachment, to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network.

Edge Computing Service Provider: A mobile network operator offering Edge Computing service.

Edge Data Network: A local Data Network that supports the architecture for enabling edge applications.

ECSP Management System: is a part of 3GPP management system that utilizes 3GPP defined management services to enable consumers (e.g., ASP. ECSP) to orchestrate and manage the EDN.

PLMN Management System: is a part of 3GPP Management System that utilizes 3GPP defined management services to enable consumers (e.g., PLMN operator) to orchestrate and manage the mobile networks.

Availability Zone: Refer to GSMA Operator Platform Telco Edge Requirements [14].

Leading Operator: The Leading Operator is the operator which consumes EDN shared by the PO. LO is considered as an ECSP Management System

Participating Operator: The Participating Operator is the operator who provides its EDN to be shared with Leading Operator. PO is considered as an ECSP Management System

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

FQDN	Fully Qualified Domain Name

GSM	Global System for Mobile Communications

GSMA	GSM Association

LO	Leading Operator

PO	Participating Operator

End Of Modified Section