3GPP TSG-SA5 Meeting #150 	S5-235616

Goteborg, Sweden, 21 - 25 August 2023

# Source:	Ericsson

# Title:	Update of quota management for NSACF

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

Include the proposed changes in TS 28.203.

# 2	References

[1]		3GPP TS 28.203: "Network Slice Admission Control charging in the 5G System (5GS); Stage 2"

# 3	Rationale

The type of units used number of simultaneous UEs or PDU sessions will be different than the ones used for seconds, bytes etc. in that these cannot be consumed only temporarily allocated. The allocation would be per S-NSSAI and NSACF.

If there is a many-to-one relationship between S-NSSAI and NSACF (i.e., one S-NSSAI can be supported by one and only one NSACF, while one NSACF can support many S-NSSAIs), then there is limited or no use of not allocating all units at start and only set thresholds. This since there is only one NSACF that can ask for quota for one S-NSSAI, and in that case the quota would have the same meaning as the trigger thresholds.

If there is a many-to-many relationship between S-NSSAI and NSACF (i.e., one S-NSSAI can be supported by many NSACFs, and one NSACF can support many S-NSSAIs), then there is a use of allocating some units to one NSACF. This since there are other NSACFs that can ask for quota for the same S-NSSAI, and in that case quota could be allocated on a NSACF need bases.

This means that either there is no need for quota management or that the maximum number of UEs or PDU sessions for the CCS is the sum of all NSCAF maximums.

# 4	Detailed proposal

First change

#### 5.2.1.3	Quota management

CHF converged charging quota management functionality is supported in Network Slice Admission Control converged charging, to allow controlling number of simultaneous UEs and/or PDU sessions per S-NSSAI. There is a difference between quota that can be consumed e.g., seconds, bytes, and quota that is temporary allocated like simultaneous UEs and PDU sessions in that the quota that can be consumed can report units used or unused, while temporary allocated quota can report units in use or not in use.

The NSACF is configured with maximum numbers for simultaneous UEs and PDU sessions per S-NSSAI, referred-to as "NSACF max number of UEs" and "NSACF max number of PDU sessions" respectively. The CCS can, based on internal criteria, have its own maximum numbers, referred-to as "CCS max number of UEs" and "CCS max number of PDU sessions" respectively.

End of changes